3GPP TSG SA WG2 Meeting #137E	S2-2002336

# February 24 – 27, 2020, Electronic Meeting	(Revision of S2-2002022)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.316 | CR | 1837 | rev | 1 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | AS level parameters to W-5GAN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Nokia, Nokia Shangahi Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5WWC |  |  |  |  |  | Date: |  |  | 2020-02-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | TS 23.316 specification indicates that the 5G-RG provides SUCI, 5G-S-TMSI or GUAMI as AS level parameters to the W-AGF together with a Registration Request. For the Service Request, the 5G-RG provides 5G-S-TMSI. These parameters are used by W-AGF to route the message to the AMF. However, it is not clear why tha AS layer need the SUCI. Furthermore, sending of 5G-S-TMSI is not inline with stage 3 specification for non-3GPP (TS 24.501) where it is specified: --- For the routing of the initial NAS message to the appropriate AMF, if the UE holds a 5G-GUTI or 4G-GUTI, the UE NAS provides the lower layers with either the 5G-S-TMSI or the registered GUAMI, or neither the 5G-S-TMSI nor registered GUAMI according to the following rules: <snip> b) if the service request procedure was initiated over non-3GPP access, the UE NAS shall provide the lower layers with the registered GUAMI, but shall not provide the lower layers with the 5G-S-TMSI; c) if the initial NAS message other than the SERVICE REQUEST or CONTROL PLANE SERVICE REQUEST message was initiated over non-3GPP access, the UE NAS shall provide the lower layers with the GUAMI of the 5G-GUTI that the UE NAS has selected as specified in the subclause 5.5.1.2.2 and 5.5.1.3.2, but shall not provide the lower layers with the 5G-S-TMSI; In addition, stage 2 does not justify why different AS level parameters are needed for 5WWC compared to non-3GPP access in general. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Provide GUAMI to W-AGF as AS layer parameter. It is clarified that AMF selection is only based on GUAMI and does not make use of 5G-T-TMSI as wireline AS protocol can only carry GUAMI |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect stage 2 specification, that may also result in inconsitency between BBF specs and 3GPP stage 3 specs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1, 7.2.1.1, 7.2.2.1, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************** First changes ***************

## 5.1	Network Function Functional description

### 5.1.1	W-AGF

The functionality of W-AGF in the case of Wireline 5G Access network includes the following:

-	Termination of N2 and N3 interfaces to 5G Core Network for control - plane and user-plane respectively.

-	Handling of N2 signalling from SMF (relayed by AMF) related to PDU Sessions and QoS.

-	Relaying uplink and downlink user-plane packets between the 5G-RG and UPF and between FN-RG and UPF. This involves:

-	Enforcing QoS corresponding to N3 packet marking, taking into account QoS requirements associated to such marking received over N2.

-	N3 user-plane packet marking in the uplink.

-	Supporting AMF discovery and selection defined in TS 23.501 [2] clause 6.3.5 where the 5G-S-TMSI is not used for AMF selection since the wireline AS layer can only carry the GUAMI.

-	Termination of wireline access protocol on Y4 and Y5.

-	In the case of FN-RG the W-AGF acts as end point of N1 on behalf of the FN-RG.

In the case of Wireline 5G Broadband Access network the definition of W-AGF functionalities is specified in WT-456 [9] and WT-457 [10].

Editor's note:	the references to BBF WT-456 and WT-457 will be revised when finalized by BBF.

In the case of Wireline 5G Cable Access network the definition of W-AGF functionalities is specified by Cablelabs [27].

*************** second changes ***************

#### 7.2.1.1	5G-RG Registration via W-5GAN

The 5G-RG registration management procedures are followed for both W-5GBAN and W-5GCAN.

Clause 7.2.1.1 specifies how a 5G-RG can register to 5GC via aW-5GAN. It is based on the Registration procedure specified in TS 23.502 [3] clause 4.2.2.2.2 and it uses the EAP method "EAP-5G" as specified in clause 4.12.2.1 in TS 23.502 [3]. If the 5G-RG needs to be authenticated, mutual authentication is executed between the 5G-RG and AUSF. The details of the authentication procedure are specified in TS 33.501 [11]. In Registration and subsequent Registration procedures via W-5GAN access, the NAS messages are always exchanged between the 5G-RG and the AMF. When possible, the 5G-RG can be authenticated by reusing the existing UE security context in AMF for the 5G-RG.

Figure 7.2.1.1-1: 5G-RG Registration via W-5GAN

NOTE 1:	EAP-5G is assumed to be used during authentication but need to be verified with BBF.

NOTE 2:	The procedure assuming that EAP-5G is used between the 5G-RG and the W-AGF. This assumption may be revised based on SA WG3 and BBF decision

1.	The 5G-RG connects to a W-5GAN with procedures outside the scope of 3GPP and creates an initial W-CP EAP signalling connection. This connection shall support EAP messages transfer between 5G-RG and W-AGF.

2.	The W-AGF sends an EAP-Request/5G-Start packet over the W-CP connection. The EAP-Request/5G-Start packet informs the 5G-RGto initiate an EAP-5G session, i.e. to start sending NAS messages encapsulated within EAP-5G packets.

3.	The 5G-RG sends an EAP-Response/5G-NAS packet that contains the Access Network parameters (GUAMI if available, the selected PLMN, Requested NSSAI and Establishment Cause) and a NAS Registration Request message (SUCI or 5G-GUTI as defined in TS 24.501, security parameters/UE security capability, NSSAI parameters, UE MM Core Network Capability, PDU session status, Follow-on request). The Establishment cause provides the reason for requesting a signalling connection with 5GC.

NOTE 3:	The steps from 1 to 3 depend on BBF decision for what protocols to use for NAS transport. The step needs to be revised based on their decision.4.	The W-AGF shall select an AMF based on the received AN parameters and local policy, as specified in TS 23.501 [2], clause 6.3.5. The W-AGF shall then forward the Registration Request received from the UE to the selected AMF within an N2 initial UE message (NAS message, Line-id based ULI, Establishment cause, UE context request).

5.	The selected AMF may decide to request the SUCI by sending a N2 Downlink NAS transport message (NAS Identity Request) message to W-AGF. This NAS message and the response are sent between W-AGF and 5G-RG encapsulated within EAP/5G-NAS packets. In this case the RG shall answer with a NAS Identity response that is carried over EAP-5G and N2 signalling.

6.	The AMF may decide to authenticate the 5G-RG by invoking an AUSF. In this case, the AMF shall select an AUSF as specified in TS 23.501 [2] clause 6.3.4 based on SUPI or SUCI.

The AUSF executes the authentication of the UE as specified in TS 33.501 [11]. The AUSF selects a UDM as described in TS 23.501 [2], clause 6.3.8 and gets the authentication data from UDM. The authentication packets are encapsulated within NAS authentication messages and the NAS authentication messages are encapsulated within EAP/5G-NAS packets. Between W-AGF and AMF, the messages are encapsulated within N2 downlink/uplink NAS transport messages. After the successful authentication:

In step 6c, the AUSF shall send the anchor key (SEAF key) to AMF which is used by AMF to derive NAS security keys. The 5G-RG also derives the anchor key (SEAF key) and from that key it derives the NAS security keys and the security key for W-AGF (W-AGF key).

Editor's note:	Whether the security key for W-AGF (W-AGF key) is also derived is to be determined with BBF and SA WG3.

In step 6c, the AUSF shall also include the SUPI, if in step 6a the AMF provided to AUSF a SUCI.

The AMF decides if the Registration Request needs to be rerouted as described in clause TS 23.502 [3] clause 4.2.2.2.3, where the initial AMF refers to the AMF.

Editor's note:	Check that procedure for authentication of 5G-RG is added to TS 33.501 [11].

NOTE 4:	EAP-AKA' or 5G-AKA are allowed for the authentication of 5G-RG via W-5GAN access, as specified in TS 33.501 [11]. Figure 7.2.1.1-1 only shows authentication flow using EAP-AKA'. Other EAP authentication methods (see TS 33.501 [11]) for 5G-CRG with non-3GPP identities and credentials can be used for Stand-alone Non-Public Network.

7a.	The AMF shall send a NAS Security Mode Command (Selected NAS security algorithm, ngKSI, Replayed UE security capabilities, IMEISV request, Additional 5G security information, EAP message) to 5G-RG in order to activate NAS security. If an EAP-AKA' authentication was successfully executed in step 6, the AMF shall encapsulate the EAP-Success received from AUSF within the NAS Security Mode Command message. The message is encapsulated within a N2 downlink NAS transport message.

7b.	The W-AGF shall forward the NAS Security Mode Command message to 5G-RG within an EAP/5G-NAS packet.

7c.	The 5G-RG completes the authentication procedure (if initiated in step 6), creates a NAS security context and sends the NAS Security Mode Complete message (IMEISV) within an EAP/5G-NAS packet.

Editor's note:	Whether a security key for W-AGF (W-AGF key) is also derived is to be determined by BBF and SA WG3.

7d.	The W-AGF relays the NAS Security Mode Complete message to the AMF in a N2 Uplink NAS transport message.

8a.	Upon receiving NAS Security Mode Complete, the AMF shall send an N2 Initial Context Setup Request message (Old AMF, RG Level Wireline Access Characteristics, RG-TMBR, GUAMI, Allowed NSSAI, UE security capability, Security Key, Trace Activation, Masked IMEISV).

8b.	This triggers the W-AGF to send an EAP-Success to 5G-RG, which completes the EAP-5G session. After this step, NAS messages between 5G-RG and W-AGF are transported without EAP-5G using W-CP signalling connection.

9.	[Conditional]: An W-CP signalling connection is established between the 5G-RG and W-AGF.

Editor's note:	Whether this step is performed or not requires input from BBF and CableLabs.

10.	W-AGF notifies the AMF that the 5G-RG context (including AN security) was created by sending a N2 Initial Context Setup Response.

11.	[Conditional] The AMF may request the PEI from the 5G-RG as described in TS 23.502 [3], clause 4.2.2.2.2 step 11.

12.	The AMF performs step 12-16 in TS 23.502 [3] clause 4.2.2.2.2. At AMF registration to UDM for the 5G-RG, the Access Type non-3GPP access is used. The RAT type used toward PCF and UDM shall indicate wireline access. The AMF determines Access Type and RAT Type based on the Global RAN Node ID associated with the N2 interface.

13.	The AMF sends N2 Downlink NAS transport with the NAS Registration Accept message (as defined in step 21 TS 23.502 [3] clause 4.2.2.2.2) to the W-AGF, which forwards the NAS Registration accept message to the 5G-RG.

14.	[Conditional]The 5G-RG responds with NAS Registration Complete message as described in TS 23.502 [3] clause 4.2.2.2.2 step 22 and W-AGF forwards the NAS Registration Complete message to AMF in a N2 Uplink NAS transport message.

15. The AMF performs step 23-24 in TS 23.502 [3] clause 4.2.2.2.2.

*************** Next Change ***************

#### 7.2.2.1	5G-RG Service Request procedure via W-5GAN Access

The Service Request procedure via W-5GAN shall be used by a 5G-RG in CM-IDLE state over W-5GAN to request the re-establishment of the NAS signalling connection and the re-establishment of the user plane for all or some of the PDU Sessions which are associated to non-3GPP access.

NOTE 1:	For a W-5GAN access, the Service Request procedure is never a response to a Paging i.e. Paging does not apply on a W-5GAN access.

The Service Request procedure via W-5GAN shall be used by a 5G-RG in CM-CONNECTED state over wireline access to request the re-establishment of the user plane for one or more PDU Sessions which are associated to non-3GPP access.

Figure 7.2.2.1-1: UE Triggered Service Request procedure via W-5GAN

Editor's note:	EAP-5G is assumed to be used during authentication but need to be verified with BBF/CableLabs.

1.	The 5G-RG connects to a W-5GAN with procedures outside the scope of 3GPP and creates an initial not authenticated W-CP EAP connection. This connection shall support EAP messages transfer between 5G-RG and W-AGF.

2.	The W-AGF sends an EAP-Request/5G-Start packet over the W-CP connection. The EAP-Request/5G-Start packet informs the 5G-RG to initiate an EAP-5G session, i.e. to start sending NAS messages encapsulated within EAP-5G packets.

3.	The 5G-RG sends an EAP-Response/5G-NAS packet that contains the Access Network parameters (GUAMI and Establishment Cause) and a NAS Service Request message (List Of PDU Sessions To Be Activated, security parameters, PDU Session status, Uplink Data Status, 5G-S-TMSI). The Establishment cause provides the reason for requesting a signalling connection with 5GC. In this release of the specification no Selected PLMN parameter is sent by a 5G RG.

4.	The W-AGF shall then forward the Service Request received from the 5G-RG to the selected AMF within an N2 initial UE message (NAS Service Request message, Line-id based User Location Information, Establishment cause, UE context request).

5.	If the Service Request was not sent integrity protected or integrity protection verification failed, the AMF shall initiate NAS authentication/security procedure as defined in step 6 and step 7 in clause 7.2.1.1.

If the UE in CM-IDLE state triggered the Service Request to establish a signalling connection only, after successful establishment of the signalling connection the UE and the network can exchange NAS signalling and steps 6 and 14 are skipped.

6.	Steps 4-11 in TS 23.502 [3] figure 4.2.3.2-1 are performed for each requested PDU session user plane.

7.	(If the 5G RG was CM-IDLE) AMF sends an N2 Initial Context Setup Request message (N2 SM information received from SMF(s), RG-TMBR, GUAMI, Allowed NSSAI, UE security capability, Security Key, Trace Activation, Masked IMEISV).

If the 5G RG was CM-CONNECTED the AMF sends N2 SM information received from SMF(s).

8.	(If the 5G RG was CM-IDLE) This triggers the W-AGF to send an EAP-Success to 5G-RG, which completes the EAP-5G session. After this step, NAS messages between 5G-RG and W-AGF are transported without EAP-5G using W-CP signalling connection.

9.	[Conditional, if the 5G RG was CM-IDLE] An W-CP signalling connection is established between the 5G-RG and W-AGF.

NOTE 2:	Step 9-11 are defined by BBF/Cablelabs.

Editor's note:	The description below for how W-AGF maps the PDU Session resource management towards 5GC with the PDU Session resource management towards the RG needs to be verified by BBF/Cablelabs. Also, how W-AGF maps the PDU Session N3 tunnel towards 5GC with the PDU Session user plane towards the RG needs to be verified by BBF/Cablelabs.

Steps 10 and 11 are carried out for each PDU Session indicated in step 7

10.	Based on its own policies and configuration and based on the QoS flows and QoS parameters received in the previous step, the W-AGF shall determine what W-UP resources are needed for the PDU session.

11.	The W-AGF sets up the W-UP resources for the PDU session. This step is specified by BBF for W-5BGAN and by CableLabs for W-5GCAN. The access dependent W-UP resource setup procedure shall map to the identity of the PDU Session associated with the W-UP resource.

12.	W-AGF notifies the AMF that the 5G-RG context was created by sending a N2 Initial Context Setup Response (N2 SM information that provides AN Tunnel Info, List of accepted QoS Flows, List of rejected QoS Flowsper PDU Session ID for PDU Sessions whose UP connections are activated).

13.	AMF sends NAS Service Accept via W-AGF to the 5G-RG.

14.	All steps after step 14 in TS 23.502 [3] figure 4.2.3.2-1 are performed for each requested PDU session user plane.

When the 5G-RG is in CM-CONNECTED state over W-5GAN access and the network receives downlink data for a PDU Session over wireline access that has no user plane connection, the steps 1-4a in TS 23.502 [3], clause 4.2.3.3 (Network Triggered Service Request) shall be performed with the following exceptions:

-	The (R)AN corresponds to an W-AGF.

-	The UE corresponds to the 5G-RG.

-	In step 4a of TS 23.502 [3] clause 4.2.3.3, the steps 2b-6 in figure 7.3.1.1-1 are performed to establish the W-UP resources and to establish N3 tunnel. In step 2b and 6, no NAS message is exchanged with the UE.

*************** End of changes ***************