3GPP SA WG2 Meeting #137E	S2-2001806

Feburary 24-28, 2020                                                                                      was S2-2001354

# Source:	Juniper

# Title:	5G-VN Integrated with IETF IP/E-VPN or Compatible with IEEE Ethernet

# Document for:	Approval

Agenda Item:	9.1

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title: 5G-VN Integrated with IETF IP/E-VPN or Compatible with IEEE Ethernet

## Acronym: 5G-VPN

## Unique identifier: 	{A number to be provided by MCC at the plenary}

Potential target Release: Rel-17.

Note that this field above indicates the proposed Release at the time of submission of the WID to TSG approval. It can later be changed without a need to revise the WID. The updated target Release is indicated in the Work Plan.

## 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  | X |
| Don't know |  |  |  |  |  |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a …

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

### 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| Vertical_LAN | SA2 | 820017 | 5GS Enhanced support of Vertical and LAN Services |

### 2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
|  | N/A |  |

Dependency on non-3GPP (draft) specification: RFC4364, RFC7432

## 3	Justification

Current 5G-VN solutions specified in TS 23.501 have many documented limitations. As discussed in S2-2000142, it is beneficial to integrate existing wireline IEEE compatible Ethernet Switch and/or IETF IP/E-VPN solutions with 5G-VN, which can achieve the following with just simple changes to 23.501, as an OPTIONAL way of supporting 5G-VN:

- Remove known limitations in the currently specified 5G-VN solutions

- Allow operators to provide Virtual Network services seamlessly across wireline and wireless networks

- Allow all existing and future features/optimizations of IP/E-VPN solutions to be applicable to 5G-VN

This WID is revised from S2-2001354, which resulted from S2-2000142 and further input received since then.

This proposal meets TEI criteria in that it is a straightforward enhancement with significant benefits, and it is expected to take 0.5 TU.

## 4	Objective

The objective is to specify routing/switching-based OPTIONs of supporting 5G-VN in a more complete and optimized way. The following is a summary, and the S2-2001807 is the corresponding proposed CR.

- When members of a 5G-VN are anchored on the same UPF or if no UPF-UPF tunnelling is used, a UPF behaves as an IP router (in case of IP PDU) or Ethernet switch (in case of Ethernet PDU) for the VN, with the PDU sessions treated as logical router/switch interfaces. In the Ethernet case, the switch is IEEE-compatible, including running Spanning Tree Protocols or using other methods to block traffic if needed to prevent looping. The other methods are outside the scope of this specification.

- When UE members of a 5G-VN are anchored on different UPFs and UPF-UPF tunnelling is used, one of the following two options can be used:

  - The UPF-UPF N19 tunnels are also treated as logical interfaces of the router/switch for the VN

    - This is similar to #1, except that in the Ethernet case, if there are more than two UPFs for the VN, Spanning Tree Protocols or other methods must be used to prevent looping among UPFs (the STP method leads to non-optimal UPF-UPF forwarding through another UPF, while the other known alternative is very similar to E-VPN).

  - Integrating IETF IP/E-VPN solutions

    - This removes the need for running SPT on UPF-UPF tunnels and avoids related non-optimal UPF-UPF forwarding in option #2b.

    - This applies all current/future IP/E-VPN features/optimizations to 5G-VN

In essence, traffic forwarding is autonomous based on the built-in routing/switching functionalities of UPF without the need of SMF programmed PDRs/FARs.

Note: even when these routing/switching-based options are used for a 5G-VN, PDRs/FARs can still be used on top of UPF autonomous forwarding. For example, traffic of different flows to the same destination could be forwarded towards the same member (based on IP/MAC address lookup) but with different QoS treatment (based on PDR/FAR where the PDR contains additional information identifying the flows and the FAR contains additional forwarding treatment).

With option #2b, a UPF behaves as an IP/E-VPN PE (Provider Edge) with IP/MAC VRFs (Virtual Routing and Forwarding tables) or BDs (Bridge Domains) corresponding to 5G-VNs, which router/switch instances mentioned above. The PDU sessions are treated as PE-CE interfaces of IPVPN or ACs (Attachment Circuits) of EVPN in the corresponding VRFs/BDs.

## 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| N/A |  |  |  |  |  |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 23.501 | A clause 5.8.2.13.4 that specifies the optional alternative, plus some minor relevant changes | TSG#88 | This will be handled as “TEI17”. |

## 6	Work item Rapporteur(s)

Zhaohui (Jeffrey) Zhang, Juniper Networks, zzhang@juniper.net

## 7	Work item leadership

SA2

## 8	Aspects that involve other WGs

N/A

## 9	Supporting Individual Members

Supporting IM name

Juniper

Orange

China Mobile

KPN

Spreadtrum