3GPP TSG-SA2 Meeting #137-E	S2-2001857

Elbonia, February 24 – 27, 2020

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0117 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Slice service experience data collection corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA |  |  |  |  |  |  | Date: |  | 2020-02-18 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Observed service experience for a Network Slice requires the UE IDs (i.e. SUPIs) to be retrieved by NWDAF when ‘any UE’ is used as target of analytics reporting in order to get the service data from an AF only for UEs on the slice of interest. The current specification lacks SUPIs as input data information that needs to be collected by NWDAF and it does not mention the corresponding AMF event ID providing such information, namely ‘Area of Interest’ in TS 23.502 (see accompanying CR 2089 to TS 23.502). Output analytics tables are proposed to be differentiated between application service experience and slice service experience, including slice instance service experience as approved in S2-2001153 during SA2#136AH. Given the large amount of UEs that may be registered on the same Network Slice/Network Slice instance within an Area of Interest, no SUPI list should be provided by NWDAF as output analytics for slice service experience to avoid overloading the network, so the slice service experience output analytics tables are updated accordingly. In addition, slice and slice instance service experience as well as NSI ID(s) may be unneccesary output information for application service experience, so it is proposed to remove those from application service experience output analytics table. Clause 6.4.5 has been updated to reflect both the updates of SA2#136AH on multiple slice instance deployment option agreed on S2-2001153 and the proposals on the accompanying CRs (CR 2140 and CR 2089) updating the AMF event. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In Clause 6.5.1, a new entry to Table 6.4.2-2 is added accounting for subscriptions with ‘any UE’ as target of analytics reporting (i.e. the analytics subscription/request does not contain UE IDs) so SUPIs need to be retrieved by NWDAF from AMF. In addition, the specific event ID (‘Area of Interest’) in TS 23.502 containing the relevant event filters is mentioned. Clause 6.4.3 has been updated to separate output statistics and predictions when they refer to an Application and to a Network Slice; slice service experience entries are removed from Tables 6.4.3-1 and 6.4.3-2 and a reference to detailed service experience is removed since the concept had been previously removed from the specification; new tables for slice service experience are introduced (i.e. Tables 6.4.3-3 and 6.4.3-4) removing list of SUPIs from output analytics and including slice service experience and slice instance service experience entries, as well as the corresponding NSI ID(s) if available. A NOTE is added to refer to the optionality of NSI IDs. The description of Spatial validity entry in the new tables also contain the approved changes in S2-2001287 in SA2#136AH for Tables 6.4.3-1 and 6.4.3-2. In addition, a number of updates are proposed to Clause 6.4.5: i) alignment with previous agreement on multiple slice instances deployment option, ii) reference to updated AMF event proposed in the accompanying CRs 2140 and 2089, and iii) editorial updates to procedure in Figure 6.4.5-1 as well as removal of now unnecessary NOTE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Slice service experience analytics will not be fully specified in Rel-16 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.2, 6.4.3, 6.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS/TR 23.501 CR 2140 |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR 23.502 CR 2089 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of Changes * * * *

### 6.4.2	Input Data

The service data collected from the AF, the network data from other 5GC NFs and the network data from OAM for observed service experience are defined in Table 6.4.2-1, Table 6.4.2-2 and Table 6.4.2-3, respectively.

Table 6.4.2-1: Service Data from AF related to the observed service experience

| Information | Source | Description |
| --- | --- | --- |
| Application ID | AF | To identify the service and support analytics per type of service (the desired level of service) |
| IP filter information | AF | Identify a service flow of the UE for the application |
| Locations of Application | AF/NEF | Locations of application represented by a list of DNAI(s). The NEF may map the AF-Service-Identifier information to a list of DNAI(s) when the DNAI(s) being used by the application are statically defined. |
| Service Experience | AF | Refers to the QoE per service flow as established in the SLA and during on boarding. It can be either e.g. MOS or video MOS as specified in ITU-T P.1203.3 [11] or a customized MOS |
| Timestamp | AF | A time stamp associated to the Service Experience provided by the AF, mandatory if the Service Experience is provided by the ASP. |

NWDAF subscribes to the service data from AF in the Table 6.4.2-1 either directly by invoking Naf_EventExposure_Subscribe service (Event ID = Service Data, Event Filter information = Application ID) as defined in TS 23.502 [3], or via NEF by invoking Nnef_EventExposure_Subscribe service.

Table 6.4.2-2: QoS flow level Network Data from 5GC NF related to the QoS profile assigned for a particular service (identified by an Application Id or IP filter information)

| Information | Source | Description |
| --- | --- | --- |
| Timestamp | 5GC NF | A time stamp associated with the collected information. |
| Location | AMF | The UE location information. |
| SUPI(s) | AMF | If UE IDs are not provided as target of analytics reporting for slice service experience, AMF returns the UE IDs matching the AMF event filters. |
| DNN | SMF | DNN for the PDU Session which contains the QoS flow |
| S-NSSAI | SMF | S-NSSAI for the PDU Session which contains the QoS flow |
| Application ID | SMF | Used by NWDAF to identify the application service provider and application for the QoS flow |
| IP filter information | SMF | Provided by the SMF, which is used by NWDAF to identify the service data flow for policy control and/or differentiated charging for the QoS flow |
| QFI | SMF | QoS Flow Identifier |
| QoS flow Bit Rate | UPF | The observed bit rate for UL direction; and The observed bit rate for DL direction |
| QoS flow Packet Delay | UPF | The observed Packet delay for UL direction; and The observed Packet delay for the DL direction |
| Packet transmission | UPF | The observed number of packet transmission |
| Packet retransmission | UPF | The observed number of packet retransmission |

NOTE:	How NWDAF collects QoS flow Bit Rate, QoS flow Packet Delay, Packet transmission and Packet retransmission information is not defined in this Release of the specification.

NWDAF subscribes to the network data from 5GC NF(s) in the Table 6.4.2-2 by invoking Nnf_EventExposure_Subscribe service operation with the following Event IDs as input parameters:

-	AMF Source: Namf_EventExposure_Subscribe (Event IDs = Location Changes, Area of Interest).

-	SMF Source: Nsmf_EventExposure_Subscribe (Event ID = QFI allocation).

Table 6.4.2-3: UE level Network Data from OAM related to the QoS profile

| Information | Source | Description |
| --- | --- | --- |
| Timestamp | OAM | A time stamp associated with the collected information. |
| Reference Signal Received Power | OAM | The per UE measurement of the received power level in a network cell, including SS-RSRP, CSI-RSRP as specified in clause 5.5 of TS 38.331 [14] and E-UTRA RSRP as specified in clause 5.5.5 of TS 36.331 [15] |
| Reference Signal Received Quality | OAM | The per UE measurement of the received quality in a network cell, including SS-RSRQ, CSI-RSRQ as specified in clause 5.5 of TS 38.331 [14] and E-UTRA RSRQ as specified in clause 5.5.5 of TS 36.331 [15] |
| Signal-to-noise and interference ratio | OAM | The per UE measurement of the received signal to noise and interference ratio in a network cell, including SS-SINR, CSI-SINR, E-UTRA RS-SINR, as specified in clause 5.1 of TS 38.215 [12] |

NWDAF subscribes the network data from OAM in the Table 6.4.2-3 by using the services provided by OAM as described in clause 6.2.3.

The Event Filter for the service data collection from AF is defined in Table 6.4.2-4.

Table 6.4.2-4: Event Filter Information related to the service data from AF

| Information | Presence | Description |
| --- | --- | --- |
| Application ID (1..n) | Conditional (NOTE 1) | An identification of the application or a set of identifications of the applications. |
| Area of Interest | Conditional (NOTE 2) | Area of Interest which restricts the area in focus |
| NOTE 1: If no Application ID is provided, the Event Filter information applies to any Application (i.e. all Applications). NOTE 2: Mandatory if Target Of Event Reporting is set to "any UE", optional otherwise. |  |  |

The Event Filter for the service data collection from SMF and AMF are defined in TS 23.502 [3].

The timestamps are provided by each NF to allow correlation of QoS and traffic KPIs. The clock reference is able to know the accuracy of the time and correlate the time series of the data retrieved from each NF.

### 6.4.3	Output Analytics

The NWDAF services as defined in the clause 7.2 and 7.3 are used to expose the analytics.

-	Service Experience statistics information is defined in Table 6.4.3-1.

-	Service Experience predictions information is defined in Table 6.4.3-2.

Table 6.4.3-1: Service Experience statistics

| Information | Description |
| --- | --- |
| S-NSSAI | Identifies the Network Slice for which analytics information is provided. |
| Service experiences (1…n) (NOTE) | List of observed service experience information for each Network Slice instance. |
| > NSI ID | Identifies the Network Slice instance within the Network Slice |
| > Slice instance service experience | Service experience across Applications on a Network Slice instance instance over the Analytics target period (average, variance). |
| > Application ServiceExperiences (1..max) | List of observed service experience information for each Application. |
| >> Application ID | Identification of the application. |
| >> Service Experience Type | Type of Service Experience analytics, e.g. on voice, video, other. |
| >> Service Experience | Service Experience over the Analytics target period (average, variance). |
| >> SUPI list (1.. SUPImax) | List of SUPI(s) for each application |
| >> Ratio | Estimated percentage of UEs with similar service experience (in the group, or among all UEs). |
| >> Spatial validity | Area where the estimated Service Experience applies. If Area of Interest information was provided in the request or subscription, spatial validity may be a subset of the requested Area of Interest. |
| >> Validity period | Validity period as defined in clause 6.1.3. |
| Slice service experience | Service experience across applications on a Network Slice over the Analytics target period (average, variance). |
| NOTE: If multiple Network Slice instances are not deployed for the S-NSSAI or NSI IDs are not available, only one Service experience entry is provided (i.e. n=1). In that case, the NSI ID is not provided and the Slice instance service experience indicates the service experience for the S-NSSAI. |  |

Table 6.4.3-2: Service Experience predictions

| Information | Description |
| --- | --- |
| S-NSSAI | Identifies the Network Slice for which analytics information is provided. |
| Service experiences (1…n) (NOTE) | List of observed service experience information for each Network Slice instance. |
| > NSI ID | Identifies the Network Slice instance within the Network Slice |
| > Slice instance service experience | Service experience across Applications on a Network Slice instance instance over the Analytics target period (average, variance). |
| > Application ServiceExperiences (1..max) | List of predicted service experience information for each Application. |
| >> Application ID | Identification of the application. |
| >> Service Experience Type | Type of Service Experience analytics, e.g. on voice, video, other. |
| >> Service Experience | Service Experience over the Analytics target period (average, variance). |
| >> SUPI list (1.. SUPImax) | List of SUPI(s) for each application. |
| >> Ratio | Estimated percentage of UEs with similar service experience (in the group, or among all UEs). |
| >> Spatial validity | Area, where the estimated Service Experience applies. If Area of Interest information was provided in the request or subscription, spatial validity may be a subset of the requested Areas of Interest. |
| >> Validity period | Validity period as defined in clause 6.1.3. |
| >> Probability assertion | Confidence of this prediction. |
| Slice service experience | Service experience across applications on a Network Slice over the Analytics target period (average, variance). |
| NOTE: If multiple Network Slice instances are not deployed for the S-NSSAI or NSI IDs are not available, only one Service experience entry is provided (i.e. n=1). In that case, the NSI ID is not provided and the Slice instance service experience indicates the service experience for the S-NSSAI. |  |

NOTE:	In case Network Slice instances are not deployed or NSI IDs are not available, the size of the list is n=1. In that case, the NSI ID and slice instance service experience entries in Table 6.4.3-1 and Table 6.4.3-2 are optional.

The number of Service Experiences and SUPIs is limited by the maximum number of results provided as input parameter.

* * * * Next Change * * * *

### 6.4.5	Procedures to request Service Experience for a Network Slice

Figure 6.4.5-1: Procedure for NWDAF providing Service Experience for a UE or a group of UEs in a Network Slice

This procedure is similar to the procedure in clause 6.4.4, with the following differences. The consumer needs to request the Analytics ID "Service Experience" for all UEs or a group of UEs on a Network Slice, identified by an S-NSSAI. If multiple Network Slice instances of the same Network Slice are deployed, associated NSI ID(s) may be used in addition to S-NSSAI. NWDAF may subscribe to UE mobility event notifications of AMF as described in clause 5.3.4.4 of TS 23.501 [2] using event ID "Area of Interest" and event filters as described in Table 5.2.2.3.1-1 of TS 23.502 [3] if it is needed to retrieve the list of SUPIs (and GPSIs if available). The event exposure service request also includes the immediate reporting flag as event reporting information as described in Table 4.15.1-1 of TS 23.502 [3]. In addition, service experience data may need to be collected from multiple Applications. If each Application is hosted in a separate AF, NWDAF subscribes the service data in the Table 6.4.2-1 from the different AFs by invoking Nnef_EventExposure_Subscribe or Naf_EventExposure_Subscribe services for each Application (Event ID = Service Data, Event Filter information = (Application ID, Area of Interest), Target of Event Reporting = Any UE) as defined TS 23.502 [3]. Figure 6.4.5-1 shows an example procedure with two AFs.

If one AF provides the service experience data of multiple Applications, the set of Application IDs is provided by NWDAF to the AF with the Naf_EventExposure_Subscribe service operation, as defined TS 23.502 [3].

The Observed Service Experience for a Network Slice when consumed by OAM could be used as described in Annex H of TS 28.550 [7].

* * * * End of Changes * * * *