3GPP TSG-WG SA2 Meeting #137E e-meeting 	S2-2001864r03

# Elbonia, February 24 – 27, 2020	(revision of S2-200xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.287 | CR | 0088 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of PDB definition |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eV2XARC |  |  |  |  |  | Date: |  |  | 2020-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The current definition of PC5 PDB is based on the following definition of PDB in clause 5.7.3.4 of TS 23.501: "The Packet Delay Budget (PDB) defines an upper bound for the time that a packet may be delayed between the UE and the UPF that terminates the N6 interface." Since the PC5 PDB is between UEs over PC5, the definition is not correct, even if in TS 23.287 it is specified that "the PDB […] does not contain any CN delay component". because also the NG-RAN is not part of the PDU transmission path. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The definition of PDB over PC5 is corrected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wrong definition of PC5 PDB. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.4.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 5.4.3.4	Packet Delay Budget

The Packet Delay Budget (PDB) is defined to be an upper bound for the time a packet may be delayed between the UEs during a successful transmission over PC5, starting with the sending UE's V2X layer receiving the V2X message from the upper layers and ending with the V2X message being successfully sent to the upper layer in the receiving UE(s) (i.e., between the V2X Layer of  related UEs).

NOTE 1:	How to determine the time consumption of receiving UE is up to UE implementation.

# * * * * End of changes * * * *