3GPP TSG-SA2 Meeting #137-E	S2-2001875

# Online, , 24th Feb 2020 - 27th Feb 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.287 | CR | 0091 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | "served by and not served by" clarification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo Mobile Communication Co. LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | 3GPP SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eV2XARC |  |  |  |  |  | Date: |  |  | 2020-02-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Parameter provisioning consists of defining parameters based on if a UE is served by E-UTRA / NR or not, it assumes if a UE is served by E-UTRA or NR it will use PC5 paramaters provided by the broadcast channel. However a UE may use PC5 in another frequency that is not the operating frequency of the cell or that advertised by the broadcast channel. The issue seems to arise from an unclear definition of what “Served by” or “not served by means |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a NOTE which describes that [When the UE is "served by E-UTRA" or "served by NR"] and [When the UE is "not served by E-UTRA" and "not served by NR"] are relevant to V2X communications over PC5 reference point. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wording would be ambiguous. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

5.1.2.1	Policy/Parameter provisioning

The following sets of information for V2X communications over PC5 reference point is provisioned to the UE:

1)	Authorization policy:

-	When the UE is "served by E-UTRA" or "served by NR" for the V2X communication over PC5 reference point:

-	PLMNs in which the UE is authorized to perform V2X communications over PC5 reference point when "served by E-UTRA" or "served by NR".

For each above PLMN:

-	RAT(s) over which the UE is authorized to perform V2X communications over PC5 reference point.

-	When the UE is "not served by E-UTRA" and "not served by NR" for the V2X communication over PC5 reference point:

-	Indicates whether the UE is authorized to perform V2X communications over PC5 reference point when "not served by E-UTRA" and "not served by NR".

-	RAT(s) over which the UE is authorized to perform V2X communications over PC5 reference point.

2)	Radio parameters when the UE is "not served by E-UTRA" and "not served by NR":

-	Includes the radio parameters per PC5 RAT (i.e. LTE PC5, NR PC5) with Geographical Area(s) and an indication of whether they are "operator managed" or "non-operator managed". The UE uses the radio parameters to perform V2X communications over PC5 reference point when "not served by E-UTRA" and "not served by NR" only if the UE can reliably locate itself in the corresponding Geographical Area. Otherwise, the UE is not authorized to transmit.

Editor's note:	The radio parameters (e.g. frequency bands) are to be defined by RAN WGs. The reference to RAN specification will be added when defined in RAN WGs.

NOTE 2:	Whether a frequency band is "operator managed" or "non-operator managed" in a given Geographical Area is defined by local regulations.

3)	Policy/parameters per RAT for PC5 Tx Profile selection:

-	The mapping of V2X service types (e.g. PSIDs or ITS-AIDs) to Tx Profiles.

Editor's note:	The Tx Profiles are to be defined by RAN WGs. The reference to RAN specification will be added when defined in RAN WGs.

4)	Policy/parameters related to privacy:

-	The list of V2X service types, e.g. PSIDs or ITS-AIDs of the V2X applications, with Geographical Area(s) that require privacy support.

5)	Policy/parameters when LTE PC5 is selected:

Same as specified in TS 23.285 [8] clause 4.4.1.1.2 item 3) Policy/parameters except for the mapping of V2X service types to Tx Profiles and the list of V2X services with Geographical Area(s) that require privacy support.

6)	Policy/parameters when NR PC5 is selected:

-	The mapping of V2X service types (e.g. PSIDs or ITS-AIDs) to V2X frequencies with Geographical Area(s).

-	The mapping of Destination Layer-2 ID(s) and the V2X service types, e.g. PSIDs or ITS-AIDs of the V2X application for broadcast.

-	The mapping of Destination Layer-2 ID(s) and the V2X service types, e.g. PSIDs or ITS-AIDs of the V2X application for groupcast.

-	The mapping of default Destination Layer-2 ID(s) for initial signalling to establish unicast connection and the V2X service types, e.g. PSIDs or ITS-AIDs of the V2X application.

NOTE 3:	The same default Destination Layer-2 ID for unicast initial signalling can be mapped to more than one V2X service types. In the case where different V2X services are mapped to distinct default Destination Layer-2 IDs, when the UE intends to establish a single unicast link that can be used for more than one V2X service types, the UE can select any of the default Destination Layer-2 IDs to use for the initial signalling.

-	PC5 QoS mapping configuration:

-	Input from V2X application layer:

-	V2X service type (e.g. PSID or ITS-AID).

-	(Optional) V2X Application Requirements for the V2X service type, e.g. priority requirement, reliability requirement, delay requirement, range requirement.

NOTE 4:	Details of V2X Application Requirements for the V2X service type is up to implementation and out of scope of this specification.

-	Output:

-	PC5 QoS parameters defined in clause 5.4.2 (i.e. PQI and conditionally other parameters such as MFBR/GFBR, etc).

-	SLRB configurations, i.e. the mapping of PC5 QoS profile(s) to SLRB(s), when the UE is "not served by E-UTRA" and "not served by NR".

-	The PC5 QoS profile contains PC5 QoS parameters described in clause 5.4.2, and value for the QoS characteristics regarding Priority Level, Averaging Window, Maximum Data Burst Volume if default value is not used as defined in Table 5.4.4-1.

7)	Validity timer indicating the expiration time of the V2X Policy/Parameter.

Editor's note:	The SLRB configurations will be determined by RAN WGs. The reference to RAN specification will be added when defined in RAN WGs.

Editor's note:	For the PC5 QoS profile, coordination with RAN WGs is needed.

Editor's note:	The V2X frequencies with Geographical Area(s) will be determined by RAN WGs. The reference to RAN specification will be added when defined in RAN WGs.

The above parameter sets from bullet 2) to 6) may be configured in the UE through the V1 reference point by the V2X Application Server.