3GPP TSG-SA2 Meeting #137E	S2-2001881

# Feb 24-27, 2020, Electronic Meeting 	(revision of S2-2001266, 0283)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1978 | rev | 2 | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Emergency service at EPC-5GC interworking |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1 |  |  |  |  |  | Date: |  |  | 2020-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In SA2#136, S-NSSAI setting in AMF is clarified in 23.501CR1984 (S2-1912193, Rel-15) and CR1985 (S2-1912194, Rel-16), however the necessary update is still missing in the procedures. For the following procedures, considering that AMF may or may not be able to get the S-NSSAI from the HSS+UDM, it’s proposed that the AMF use the S-NSSAI locally configured in the Emergency Configuration Data to simplify. - EPS to 5GS mobility without N26; - Handover from EPS to 5GC/N3IWF; - Handover from EPC/ePDG to 5GS In addition, how 5GC IWK indication is set in EPC and how EPC IWK indication is set in 5GC for emergency PDU Session is also missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Note that changes to clauses 4.11.4.3.3a (new), 4.11.4.3.5 and 4.11.4.3.6 are only applicable for Rel-16, and changes to other clauses are mirror of Rel-15 CR1977. #1 The AMF uses the S-NSSAI in the Emergency Configuration Data in the following mobility procedures: - EPS to 5GS mobility without N26; - Handover from EPS to 5GC/N3IWF; - Handover from EPC/ePDG to 5GS. (For EPS to 5GS mobility with N26, PGW-C+SMF returns S-NSSAI to AMF as for non-emergency PDU Session). #2 At emergency PDN connection setup in EPC: 1-1 Clarify that MME or ePDG sets the 5GC IWK indication based on UE’s 5GC NAS capability and local configuration. 1-2 Clarify the PGW-C+SMF handles S-NSSAI as follows: Only one S-NSSAI shall be configured in PGW-C+SMF for the emergency APN; No check against the UDM is needed (UE may not be authenticated) S-NSSAI is not sent to UE in PCO. 1-3 Clarify that MME or ePDG may select combined PGW-C+SMF based on Emergency Configuration Data configured in MME or ePDG, which may contain statically configured emergency PGW-C+SMF. #3 At emergency PDU Session setup in 5GC, The AMF sets EPC IWK indication based on UE’s S1 mode support and local configuration. Rev2: Clause 4.11.5.3, correct the description for PDU Session with Request type “initial emergency request”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Emergency service not properly specified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.11.0a.5, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * First Changes * * * *

#### 4.11.0a.5	PDN Connection Establishment

During PDN connection establishment in the EPC, the UE and the PGW-C+SMF exchange information via PCO as described in TS 23.501 [2] clause 5.15.7. In case the PGW-C+SMF supports more than one S-NSSAI and the APN is valid for more than one S-NSSAI, before the PGW-C+SMF provides an S-NSSAI to the UE, the PGW-C+SMF should check such that the selected S-NSSAI is among the UE's subscribed S-NSSAIs, and that the S-NSSAI is not subject to Network Slice-Specific Authentication and Authorization, by retrieving the Subscribed S-NSSAI from UDM using the Nudm_SDM_Get service operation (the PGW-C+SMF discovers and selects a UDM as described in TS 23.501 [2] clause 6.3.8). If the PGW-C+SMF is in a VPLMN, the PGW-C+SMF uses the Nnssf_NSSelection_Get service operation to retrieve a mapping of the Subscribed S-NSSAIs to Serving PLMN S-NSSAI values. If the S-NSSAIs supported by the PGW-C+SMF are all subject to NSSAA, then the PGW-C+SMF should reject the PDN connection establishment.

If PGW-C+SMF is selected and interaction with UDM, PCF and UPF is required for a UE that has 5GS subscription but does not support 5GC NAS and is accessing via EPC/E-UTRAN, the PGW-C+SMF creates unique PDU Session ID for each PDN connection of the UE. If the PGW-C+SMF supports more than one S-NSSAI and the APN is valid for more than one S-NSSAI, the SMF+PGW-C may proceed as specified in first paragraph of this clause or select any S-NSSAI associated with the APN of the PDN connection. The PGW-C+SMF shall not provide any 5GS related parameters to the UE.

The unique PDU Session ID can be created based on the EPS Bearer IDs assigned by the MME for the PDN Connections associated with the UE and not be in the range of PDU Session ID values that can be created by a 5GC NAS capable UE.

During establishment of emergency PDN connection,

- The PGW-C+SMF is to be derived from the emergency APN or to be statically configured in the Emergency Configuration Data in MME;

- If the emergency APN is valid for more than one S-NSSAI, PGW-C+SMF selects an S-NSSAI based on local policy.

NOTE:	The PGW-C+SMF knows that the UE does not support 5GS NAS if the UE does not provide PDU Session ID in PCO (see TS 23.501 [2] clause 5.15.7).

-

-

* * * * End of Changes * * * *