3GPP TSG-WG SA2 Meeting #137E e-meeting 	S2-2001901

# Elbonia, February 24 – 27, 2020	(revision of S2-2001351, 1289)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0110 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on UE communication analytics type by NWDAF service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA |  |  |  |  |  | Date: |  |  | 2019-12-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For output analytics result, it’s possible that not all the indicated ratio UEs within the group own the same analytics communication result, i.e. a list of (predicted) communication time slots. For example, within the group, UE 1, UE 3 and UE 5 own the same list of (predicted) communication time slots, and UE 2, UE 10 and UE 12 own the same list of (predicted) communication time slots. However, no such possibility described in TS 23.288 yet. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify that it’s possible that not all the indicated ratio UEs within the group ID own the same analytics mobility result. In this case, a UE group ID subset should be provided in the analytics to indicate all the UEs in the UE group ID subset have the same analytics. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete requirement of UE communication |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.3.2; 6.7.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.7.3.2	Input Data

The NWDAF supporting data analytics on UE communication shall be able to collect communication information for the UE from 5GC. The detailed information collected by the NWDAF includes service data related to UE communication as defined in the Table 6.7.3.2-1.

Table 6.7.3.2-1: Service Data from 5GC related to UE communication

| Information | Source | Description |
| --- | --- | --- |
| UE ID | SMF, AF | SUPI in the case of SMF, GPSI or external UE ID in the case of AF |
| Group ID | SMF, AF | To identify UE group if available Internal Group ID in the case of SMF, External Group ID in the case of AF |
| S-NSSAI | SMF | Information to identify a Network Slice |
| DNN | SMF | Data Network Name where PDU connectivity service is provided |
| Application ID | SMF, AF | Identifying the application providing this information |
| Communication pattern parameters | AF | Same as communication pattern specified in TS 23.502 [3] |
| UE communication (1..max) | UPF, AF | Communication description per application |
| >Communication start |  | The time stamp that this communication starts |
| >Communication stop |  | The time stamp that this communication stops |
| >UL data rate |  | UL data rate of this communication |
| >DL data rate |  | DL data rate of this communication |
| >Traffic volume |  | Traffic volume of this communication |
| Type Allocation code (TAC) | AMF | To indicate the terminal model and vendor information of the UE. The UEs with the same TAC may have similar communication behavior. The UE whose communication behavior is unlike other UEs with the same TAC may be an abnormal one. |

NOTE:	How NWDAF collects UE communication related data is not defined in this Release of the specification.

Depending on the requested level of accuracy, data collection may be provided on samples (e.g. spatial subsets of UEs or UE group, temporal subsets of UE communication information).

The application Id is optional. If the application Id is omitted, the collected UE communication information can be applicable to all the applications for the UE.

# * * * * Second change * * * *

6.7.3.3	Output Analytics

The NWDAF supporting UE Communication Analytics provides the analytics results to consumer NFs. The analytics results provided by the NWDAF include the UE communication statistics as defined in Table 6.7.3.3-1 or predictions as defined in Table 6.7.3.3-2.

Table 6.7.3.3-1: UE Communication Statistics

| Information | Description |
| --- | --- |
| UE group ID subset or UE ID | Identifies an UE or UEs within the requested group, e.g. internal group ID defined in TS 23.501 [2] clause 5.9.7 or SUPI. (NOTE) |
| UE communications (1..max) | List of communication time slots. |
| > Periodic communication indicator | Identifies whether the UE communicates periodically or not. |
| > Periodic time | Interval Time of periodic communication (average and variance) if periodic. Example: every hour |
| > Start time | Start time observed (average and variance) |
| > Duration time | Duration interval time of communication (average and variance). |
| > Traffic characterization | S-NSSAI, DNN, ports, other useful information. |
| > Traffic volume | Volume UL/DL (average and variance). |
| > Ratio | Percentage of UEs in the group (in case of an UE group). |

Table 6.7.3.3-2: UE Communication Predictions

| Information | Description |
| --- | --- |
| UE group ID subset or UE ID | Identifies an UE or UEs within the requested group, e.g. internal group ID defined in TS 23.501 [2] clause 5.9.7 or SUPI. (NOTE) |
| UE communications (1..max) | List of communication time slots. |
| > Periodic communication indicator | Identifies whether the UE communicates periodically or not. |
| > Periodic time | Interval Time of periodic communication (average and variance) if periodic. Example: every hour. |
| > Start time | Start time predicted (average and variance). |
| > Duration time | Duration interval time of communication. |
| > Traffic characterization | S-NSSAI, DNN, ports, other useful information. |
| > Traffic volume | Volume UL/DL (average and variance). |
| > Confidence | Confidence of the prediction. |
| > Ratio | Percentage of UEs in the group (in case of an UE group). |

NOTE:	When target of analytics reporting is an individual UE or a group of UE, one UE ID (i.e. SUPI) or UE group ID will be included, the NWDAF will provide the analytics communication result (i.e. list of (predicted) communication time slots) to NF service consumer(s) for the UE (i.e. SUPI) or a UE group ID subset (i.e. the UE group ID in case all the UEs within the requested UE group shares the same analytics or SUPIs within the requested UE group if some UEs within the requested UE group shares the same analytics).

The results for UE groups address the group globally. The ratio is the proportion of UEs in the group for a given communication at a given time and duration.

The number of UE communication entries (1..max) is limited by the maximum number of results provided as input parameter. The communications shall be provided by order of time, possibly overlapping.

Depending on the list size limitation, the least probable communications on a given Analytics target period may not be provided.

# * * * * End of changes * * * *