3GPP TSG-WG SA2 Meeting #137E e-meeting	S2-2001903r08

Elbonia, February 24 – 27, 2020	(revision of xx-yyxxxx)

# Source:	China Mobile

# Title:	New WID on N7/N40 Interfaces Enhancements to Support GERAN and UTRAN

# Document for:	Approval

Agenda Item:	9.1

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title: N7/N40 Interfaces Enhancements to Support GERAN and UTRAN

## Acronym: NIESGU

## Unique identifier: 	{A number to be provided by MCC at the plenary}

Potential target Release: Rel-17.

Note that this field above indicates the proposed Release at the time of submission of the WID to TSG approval. It can later be changed without a need to revise the WID. The updated target Release is indicated in the Work Plan.

## 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  | X |
| Don't know |  |  |  |  |  |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a …

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

### 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

### 2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
|  |  | {optional free text} |

Dependency on non-3GPP (draft) specification:

## 3	Justification

To simplify the network architecture and network management when 5GS is deployed and 2G or 3G still exists in their network, operators would like to be able to deploy a combined node serving as both SMF+PGW-C and PGW-C, to simultaneously serve 2G/3G/4G/5G users.

However, as shown in Table 1, per current 3GPP specification, the policy control interface and charging interfaces differ.

Table 1: Policy control interface and charging interface as defined in the current specification

| Scenario | Entity | Policy Control Reference Point | Charging Reference Point | Note |
| --- | --- | --- | --- | --- |
| 4G and 2/3G | PGW | Gx | Gy | Diameter based interface using the DRAs for diameter signalling routing |
| 4G and 5G | PGW-C+SMF | N7 | N40 | Service based interfaces Nchf using NRFs for 5GC NFs discovery and selection |

To simplify the network deployments and reduce CAPEX and OPEX for operators, it is beneficial to enable the use of the N7 interface for policy control in 2/3/4/5G accesses, and enable the use of the N40 interface for charging in 2/3/4/5G accesses, the main impact being that PGW supports N7 and N40 interfaces.

This will beneficially allow PCF and CHF to handle UEs using 2G/3G and therefore separate PCRF and OCS/OFCS are not required. This will also allow the PCF and CHF to be used to handle UEs roaming onto 2G/3G access in a different PLMN.

It is not the aim of the WID to change how LBO currently works.

## 4	Objective

This work item aims at specifying a mechanism that would allow to extend the interfaces on N7/N40 reference points  to support 2G/3G. The work may contain the following aspects:

- Support of RAT Type for 2G/3G on interface extension for N7/N40 reference points.

- Support of location change report of RAI, and SGSN level for 2G/3G on interface extension for N7/N40 reference points.

- Selection of PGW supporting N7 vs. those supporting Gx/Gy without impact on UE.

- Creation of PDU Session ID for the UE by the PGW.

Interworking between 2G/3G and 4G/5G is out of scope.

New inter PLMN interfaces or extensions, in particular on Nchf, are out of scope for this work item.

The expected time for completing this work is 0.7 TU.

## 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 23.060 23.401 23.503 23.203 |  | TSG#88 | This will be handled as “TEI17”. |

## 6	Work item Rapporteur(s)

Aihua, Li, China Mobile, liaihua@ chinamobile.com

## 7	Work item leadership

SA2.

## 8	Aspects that involve other WGs

The Charging aspects will be covered by SA5.

## 9	Supporting Individual Members

Supporting IM name

China Mobile

Ericsson

China Unicom

Orange

Huawei

CATT

ZTE

Samsung

CAICT

Deutsche Telekom

Matrixx