SA WG2 Meeting #S2-137E	S2-2002047

24 – 27 February, 2020, Electronic Meeting                                        (revision of S2-2000524)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 372 | rev | 2 | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MDBV mapping and configuration for TSC QoS Flow |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2020-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | See S2-1911701 for details. MDBV identifies the maximum cumulative payload (from all TSN streams that map to a TSN traffic class and its corresponding 5G QoS flow) that can be sent per instance of traffic transmission over the radio interface for any given 5G QoS flow and therefore limits the throughput of a QoS flow. At the 5GS QoS capability report phase, currently MDBV is not part of QoS capability report (sent to the AF), and it is not part of minimum set of TSN QoS-related parameters for QoS mapping either. Although MDBV is part of 5QI, but only the bridge delay and priority related parameters are reported to AF during the QoS capability report phase. The mapping of a TSC traffic class or TSC streams to a 5G QoS flow requires to match bridge delay, priority, and MDBV in the same time. During the capability report phase, the MDBV value of a reported 5QI has been decided already. There is a risk that the maximum value of TSC burst size of a traffic class (aggregated TSC streams) is larger than any reported MDBV of 5QIs. Therefore, it is not possible to find a 5QI that matches MDBV and in the same time also matches bridge delay and priority requirements for the corresponding traffic class. If AF doesn’t provide TSC burst size information, the default value for the TSC 5QI may not be enough to accommodate aggregated TSC streams. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In case the Maximum Burst Size of the aggregated TSC streams in the traffic class is provided by CNC via TSN AF to PCF, PCF can derive the required MDBV taking the Maximum Burst Size and TSCAI as input. If the default MDBV associated with a 5QI in the QoS mapping table can’t satisfy the aggregated TSC Burst Size, the PCF provides the derived MDBV in the PCC rule and then the SMF performs QoS Flow binding. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification of the TSN-5GS QoS mapping leading to stage 3 delay. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.1, 6.2.1.2, 6.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** BEGIN CHANGES ***

#### 6.1.3.23	Support of integration with Time Sensitive Networking

Time Sensitive Networking (TSN) support is defined in TS 23.501 [2], where the 5GS represents virtual TSN bridge(s) based on the defined granularity model. The TSN AF and PCF interact to perform QoS mapping as described in clause 5.28.4 of TS 23.501 [2].

The PCF provides the following parameters to the TSN AF: Port Management Container, port numbers associated with the NW-TT and DS-TT, and a UE MAC address (i.e. MAC address of the DS-TT port). The TSN AF may use this information to construct IEEE managed objects, to interwork with IEEE TSN networks.

The TSN AF decides the TSN QoS information (i.e. priority, aggregated TSC burst size and delay) based on the received the configuration information of 5GS Bridge from the CNC as defined in clause 5.28.2 of TS 23.501 [2] and the bridge delay information at the TSN AF.

The PCF receives a request from the TSN AF that includes UE MAC address (i.e. MAC address of the DS-TT port) for PDU session and the TSN QoS parameters, i.e. priority, aggregated TSC burst size and delay. The PCF performs Session binding using the UE MAC address, and then the PCF derives the TSN QoS parameters into a 5QI. The PCF generates a PCC Rule with service data flow filter containing the UE MAC address and the mapped 5QI. The SMF binds the PCC Rule to a QoS Flow as defined in clause 6.1.3.2.4.

*** END CHANGES ***