SA WG2 Meeting #S2-137e	S2-2002073

E-Meeting, 24-28 February 2020	(revision of )

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.287 | CR | 0099 | rev |  | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on Link identifier update for unicast link procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eV2XARC |  |  |  |  |  | Date: |  |  | 2020-02-18 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For privacy reason, clause 6.3.3.2 introduces a link identifier update for a unicast link procedure in TS 23.287. The PC5 link identifier is unique in a V2X UE, this procedure called link identifier update maybe cause a misunderstanding that the PC5 link identifier can be changed, so it is proposed to rename the “link identifier update procedure” to “layer-2 link identifier update procedure” |  |  |  |  |  |  |  |  |
| Summary of change: |  | Rename the link identifier update to the layer-2 link indetifier update. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It may cause the PC5 link identifier changes. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1.4, 6.3.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

5.2.1.4	Unicast mode communication over PC5 reference point

Unicast mode of communication is only supported over NR based PC5 reference point. Figure 5.2.1.4-1 illustrates an example of PC5 unicast links.

Figure 5.2.1.4-1: Example of PC5 Unicast Links

The following principles apply when the V2X communication is carried over PC5 unicast link:

-	A PC5 unicast link between two UEs allows V2X communication between one or more pairs of peer V2X services in these UEs. All V2X services in the UE using the same PC5 unicast link use the same Application Layer ID.

NOTE 1:	An Application Layer ID can change in time as described in clauses 5.6.1.1 and 6.3.3.2, due to privacy. This does not cause a re-establishment of a PC5 unicast link. The UE triggers a Layer-2 link Identifier Update procedure as specified in clause 6.3.3.2.

-	One PC5 unicast link supports one or more V2X service types (e.g. PSIDs or ITS-AIDs) if these V2X service types are at least associated with the pair of peer Application Layer IDs for this PC5 unicast link. For example, as illustrated in Figure 5.2.1.4-1, UE A and UE B have two PC5 unicast links, one between peer Application Layer ID 1/UE A and Application Layer ID 2/UE B and one between peer Application Layer ID 3/UE A and Application Layer ID 4/UE B.

NOTE 2:	A source UE is not required to know whether different target Application Layer IDs over different PC5 unicast links belong to the same target UE.

-	A PC5 unicast link supports V2X communication using a single network layer protocol e.g. IP or non-IP.

-	A PC5 unicast link supports per-flow QoS model as specified in clause 5.4.1.

When the Application layer in the UE initiates data transfer for a V2X service type which requires unicast mode of communication over PC5 reference point:

-	the UE shall reuse an existing PC5 unicast link if the pair of peer Application Layer IDs and the network layer protocol of this PC5 unicast link are identical to those required by the application layer in the UE for this V2X service, and modify the existing PC5 unicast link to add this V2X service type as specified in clause 6.3.3.4; otherwise

-	the UE shall trigger the establishment of a new PC5 unicast link as specified in clause 6.3.3.1.

After successful PC5 unicast link establishment, UE A and UE B use the same pair of Layer-2 IDs for subsequent PC5-S signalling message exchange and V2X service data transmission as specified in clause 5.6.1.4. The V2X layer of the transmitting UE indicates to the AS layer whether a transmission is for a PC5-S signalling message (i.e. Direct Communication Request/Accept, Link Identifier Update Request/Response, Disconnect Request/Response, Link Modification Request/Accept) or V2X service data.

For every PC5 unicast link, a UE self-assigns a distinct PC5 Link Identifier that uniquely identifies the PC5 unicast link in the UE for the lifetime of the PC5 unicast link. Each PC5 unicast link is associated with a Unicast Link Profile which includes:

-	V2X service type(s) (e.g. PSID(s) or ITS-AID(s)); and

-	Application Layer ID and Layer-2 ID of UE A; and

-	Application Layer ID and Layer-2 ID of UE B; and

-	network layer protocol used on the PC5 unicast link; and

-	for each V2X service type, a set of PC5 QoS Flow Identifier(s) (PFI(s)). Each PFI is associated with QoS parameters (i.e. PQI).

For privacy reason, the Application Layer IDs and Layer-2 IDs may change as described in clauses 5.6.1.1 and 6.3.3.2 during the lifetime of the PC5 unicast link and, if so, shall be updated in the Unicast Link Profile accordingly. The UE uses PC5 Link Identifier to indicate the PC5 unicast link to V2X Application layer, therefore V2X Application layer identifies the corresponding PC5 unicast link even if there are more than one unicast link associated with one V2X service type (e.g. the UE establishes multiple unicast links with multiple UEs for a same V2X service type).

The Unicast Link Profile shall be updated accordingly after a Layer-2 link modification for an established PC5 unicast link as specified in clause 6.3.3.4 or Layer-2 link identifier update as specified in clause 6.3.3.2.

V2X Service Info and QoS Info are carried in PC5-S signalling messages and exchanged between two UEs as specified in clause 6.3.3. Based on the exchanged information, PFI is used to identify V2X service. When the receiving UE receives V2X service data over the established PC5 unicast link, the receiving UE determines the appropriate V2X service based on the PFI to forward the received V2X service data to the upper layer.

* * * Next Changes * * *

6.3.3.2	Layer-2 link identifier update for a unicast link

Figure 6.3.3.2-1 shows the layer-2 link identifier update procedure for a unicast link. Due to the privacy requirements, identifiers used for unicast mode of V2X communication over PC5 reference point (e.g. Application Layer ID, Source Layer-2 ID and IP address/prefix) shall be changed over time as specified in clauses 5.6.1.1 and 5.6.1.4. This procedure is used to update the peer UE for a unicast link of the impending change of the identifiers used for this link before the identifier changes happen, to prevent service interruptions.

If a UE has multiple unicast links using the same Application Layer IDs or Layer-2 IDs, the UE needs to perform the layer-2 link identifier update procedure over each of the unicast link.

Figure 6.3.3.2-1: Layer-2 link identifier update procedure

0.	UE-1 and UE-2 have a unicast link established as described in clause 6.3.3.1.

1.	UE-1 decides the change of identifiers, e.g. due to the Application Layer ID change or upon expiry of a timer, and send a Link Identifier Update Request message to UE-2 before it changes the identifiers.

The Link Identifier Update Request message includes the new identifiers to use (including the new Application Layer ID, new Layer-2 ID, new IP address/prefix if IP communication is used). The new identifiers should be cyphered to protect privacy.

NOTE:	The timer is running on per Source Layer-2 ID.

2.	UE-2 responds with a Link Identifier Update Response message. Upon reception of the message, UE-1 and UE-2 start to use the new identifiers for the data traffic. UE-1 shall receive traffic on its old Layer-2 ID until it receives the Link Identifier Update Response message from UE-2.

The V2X layer of each UE passes the PC5 Link Identifier for the unicast link and the updated Layer-2 ID (i.e. source Layer-2 ID for UE-1 while destination Layer-2 ID for UE-2) down to the AS layer. This enables the AS layer to update the provided Layer-2 ID for the unicast link.

* * * End of Changes * * *