3GPP TSG-WG SA2 Meeting #137E e-meeting 	S2-2002153

# Elbonia, February 24 – 27, 2020	(revision of S2-2001611)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2028 | rev | 4 | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the 5G VN usage of IP Multicast mechanisms from TS 23.316 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [Nokia, Nokia Shanghai Bell], Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2020-02-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 5.8.2.13.3.1 refers to 23.316 for the handling of IP Multicast. The conditions need to be clarified e.g. that this applies for any UE on any access and is not meant for IPTV services. Clarify when and where PIM signalling is exchanged in case of IP Multicast Clarify how to apply IGMP/MLD signalling to UE members of 5G VN group |  |  |  |  |  |  |  |  |
| Summary of change: |  | See Reason for change |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Confusion on the cases where § 5.8.2.13.3.1 can apply, inconsistent support of IGMP/MLD signalling at § 5.8.2.13.3.1 and § 5.8.2.13.3.2 and possibly mis understanding at stage 3 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.2.13.3.1, 5.8.2.13.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1 - Wording: They are not limited to Wireline, is not meant for IPTV services - 5.8.2.13.3.0 General removed and modified text put in 5.8.2.13.3.1 Rev2 Text on PIM slimmed down Rev4 Clarification on how to apply IGMP/MLD to § 5.8.2.13.3.2 |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.8.2.13.3	Support for user plane traffic replication in a 5G VN

5.8.2.13.3.1	User plane traffic replication based on UPF internal functionality

For Ethernet PDU Sessions, the SMF may instruct the UPF to route traffic to be replicated as described in clause 5.8.2.5.

For IP PDU Session types, the SMF may instruct the UPF to manage IP multicast traffic as described in TS 23.316 [84] clauses 4.6.6 and 7.7.1. The UPF replicates the IP multicast traffic received from PDU Sessions or N6 interface and sends the packets over other PDU Sessions and other N6 interface subscribed to the IP Multicast groups.

Mechanisms described in TS 23.316 [84] clauses 4.6.6 and 7.7.1 apply to support 5G VN group communication with following clarifications:

- These mechanisms are not limited to Wireline access and can apply on any access,

- IP Multicast traffic allowed for a PDU Session is not meant for IPTV services reachable over N6,

- IGMP /MLD signalling does not relate with STB or 5G-RG: TS 23.316 [84] clauses 4.6.6 and 7.7.1, apply to UE members of a 5G VN group instead of 5G-RG, and

- TS 23.316 [84] clauses 7.7.1.1.2 and 7.7.1.1.4 are not applicable to 5G VN groups: members of the 5G VN groups may receive any multicast traffic associated with the (DNN, S-NSSAI) of the 5G VN group.

- UPF exchange of signalling such as PIM (Protocol-Independent Multicast) may apply as defined in TS 23.316, with following clarification:

-	PIM signalling is generally exchanged over N6 but may be sent towards the PDU Session supporting the source address of multicast traffic identified by IGMP / MLD signalling for Source Specific Multicast. In case of IGMP / MLD signalling not related with Source Specific Multicast no PIM signalling is sent towards any PDU Session

# * * * * Second change * * * *

5.8.2.13.3.2	User plane traffic replication based on PDRs with replication instructions

Alternatively, for IP or Ethernet type data communication, the SMF instructs the UPF via PDRs and FARs how to replicate user plane traffic.

The mechanism is supported in the following conditions:

-	When N19 is used, there is a full mesh of N19 tunnels between UPFs serving the 5G VN group;

-	There is no support of forwarding packets with destination MAC address not known by SMF/UPF (i.e. no support for new UE MAC addresses from the UE during the PDU Session lifetime)

-	There is no support for forwarding a broadcast/multicast packet with source address not known to SMF/UPF.

-	Each UPF supports one N6 interface instance towards the data network, or only supports N19-based forwarding without N6;

-	Multicast group formation of selected members of a 5G VN for Ethernet type data communication is not described in this release of the specification.

In this case, when the UPF receives a broadcast packet of a 5G VN group from N19 or N6, it shall distribute it to all 5G VN group members connected to this UPF. When the UPF receives a broadcast packet from a UE (source UE) via PDU Session associated with a 5G VN group, it shall distribute it to:

-	All 5G VN group members (except the source UE) connected to this UPF via local switch, and

-	All 5G VN group members connected to other UPFs via N19-based forwarding, and

-	The devices on the DN via N6-based forwarding.

To enable broadcast traffic forwarding of a 5G VN group in a UPF, the following applies:

-	The SMF provides group-level N4 Session and each 5G VN group member' N4 Session with the PDR that detect the broadcast packet sent via "internal interface". When UPF receives the broadcast packets sent via "internal interface", it matches the broadcast packet against all PDRs installed at the "internal interface". A successful matching with a PDR that detect the broadcast packet instructs the UPF to continue the lookup of the other PDRs without higher precedence. A matching PDR that detects the broadcast packet shall instruct the UPF to duplicate the broadcast packet and perform processing (using associated FAR, URR, QER) on the copy instead of the original packet if the broadcast packet does not satisfy the packet replication skip information, otherwise the PDR instructs the UPF to skip the processing of the broadcast packet.

-	The broadcast packets received from N19 or N6 are forwarded to the UPF internal interface together with a N19 or N6 indication, GTP-U header can carry the N19 or N6 indication.

-	The SMF provides for each 5G VN group member' N4 Session (i.e. N4 session corresponding to PDU Session) the following N4 rules that enable the processing of broadcast packets towards this UE.

-	in order to detect the traffic, a PDR containing Source Interface set to "5G VN internal", Destination Address set to the broadcast address, the Packet replication skip information set to the IP/MAC address (es) of this 5G VN group member, and the indication to carry on matching; and

-	in order to forward the traffic, a FAR containing Outer Header Creation indicating the PDU Session tunnel information, and Destination Interface set "access side".

-	The SMF configures the group-level N4 Session for processing packets received from a N19 tunnel with the following N4 rules for each N19 tunnel.

-	in order to detect the traffic, a PDR containing Source Interface set to "core side", Destination Address set to the broadcast address, and CN Tunnel Information set to N19 tunnel header (i.e., N19 GTP-U TEID); and

-	in order to forward the traffic, a FAR containing Destination Interface set to "5G VN internal", Outer Header Creation with the N19 indication.

-	The SMF provides for the group-level N4 Session the following N4 rules that enable the processing of broadcast packets towards the other UPFs.

-	in order to detect the traffic, a PDR containing Source Interface set to "5G VN internal", Destination Address set to the broadcast address, the Packet replication skip information set to the N19 indication, and the indication to carry on matching; and

-	in order to forward the traffic to each involved UPF via the corresponding N19 tunnel, a FAR containing "Duplication" instruction, Outer Header Creation indicating the N19 tunnel information, Destination Interface set to "core side".

-	The SMF configures the group-level N4 Session for processing packets received from N6 with the following N4 rules.

-	in order to detect the traffic, a PDR containing Source Interface set to "core side", and Destination Address set to the broadcast address; and

-	in order to forward the traffic, a FAR containing Destination Interface set to "5G VN internal", Outer Header Creation with the N6 indication.

-	The SMF provides for the group-level N4 Session the following N4 rules that enable the processing of broadcast packets towards N6.

-	in order to detect the traffic, a PDR containing Source Interface set to "5G VN internal", a match-all packet filter, and the Packet replication skip information set to the N6 indication; and

-	in order to forward the traffic to N6, a FAR containing Destination Interface set to "core side".

In this case, to enable multicast traffic forwarding of a 5G VN group in a UPF, broadcast traffic forwarding of a 5G VN applies to multicast traffic forwarding of a 5G VN with the following modifications:

-	The SMF installs PDRs for the multicast address instead of the broadcast address.

-	The PDRs and FARs are installed for PDU Sessions corresponding to the members of the multicast group.

-	In case of IP Multicast traffic, IGMP /MLD mechanism in TS 23.316 [84] clauses 4.6.6 and 7.7.1 apply to UE members of a 5G VN group instead of 5G-RG:

-	The PDRs that detect the IP Multicast traffic contains the ACTIVE indication or INACTIVE indication. The UPF will skip the PDR with INACTIVE indication and use the PDR with ACTIVE indication in the PDRs lookup process. During PDU Session establishment procedure, the SMF configures the PDRs that detect the IP Multicast traffic with INACTIVE indication in the N4 Session corresponding to the PDU Session.

-	The FARs that handle the IGMP/MLD messages use "Action" value “5GLAN Multicast Join Accept”, and “5GLAN Multicast Leave Accept”. “5GLAN Multicast Join Accept” indicates the UPF to accept the multicast join and activate the PDR of the requested multicast address (i.e., change the INACTIVE indication to ACTIVE indication), “5GLAN Multicast Leave Accept” indicates the UPF to accept the multicast leave and de-activate the PDR of the requested multicast address (i.e., change the ACTIVE indication to INACTIVE indication).

# * * * * Third change * * * *

5.8.2.11.3	Packet Detection Rule

The following table describes the Packet Detection Rule (PDR) containing information required to classify a packet arriving at the UPF. Every PDR is used to detect packets in a certain transmission direction, e.g. UL direction or DL direction.

Table 5.8.2.11.3-1: Attributes within Packet Detection Rule

| Attribute |  | Description | Comment |
| --- | --- | --- | --- |
| N4 Session ID |  | Identifies the N4 session associated to this PDR. NOTE 5. |  |
| Rule ID |  | Unique identifier to identify this rule. |  |
| Precedence |  | Determines the order, in which the detection information of all rules is applied. |  |
| Status indication |  | Contains the values “ACTIVE", "INACTIVE". (NOTE 6) | The UPF will skip the PDR with INACTIVE indication and use the PDR with ACTIVE indication in the PDRs lookup process. |
| Packet | Source interface | Contains the values "access side", "core side", "SMF", "N6-LAN" , "5G VN internal". | Combination of UE IP address (together with Network instance, if necessary), CN tunnel info, |
| Detection | UE IP address | One IPv4 address and/or one IPv6 prefix with prefix length (NOTE 3). | packet filter set, application ID, Ethernet PDU Session |
| Information. NOTE 4. | Network instance (NOTE 1) | Identifies the Network instance associated with the incoming packet. | Information, QoS Monitoring Packet indicator and QFI are used for traffic detection. Source interface identifies the |
|  | CN tunnel info | CN tunnel info on N3, N9 interfaces, i.e. F-TEID. | interface for incoming packets |
|  | Packet Filter Set | Details see clause 5.7.6. | where the PDR applies, e.g. from access side (i.e. up-link), |
|  | Application ID |  | from core side (i.e. down-link), |
|  | QoS Flow ID | Contains the value of 5QI or non-standardized QFI. | from SMF, from N6-LAN (i.e. the |
|  | Ethernet PDU Session Information | Refers to all the (DL) Ethernet packets matching an Ethernet PDU session, as further described in clause 5.6.10.2 and in TS 29.244 [65]. | DN or the local DN), or from "5G VN internal" (i.e. local switch). |
|  | Framed Route Information | Refers to Framed Routes defined in clause 5.6.14. | Details like all the combination possibilities on N3, N9 interfaces are left for stage 3 decision. |
|  | QoS Monitoring Packet indicator (NOTE 8) | Identifies the packet is used for QoS Monitoring as defined in clause 5.33.3. |  |
| Packet replication and detection carry on information | Packet replication skip information NOTE 7 | Contains UE address indication or N19/N6 indication. If the packet matches the packet replication skip information, i.e., source address of the packet is the UE address or the packet has been received on the interface in the packet replication skip information, the UP function neither creates a copy of the packet nor applies the corresponding processing (i.e., FAR, QER, URR). Otherwise the UPF performs a copy and applies the corresponding processing (i.e., FAR, QER, URR). |  |
| NOTE 6 | Carry on indication | Instructs the UP function to continue the packet detection process, i.e., lookup of the other PDRs without higher precedence. |  |
| Outer header removal |  | Instructs the UP function to remove one or more outer header(s) (e.g. IP+UDP+GTP, IP + possibly UDP, VLAN tag), from the incoming packet. | Any extension header shall be stored for this packet. |
| Forwarding Action Rule ID (NOTE 2) |  | The Forwarding Action Rule ID identifies a forwarding action that has to be applied. |  |
| Multi-Access Rule ID (NOTE 2) |  | The Multi-Access Rule ID identifies an action to be applied for handling forwarding for a MA PDU Session. |  |
| List of Usage Reporting Rule ID(s) |  | Every Usage Reporting Rule ID identifies a measurement action that has to be applied. |  |
| List of QoS Enforcement Rule ID(s) |  | Every QoS Enforcement Rule ID identifies a QoS enforcement action that has to be applied. |  |
| NOTE 1: Needed e.g. in case: - UPF supports multiple DNN with overlapping IP addresses; - UPF is connected to other UPF or AN node in different IP domains. - UPF "local switch", N6-based forwarding and N19 forwarding is used for different 5G LAN groups. NOTE 2: Either a FAR ID or a MAR ID is included, not both. NOTE 3: The SMF may provide an indication asking the UPF to allocate one IPv4 address and/or IPv6 prefix. When asking to provide an IPv6 Prefix the SMF provides also an IPv6 prefix length. NOTE 4: When in the architecture defined in clause 5.34, a PDR is sent over N16a from SMF to I-SMF, the Packet Detection Information may indicate that the I-SMF is to locally determine CN tunnel info in order to build the N4 PDR sent to the actual UPF controlled by the I-SMF. This is further defined in clause 5.34.6. NOTE 5: In the architecture defined in clause 5.34, the rules exchanged between I-SMF and SMF are not associated with a N4 Session ID but are associated with a N16a association. NOTE 6: Needed in case of support for broadcast/multicast traffic forwarding using packet replication with SMF-provided PDRs and FARs as described in clause 5.8.2.13.3.2. NOTE 7: Needed in the case of packet replication with SMF-provided PDRs and FARs as described in clause 5.8.2.13.3.2, to prevent UPF from sending the broadcast/multicast packets back to the source UE or source N19/N6. NOTE 8: The UPF identifies the UL packet or dummy packet to be the QoS Monitoring packet based on the QoS Monitoring Packet indicator encapsulated in the GTP-U header by NG-RAN as defined in clause 5.33.3. |  |  |  |

# * * * * End of changes * * * *