SA WG2 Meeting #137E	S2-2002212r02

# Elbonia, February 24 – 27, 2020 	(revision of S2-19yyyyy)

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2198 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Vertical_LAN TSN related CR for non-FASMO corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Tencent, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2020-02-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Following items: - Missing abbreviations - Redundant text in the description regarding integration with IEEE TSN. - IEEE Std 802.1Qbv-2016 has been rolled into IEEE Std 802.1Q-2018 ,so the reference to th IEEE802.1Qbu is replaced by IEE 802.1Q ,so the table number in the 802.1Qbv is replaced with the table number in the IEEE 802.1Q - Term alignment: “5GS xxx bridge” -> “5GS Bridge”, “AF” -> “TSN AF” - Bridge name exists in the spec but it is unclear how it is generated, used, sent etc |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduces: - Abbreviations missing for TSN - Retained one occurrence with appropriate reference to IEEE TSN, removed the duplicates. - The reference to th IEEE802.1Qbv is replaced by IEE 802.1Q ,so the table number in the 802.1Qbv is replaced with the table number in the IEEE 802.1Q - Term alignment: “5GS xxx bridge” -> “5GS Bridge”, “AF” -> “TSN AF” - Remove Bridge name. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing abbreviations, duplicate text etc. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.2, 4.4.8.1, 4.4.8.2, 5.27.4, 5.28.2, 5.28.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for next generation new services and markets; Stage 1".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 23.203: "Policies and Charging control architecture; Stage 2".

[5]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS); Stage 2".

[6]	3GPP TS 24.011: "Point-to-Point (PP) Short Message Service (SMS) support on mobile radio interface: Stage 3".

[7]	IETF RFC 7157: "IPv6 Multihoming without Network Address Translation".

[8]	IETF RFC 4191: "Default Router Preferences and More-Specific Routes".

[9]	IETF RFC 2131: "Dynamic Host Configuration Protocol".

[10]	IETF RFC 4862: "IPv6 Stateless Address Autoconfiguration".

[11]	ITU-T Recommendation I.130: "Method for the characterization of telecommunication services supported by an ISDN and network capabilities of an ISDN".

[12]	ITU-T Recommendation Q.65: "The unified functional methodology for the characterization of services and network capabilities".

[13]	3GPP TS 24.301: "Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS): Stage 3".

[14]	IETF RFC 3736: "Stateless DHCP Service for IPv6".

[15]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[16]	3GPP TS 22.173: "IMS Multimedia Telephony Service and supplementary services; Stage 1".

[17]	3GPP TS 23.122: "Non-Access-Stratum (NAS) functions related to Mobile Station in idle mode".

[18]	3GPP TS 23.167: "3rd Generation Partnership Project; Technical Specification Group Services and Systems Aspects; IP Multimedia Subsystem (IMS) emergency sessions".

[19]	3GPP TS 23.003: "Numbering, Addressing and Identification".

[20]	IETF RFC 7542: "The Network Access Identifier".

[21]	3GPP TS 23.002: "Network Architecture".

[22]	3GPP TS 23.335: "User Data Convergence (UDC); Technical realization and information flows; Stage 2".

[23]	3GPP TS 23.221: "Architectural requirements".

[24]	3GPP TS 22.153: "Multimedia priority service".

[25]	3GPP TS 22.011: "Service Accessibility".

[26]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[27]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description".

[28]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol Specification".

[29]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[30]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[31]	3GPP TS 37.340: "Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Multi-connectivity; Stage 2".

[32]	3GPP TS 23.214: "Architecture enhancements for control and user plane separation of EPC nodes; Stage 2".

[33]	3GPP TS 22.101: "3rd Generation Partnership Project; Technical Specification Group Services and Systems Aspects; Service aspects; Service principles".

[34]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[35]	3GPP TS 33.126: "Lawful Interception Requirements".

[36]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

[37]	3GPP TS 22.280: "Mission Critical Services Common Requirements (MCCoRe); Stage 1".

[38]	3GPP TS 23.379: "Functional architecture and information flows to support Mission Critical Push To Talk (MCPTT); Stage 2".

[39]	3GPP TS 23.281: "Functional architecture and information flows to support Mission Critical Video (MCVideo); Stage 2".

[40]	3GPP TS 23.282: "Functional architecture and information flows to support Mission Critical Data (MCData); Stage 2".

[41]	3GPP TS 32.240: "Charging management; Charging architecture and principles".

[42]	3GPP TS 38.401: "NG-RAN Architecture description".

[43]	3GPP TS 23.402: "Architecture enhancements for non-3GPP accesses".

[44]	IETF RFC 4960: "Stream Control Transmission Protocol".

[45]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[46]	3GPP TS 23.041: "Public Warning System".

[47]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[48]	3GPP TS 24.502: "Access to the 5G System (5GS) via non-3GPP access networks; Stage 3".

[49]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[50]	3GPP TS 38.304: "NR; User Equipment (UE) procedures in idle mode".

[51]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[52]	3GPP TS 36.304: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) procedures in idle mode".

[53]	Void.

[54]	IETF RFC 4861: "Neighbor Discovery for IP version 6 (IPv6)".

[55]	3GPP TS 23.271: "Functional stage 2 description of Location Services (LCS)".

[56]	3GPP TS 23.060: "General Packet Radio Service (GPRS); Service description; Stage 2".

[57]	IETF RFC 4555: "IKEv2 Mobility and Multihoming Protocol (MOBIKE)".

[58]	3GPP TS 29.510: "5G System: Network function repository services; Stage 3".

[59]	3GPP TS 29.502: "5G System: Session Management Services: Stage 3".

[60]	IETF RFC 7296: "Internet Key Exchange Protocol Version 2 (IKEv2) ".

[61]	3GPP TS 23.380: "IMS Restoration Procedures".

[62]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[63]	3GPP TS 23.292: "IP Multimedia Subsystem (IMS) centralized services; Stage 2".

[64]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs".

[65]	3GPP TS 29.244: "Interface between the Control Plane and the User Plane Nodes; Stage 3".

[66]	3GPP TS 32.421: "Telecommunication management; Subscriber and equipment trace; Trace concepts and requirements".

[67]	3GPP TS 32.290: "5G system; Services, operations and procedures of charging using Service Based Interface (SBI)".

[68]	3GPP TS 32.255: "5G Data connectivity domain charging; Stage 2".

[69]	3GPP TS 38.306: "NR; User Equipment -UE) radio access capabilities".

[70]	3GPP TS 36.306: "Evolved Universal Terrestrial Radio Access -E-UTRA); User Equipment -UE) radio access capabilities".

[71]	3GPP TS 29.518: "5G System; Access and Mobility Management Services; Stage 3".

[72]	3GPP TS 23.285: "Architecture enhancements for V2X services".

[73]	IETF RFC 2865: "Remote Authentication Dial In User Service (RADIUS)".

[74]	IETF RFC 3162: "RADIUS and IPv6".

[75]	3GPP TS 29.281: "General Packet Radio System (GPRS) Tunnelling Protocol User Plane (GTPv1-U)".

[76]	3GPP TS 26.238: "Uplink streaming".

[77]	3GPP TR 26.939: "Guidelines on the Framework for Live Uplink Streaming (FLUS)".

[78]	International Telecommunication Union (ITU), Standardization Bureau (TSB): "Operational Bulletin No. 1156"; http://handle.itu.int/11.1002/pub/810cad63-en (retrieved October 5, 2018).

[79]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[80]	3GPP TS 24.250: "Protocol for Reliable Data Service; Stage 3".

[81]	draft-ietf-mptcp-rfc6824bis-18: "TCP Extensions for Multipath Operation with Multiple Addresses".

Editor's note:	The above document cannot be formally referenced until it is published as an RFC.

[82]	draft-ietf-tcpm-converters-14: "0-RTT TCP Convert Protocol".

Editor's note:	The above document cannot be formally referenced until it is published as an RFC.

[83]	IEEE 802.1CB-2017: "IEEE Standard for Local and metropolitan area networks-Frame Replication and Elimination for Reliability".

[84]	3GPP TS 23.316: "Wireless and wireline convergence access support for the 5G System (5GS)".

[85]	WiFi Alliance Technical Committee, Hotspot 2.0 Technical Task Group: "Hotspot 2.0 (Release 2) Technical Specification".

[86]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[87]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[88]	3GPP TS 23.216: "Single Radio Voice Call Continuity (SRVCC); Stage 2".

[89]	CableLabs DOCSIS MULPI: "Data-Over-Cable Service Interface Specifications DOCSIS 3.1, MAC and Upper Layer Protocols Interface Specification".

[90]	BBF TR-124 issue 5: "Functional Requirements for Broadband Residential Gateway Devices".

[91]	BBF TR-101 issue 2: "Migration to Ethernet-Based Broadband Aggregation".

[92]	BBF TR-178 issue 1: "Multi-service Broadband Network Architecture and Nodal Requirements".

[93]	BBF WT-456: "AGF Functional Requirements".

[94]	BBF WT-457: "FMIF Functional Requirements".

Editor's note:	The references to BBF WT-456 and WT-457 will be revised when finalized by BBF.

[95]	IEEE P802.1Qcc: "Standard for Local and metropolitan area networks - Bridges and Bridged Networks - Amendment: Stream Reservation Protocol (SRP) Enhancements and Performance Improvements".

[96]	void.

[97]	IEEE Std 802.1AB-2016: "IEEE Standard for Local and metropolitan area networks -- Station and Media Access Control Connectivity Discovery".

[98]	IEEE P802.1Q: "Standard for Local and metropolitan area networks--Bridges and Bridged Networks".

[99]	3GPP TS 38.423: "NG-RAN; Xn Application Protocol (XnAP)".

[100]	3GPP TS 36.413: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 Application Protocol (S1AP)".

[101]	3GPP TS 29.274: "Evolved General Packet Radio Service (GPRS) Tunnelling Protocol for Control plane (GTPv2-C); Stage 3".

[102]	3GPP TS 23.632: "User Data Interworking, Coexistence and Migration; stage 2".

[103]	3GPP TS 29.563: "5G System (5GS); HSS services for interworking with UDM; Stage 3".

[104]	IEEE Std 802.1AS-Rev/D7.3, August 2018: "IEEE Standard for Local and metropolitan area networks--Timing and Synchronization for Time-Sensitive Applications".

[105]	3GPP TS 22.104: "Service requirements for cyber-physical control applications in vertical domains".

[106]	IEEE Std 802.11-2012: "IEEE Standard for Information technology - Telecommunications and information exchange between systems - Local and metropolitan area networks - Specific requirements - Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications".

[107]	IEEE 1588-2008: "IEEE Standard for a Precision Clock Synchronization Protocol for Networked Measurement and Control".

[108]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[109]	3GPP TS 24.193: "Access Traffic Steering, Switching and Splitting; Stage 3".

[110]	3GPP TS 24.526: "User Equipment (UE) policies for 5G System (5GS); Stage 3".

[111]	3GPP TS 22.186: "Enhancement of 3GPP support for V2X scenarios; Stage 1".

[112]	3GPP TR 38.824: "Study on physical layer enhancements for NR ultra-reliable and low latency case (URLLC)".

[113]	IEEE: "Guidelines for Use of Extended Unique Identifier (EUI), Organizationally Unique Identifier (OUI), and Company ID (CID)", https://standards.ieee.org/content/dam/ieee-standards/standards/web/documents/tutorials/eui.pdf.

[114]	3GPP TS 32.256: "Charging Management; 5G connection and mobility domain charging; Stage 2".

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

5GC	5G Core Network

5GLAN	5G Local Area Network

5GS	5G System

5G-AN	5G Access Network

5G-EIR	5G-Equipment Identity Register

5G-GUTI	5G Globally Unique Temporary Identifier

5G-BRG	5G Broadband Residential Gateway

5G-CRG	5G Cable Residential Gateway

5G-RG	5G Residential Gateway

5G-S-TMSI	5G S-Temporary Mobile Subscription Identifier

5G GM	5G Grand Master

5QI	5G QoS Identifier

AF	Application Function

AMF	Access and Mobility Management Function

AS	Access Stratum

ATSSS	Access Traffic Steering, Switching, Splitting

ATSSS-LL	ATSSS Low-Layer

AUSF	Authentication Server Function

BMCA	Best Master Clock Algorithm

BSF	Binding Support Function

CAG	Closed Access Group

CAPIF	Common API Framework for 3GPP northbound APIs

CHF	Charging Function

CN PDB	Core Network Packet Delay Budget

CP	Control Plane

DL	Downlink

DN	Data Network

DNAI	DN Access Identifier

DNN	Data Network Name

DRX	Discontinuous Reception

DS-TT	Device-side TSN translator

ePDG	evolved Packet Data Gateway

EBI	EPS Bearer Identity

EUI	Extended Unique Identifier

FAR	Forwarding Action Rule

FN-BRG	Fixed Network Broadband RG

FN-CRG	Fixed Network Cable RG

FN-RG	Fixed Network RG

FQDN	Fully Qualified Domain Name

GFBR	Guaranteed Flow Bit Rate

GMLC	Gateway Mobile Location Centre

GPSI	Generic Public Subscription Identifier

GUAMI	Globally Unique AMF Identifier

HR	Home Routed (roaming)

IAB	Integrated access and backhaul

I-SMF	Intermediate SMF

LADN	Local Area Data Network

LBO	Local Break Out (roaming)

LMF	Location Management Function

LoA	Level of Automation

LPP	LTE Positioning Protocol

LRF	Location Retrieval Function

MCX	Mission Critical Service

MDBV	Maximum Data Burst Volume

MFBR	Maximum Flow Bit Rate

MICO	Mobile Initiated Connection Only

MPS	Multimedia Priority Service

MPTCP	Multi-Path TCP Protocol

N3IWF	Non-3GPP InterWorking Function

N5CW	Non-5G-Capable over WLAN

NAI	Network Access Identifier

NEF	Network Exposure Function

NF	Network Function

NGAP	Next Generation Application Protocol

NID	Network identifier

NPN	Non-Public Network

NR	New Radio

NRF	Network Repository Function

NSI ID	Network Slice Instance Identifier

NSSAA	Network Slice-Specific Authentication and Authorization

NSSAI	Network Slice Selection Assistance Information

NSSF	Network Slice Selection Function

NSSP	Network Slice Selection Policy

NW-TT	Network-side TSN translator

NWDAF	Network Data Analytics Function

PCF	Policy Control Function

PDB	Packet Delay Budget

PDR	Packet Detection Rule

PDU	Protocol Data Unit

PEI	Permanent Equipment Identifier

PER	Packet Error Rate

PFD	Packet Flow Description

PPD	Paging Policy Differentiation

PPF	Paging Proceed Flag

PPI	Paging Policy Indicator

PSA	PDU Session Anchor

PTP	Precision Time Protocol

QFI	QoS Flow Identifier

QoE	Quality of Experience

RACS	Radio Capabilities Signalling optimisation

(R)AN	(Radio) Access Network

RG	Residential Gateway

RIM	Remote Interference Management

RQA	Reflective QoS Attribute

RQI	Reflective QoS Indication

RSN	Redundancy Sequence Number

SA NR	Standalone New Radio

SBA	Service Based Architecture

SBI	Service Based Interface

SCP	Service Communication Proxy

SD	Slice Differentiator

SEAF	Security Anchor Functionality

SEPP	Security Edge Protection Proxy

SMF	Session Management Function

SMSF	Short Message Service Function

SN	Sequence Number

SNPN	Stand-alone Non-Public Network

S-NSSAI	Single Network Slice Selection Assistance Information

SSC	Session and Service Continuity

SSCMSP	Session and Service Continuity Mode Selection Policy

SST	Slice/Service Type

SUCI	Subscription Concealed Identifier

SUPI	Subscription Permanent Identifier

SV	Software Version

TAC	IMEI Type Allocation Code

TNAN	Trusted Non-3GPP Access Network

TNAP	Trusted Non-3GPP Access Point

TNGF	Trusted Non-3GPP Gateway Function

TNL	Transport Network Layer

TNLA	Transport Network Layer Association

TSC	Time Sensitive Communication

TSCAI	TSC Assistance Information

TSN	Time Sensitive Networking

TSN GM	TSN Grand Master

TSP	Traffic Steering Policy

TT	TSN Translator

UCMF	UE radio Capability Management Function

UDM	Unified Data Management

UDR	Unified Data Repository

UDSF	Unstructured Data Storage Function

UL	Uplink

UL CL	Uplink Classifier

UPF	User Plane Function

URLLC	Ultra Reliable Low Latency Communication

URRP-AMF	UE Reachability Request Parameter for AMF

URSP	UE Route Selection Policy

VID	VLAN Identifier

VLAN	Virtual Local Area Network

W-5GAN	Wireline 5G Access Network

W-5GBAN	Wireline BBF Access Network

W-5GCAN	Wireline 5G Cable Access Network

W-AGF	Wireline Access Gateway Function

### 4.4.8	Time Sensitive Communication

#### 4.4.8.1	General

The 5G System is extended to support Time sensitive communication as defined in IEEE P802.1Qcc [95].

In this Release of the specification, integration of 5G System with TSN networks that are based on IEEE TSN (IEEE P802.1Qcc [95]) is supported. IEEE TSN is a set of standards to define mechanisms for the time-sensitive (i.e. deterministic) transmission of data over Ethernet networks.

#### 4.4.8.2	Architecture to support Time Sensitive Communication

The 5G System is integrated with the external network as a TSN bridge. This "logical" TSN bridge (see Figure 4.4.8.2-1) includes TSN Translator functionality for interoperation between TSN System and 5G System both for user plane and control plane. 5GS TSN translator functionality consists of Device-side TSN translator (DS-TT) and Network-side TSN translator (NW-TT). 5G System specific procedures in 5GC and RAN, wireless communication links, etc. remain hidden from the TSN network. To achieve such transparency to the TSN network and the 5GS to appear as any other TSN Bridge, the 5GS provides TSN ingress and egress ports via DS-TT and NW-TT. DS-TT and NW-TT optionally support:

-	hold and forward functionality for the purpose of de-jittering;

-	per-stream filtering and policing as defined in IEEE 802.1Q [98] clause 8.6.5.1.

DS-TT optionally supports link layer connectivity discovery and reporting as defined in IEEE 802.1AB [97] for discovery of Ethernet devices attached to DS-TT. NW-TT supports link layer connectivity discovery and reporting as defined in IEEE 802.1AB [97] for discovery of Ethernet devices attached to NW-TT. If a DS-TT does not support link layer connectivity discovery and reporting, then NW-TT performs link layer connectivity discovery and reporting as defined in IEEE 802.1AB [97] for discovery of Ethernet devices attached to DS-TT on behalf of DS-TT.

NOTE 1:	If NW-TT performs link layer connectivity discovery and reporting on behalf of DS-TT, it is assumed that LLDP frames are transmitted between NW-TT and UE on the default QoS Flow. Alternatively, SMF can establish a dedicated QoS Flow matching on the Ethertype defined for LLDP (IEEE 802.1AB [97]).

There are three TSN configuration models defined in IEEE P802.1Qcc [95]. Amongst the three models:

-	fully centralized model is supported in this release of the specification;

-	fully distributed model is not supported in this release of the specification;

-	hybrid model is not supported in this Release of the specification.

NOTE 2:	This release only supports interworking with TSN using IEEE 802.1Q [98] clause 8.6.8.4 based scheduled traffic and IEEE 802.1Q [98] clause 8.6.5.1 based per-stream filtering and policy.

Figure 4.4.8.2-1: System architecture view with 5GS appearing as TSN bridge

NOTE 3:	Whether DS-TT and UE are combined or are separate is up to implementation.

## 5.27	Time Sensitive Communications

### 5.27.0	General

This clause describes 5G System features that support TSC and allow the 5G System to be integrated transparently as a bridge in an IEEE TSN network.

During the PDU Session establishment, the UE shall request to establish a PDU Session as an always-on PDU Session, and the PDU Sessions used for TSC are established as Always-on PDU session as described in clause 5.6.13. In this release of the specification:

-	Home Routed PDU Sessions are not supported for TSC services;

-	TSC PDU Sessions are supported only with PDU Session type Ethernet and SSC mode 1;

-	Service continuity for TSC PDU Sessions is not supported when the UE moves from 5GS to EPS.

### 5.27.1	TSN Time Synchronization

#### 5.27.1.1	General

For supporting TSN time synchronization, the 5GS is integrated with the external network as a TSN bridge as described in clauses 4.4.8 and 5.28.1. It shall be modelled as an IEEE 802.1AS [104] compliant entity according to TS 22.104 [105]. For TSN Synchronization, the entire E2E 5G system can be considered as an IEEE 802.1AS [104] "time-aware system". Only the TSN Translators (TTs) at the edges of the 5G system need to support the IEEE 802.1AS [104] operations. UE, gNB, UPF, NW-TT and DS- TTs are synchronized with the 5G GM (i.e. the 5G internal system clock) which shall serve to keep these network elements synchronized. The TTs located at the edge of 5G system fulfil all functions related to IEEE 802.1AS [104], e.g. (g)PTP support, timestamping, Best Master Clock Algorithm (BMCA), rateRatio. Figure 5.27.1-1 illustrates the 5G and TSN clock distribution model via 5GS.

Figure 5.27.1-1: 5G system is modelled as IEEE 802.1AS compliant time aware system for supporting TSN time synchronization

Figure 5.27.1-1 depicts the two synchronizations systems considered: the 5GS synchronization and the TSN domain synchronization, as well as the Master (M) and Slave (S) ports considered when the TSN GM is located at TSN working domain.

-	5GS synchronization: Used for NG RAN synchronization. 5G RAN synchronization is specified in TS 38.331 [28].

-	TSN domain synchronization: Provides synchronization service to TSN network. This process follows IEEE 802.1AS [104].

The two synchronization processes can be considered independent from each other and the gNB only needs to be synchronized to the 5G GM clock.

To enable TSN synchronization, the 5GS calculates and adds the measured residence time between the TTs into the Correction Field (CF) of the synchronization packets of the TSN domain.

#### 5.27.1.2	Distribution of timing information

##### 5.27.1.2.1	Distribution of 5G internal system clock

The 5G internal system clock shall be made available to all user plane nodes in the 5G system. The UPF and NW-TT may get the 5G internal system clock via the underlying PTP compatible transport network with mechanisms outside the scope of 3GPP. The 5G internal system clock shall be made available to UE with signaling of time information related to absolute timing of radio frames as described in TS 38.331 [28]. The 5G internal system clock shall be made available to DS-TT by the UE.

##### 5.27.1.2.2	Distribution of TSN clock and time-stamping

The mechanisms for distribution of TSN clock and time-stamping described in this clause are according to IEEE 802.1AS [104].

Upon reception of a downlink gPTP message the NW-TT makes an ingress timestamping (TSi) for each gPTP event (Sync) message and uses the cumulative rateRatio received inside the gPTP message payload (carried within Sync message for one-step operation or Follow_up message for two-step operation) to calculate the link delay from the upstream TSN node (gPTP entity) expressed in TSN GM time as specified in IEEE 802.1AS [104]. NW-TT then calculates the new cumulative rateRatio (i.e. the cumulative rateRatio of the 5GS) as specified in IEEE 802.1AS [104] and modifies the gPTP message payload (carried within Sync message for one-step operation or Follow_up message for two-step operation) as follows:

-	Adds the link delay from the upstream TSN node in TSN GM time to the correction field.

-	Replaces the cumulative rateRatio received from the upstream TSN node with the new cumulative rateRatio.

-	Adds TSi in the Suffix field of the gPTP packet as described in Annex H.

UPF then forwards the gPTP message to the UE via user plane (i.e. using the PDU session applicable for sending gPTP messages). Only one PDU session per UE per UPF is used for sending gPTP messages regardless of how many external TSN working domains have their clock information delivered through a given UPF serving that UE. All gPTP messages are transmitted on a QoS Flow that complies with the residence time upper bound requirement specified in IEEE 802.1AS [104].

NOTE:	The sum of the UE-DS-TT residence time and the PDB of the QoS Flow needs to be lower than the residence time upper bound requirement for a time-aware system specified in IEEE 802.1AS [104].

A UE receives the gPTP messages and forwards them to the DS-TT. The DS-TT then creates egress timestamping (TSe) for the gPTP event (Sync) messages for external TSN working domains. The difference between TSi and TSe is considered as the calculated residence time spent within the 5G system for this gPTP message expressed in 5GS time. The DS-TT then uses the rateRatio contained inside the gPTP message payload (carried within Sync message for one-step operation or Follow_up message for two-step operation) to convert the residence time spent within the 5GS in TSN GM time and modifies the payload of the gPTP message that it sends towards the downstream TSN node as follows:

-	Adds the calculated residence time expressed in TSN GM time to the correction field.

-	Removes TSi from the Suffix field.

#### 5.27.1.3	Support for multiple TSN working domains

Each TSN working clock domain sends its own gPTP messages. The related Ethernet frames carry the gPTP multicast Ethernet destination MAC address and the gPTP message carries a specific PTP "domainNumber" that indicates the time domain they are referring to. The NW-TT makes ingress timestamping (TSi) for the gPTP event messages of all domains.

A UE receives gPTP messages and forwards them all to the DS-TT. The DS-TT receives the original TSN clock timing information and the corresponding TSi via gPTP messages for one or more working domains. The DS-TT then makes egress timestamping (TSe) for the gPTP event messages for every external TSN working domain. Ingress and egress time stamping is based on the 5G system clock at NW-TT and DS-TT.

NOTE 1:	An end-station can select TSN timing information of interest based on the "domainNumber" in the gPTP message.

The process described in "Distribution of TSN clock and time-stamping" is thus repeated for each TSN working domain between a UE and the UPF it is connected to. When a UE is connected to multiple TSN working domains via the same UPF, then all gPTP messages from these domains are forwarded to the UE via the same PDU session.

NOTE 2:	If all TSN working domains can be made synchronous and the synchronization can be provided by the 5G clock, the NW-TT output ports towards the connected TSN networks propagate the 5G clock using the 802.1AS profile (i.e. the 5G system as an IEEE 802.1AS [104] compliant time-aware system).

### 5.27.1a	Periodic deterministic QoS

This feature allows the 5GS to support periodic deterministic communication where the traffic characteristics are known a-priori, and a schedule for transmission from the UE to a downstream node, or from the UPF to an upstream node is provided via external protocols outside the scope of 3GPP (e.g. IEEE TSN).

The features include the following:

-	Providing TSC Assistance Information (TSCAI) that describe TSC flow traffic patterns at the gNB ingress and UE egress interfaces for traffic in downlink and uplink direction, respectively;

-	Support for hold & forward buffering mechanism in DS-TT and NW-TT to de-jitter flows that have traversed the 5G System.

### 5.27.2	TSC Assistance Information (TSCAI)

TSC assistance information describes TSC traffic characteristics for use in the 5G System. The knowledge of TSN traffic pattern is useful for the gNB to allow it to more efficiently schedule periodic, deterministic traffic flows either via Configured Grants, Semi-Persistent Scheduling or with dynamic grants. TSC assistance information, as defined in Table 5.27.2-1, is provided from SMF to 5G-AN, e.g. upon QoS Flow establishment. The TSCAI parameters are set according to corresponding parameters obtained from the TSN AF. The TSN AF gets per TSN stream traffic characteristics according to IEEE 802.1Q [98] clause 8.6.5.1 for 5GS bridge to identify the PDU session for the UL traffic.

The determination of the TSCAI by the 5GC is based on information received from the TSN AF via PCF. The TSN AF is responsible for obtaining PSFP (IEEE 802.1Q [98]) parameters and use them to calculate traffic pattern parameters (such as burst arrival time with reference to the ingress port, periodicity, and flow direction) and responsible of forwarding these parameters to the SMF (via PCF). TSN AF may aggregate TSN streams if the TSN streams belong to the same traffic class, terminate in the same egress port and have the same periodicity.

Annex I describe how the traffic pattern information is determined.

NOTE 1:	Further details of aggregation of TSN streams are left for implementation.

The TSN AF may aggregate multiple TSN Streams if they have the same periodicity and traffic class. In this case, TSN AF creates one TSN QoS container for the aggregated TSN streams. 5GC derives TSCAI on a per QoS Flow basis and send it to 5G-AN. The Burst Arrival Time and Periodicity component of the TSCAI that the SMF signals to the 5G-AN are specified with respect to the 5G clock. The 5GC is responsible for mapping the Burst Arrival Time and Periodicity from a TSN clock (that the TSN stream is based on) to the 5G clock based on the time offset and cumulative rateRatio between TSN time and 5GS time as measured and reported by the UPF. 5GC sets the DL TSCAI Burst Arrival Time as the sum of TSN QoS Burst Arrival Time and CN PDB. 5GC sets the UL TSCAI Burst Arrival Time as the sum of TSN QoS Burst Arrival Time and UE-DS-TT Residence Time. 5GC sets the Periodicity as the TSN QoS Periodicity.

NOTE 2:	In order to get Burst Arrival Time, Periodicity on a per TSN stream basis, support for IEEE 802.1Q [98] (as stated in clause 4.4.8.2) Per-Stream Filtering and Policing (PSFP) with stream gate operation is a prerequisite.

In the case of drift between TSN time and 5G time, the UPF updates the offset to SMF. In the case of change of cumulative rateRatio between TSN time and 5G time, the UPF updates the cumulative rateRatio to SMF. The SMF may then trigger a PDU Session Modification as defined in TS 23.502 [3] clause 4.3.3 in order to update the TSCAI parameter to the NG-RAN without requiring AN or N1 specific signalling exchange with the UE.

NOTE 3:	In order to prevent frequent updates from the UPF, the UPF sends the offset or the cumulative rateRatio only when the difference between the current measurement and the previously reported measurement is larger than a threshold.

Table 5.27.2-1: TSC Assistance Information

| Assistance Information | Description |
| --- | --- |
| Flow Direction | The direction of the TSC flow (uplink or downlink). |
| Periodicity | It refers to the time period between start of two bursts. |
| Burst Arrival time | The arrival time of the data burst at either the ingress of the RAN (downlink flow direction) or egress interface of the UE (uplink flow direction). |

### 5.27.3	Support for TSC QoS Flows

TSC QoS Flows use a Delay Critical GBR resource type and TSC Assistance Information. TSC QoS Flows may use standardized 5QIs, pre-configured 5QIs or dynamically assigned 5QI values (which requires signalling of QoS characteristics as part of the QoS profile) as specified in clause 5.7.2. For each instance of Periodicity, within each Period (defined by periodicity value), TSC QoS Flows are required to transmit only one burst of maximum size MDBV within the AN-PDB. Known QoS Flow traffic characteristics provided in the TSCAI may be used to optimize scheduling in the 5GS.

The following is applicable for the 5QI defined for TSC QoS Flows:

1.	The TSC Burst Size may be used to set the MDBV as follows:

The maximum TSC Burst Size is considered as the largest amount of data within a time period that is equal to the value of 5G-AN PDB of the 5QI that was set for this traffic class. The maximum value of TSC Burst Size should be mapped to a 5QI with MDBV that is equal or higher. This 5QI also shall have a PDB value that satisfies the bridge delay capabilities reported for the corresponding traffic class. For TSC QoS Flows, the Maximum Burst Size of the aggregated TSC streams to be allocated to this QoS Flow can be similarly mapped to a 5QI with MDBV value that is equal or higher, and the PDB of this 5QI shall also satisfy the bridge delay capabilities reported.

2.	The PDB is explicitly divided into 5G-AN PDB and CN PDB. The 5G-AN PDB is the packet delay budget applicable to the radio interface, including AN processing. The CN PDB is the delay between a UPF terminating N6 and a 5G-AN. Separate delay budgets are necessary for calculation of expected packet transmit times on 5G System interfaces.

### 5.27.4	Hold and Forward Buffering mechanism

DS-TT and NW-TT support a hold and forward mechanism to schedule traffic as defined in IEEE 802.1Q [98] if 5GS is to participate transparently as a bridge in a TSN network. The Hold and Forward buffering mechanism allows PDB based 5GS QoS to be used for TSC traffic since packets need only arrive at NW-TT or DS-TT egress prior to their scheduled transmission time.

5GS provides AdminControlList and AdminBaseTime as defined in IEEE 802.1Q [98] on a per Ethernet port basis to DS-TT and NW-TT for the hold and forward buffer as described in clause 5.28.3.

NOTE:	How Hold and Forward buffer is supported by the TSN Translator is up to implementation.

### 5.27.5	5G System Bridge delay

In order for 5G System to participate as a TSN bridge according to gate schedules specified, the 5GS Bridge is required to provide 5GS Bridge Delays for each port pair and traffic class to an IEEE TSN system. In order to determine 5GS Bridge Delays, following components are needed:

1.	UE-DS-TT Residence Time: the time taken within the UE and DS-TT to forward a packet between the UE and DS-TT port.

2.	The PDB as defined in clause 5.7.3.4: packet delay between the UE and the UPF that terminates the N6 interface. The PDB is determined from the 5QI (as in standardized 5QI to QoS characteristics mapping as in clause 5.7.4) assigned for the TSC QoS Flow. It is assumed that the PDB includes the UPF Residence Time and NW-TT Residence time.

Residence times are the same for uplink and downlink traffic. The TSN AF calculates the 5GS independentDelayMax/independentDelayMin value of bridge for each port pair and for each traffic class using the above components.

NOTE 1:	With supporting the hold and forward buffering mechanism in UE-DS-TT and NW-TT, the values of independentDelayMax and independentDelayMin for 5GS Bridge can be the same.

The dependentDelayMax and dependentDelayMin for 5GS Bridge are derived from frame length-dependent delays in DS-TT and in NW-TT for inputting the packet via the ingress port and outputting the packet via the egress port.

NOTE 2:	Whether the values of dependentDelayMax and dependentDelayMin are the same is dependent on implementation.

Since Residence times may vary among UEs and among UPFs, the 5GS Bridge Delay is determined after the PDU Session Establishment for the corresponding UPF and the UE. UE-DS-TT Residence Time is provided at the time of PDU Session Establishment by the UE to the network.

## 5.28	Support of integration with TSN

### 5.28.1	5GS logical TSN bridge management

5GS functions acts as one or more TSN Bridges of the TSN network. The 5GS Bridge is composed of the ports on a single UPF (i.e. PSA) side, the user plane tunnel between the UE and UPF, and the ports on the DS-TT side. For each 5GS Bridge of a TSN network, the ports on NW-TT support the connectivity to the TSN network, the ports on DS-TT side are associated to the PDU Session providing connectivity to the TSN network.

The granularity of the logical TSN bridge is per UPF. The bridge ID of the logical TSN bridge is bound to the UPF ID of the UPF as identified in TS 23.502 [3]. The TSN AF stores the binding relationship between a port on UE/DS-TT side, a port on UPF/NW-TT side, and a PDU Session during reporting of logical TSN bridge information.

There is only one PDU Session per DS-TT port for a given UPF. All PDU Sessions which connect to the same TSN network via a specific UPF are grouped into a single virtual bridge. The capabilities of each port on UE/DS-TT side and UPF/NW-TT side are integrated as part of the configuration of the 5GS Bridge and are notified to TSN AF and delivered to CNC for TSN bridge registration and modification.

NOTE 1:	It is assumed that all PDU sessions which connect to the same TSN network via a specific UPF are handled by the same TSN AF.

Figure 5.28.1-1: Per UPF based virtual bridge

NOTE 2:	If a UE establishes multiple PDU Sessions terminating in different UPFs, then the UE is represented by multiple logical TSN bridges.

In order to support TSN traffic scheduling over 5GS Bridge, the 5GS supports the following functions:

-	Report the bridge information of 5GS Bridge to TSN network.

-	Map the configuration information obtained from TSN network into 5GS QoS information (e.g. 5QI, TSC Assistance Information) of a QoS Flow in corresponding PDU Session for efficient time-aware scheduling, as defined at clause 5.28.2.

The bridge information of 5GS Bridge is used by the TSN network to make appropriate management configuration for the 5GS Bridge. The bridge information of 5GS Bridge includes at least the following:

-	Information for 5GS Bridge:

-	Bridge Address (unique MAC address that identifies the bridge used to derive the bridge ID);

-	Bridge Name;

-	Number of Ports;

-	list of port numbers.

-	Capabilities of 5GS Bridge as defined in 802.1Qcc [95]:

-	5GS Bridge delay per port pair per traffic class, including 5GS Bridge delay (dependent and independent of frame size, and their maximum and minimum values: independentDelayMax, independentDelayMin, dependentDelayMax, dependentDelayMin), ingress port number, egress port number and traffic class.

-	Propagation delay per port (txPropagationDelay), including transmission propagation delay, egress port number.

-	Topology of 5GS Bridge as defined in IEEE 802.1AB [97]:

-	Chassis ID subtype and Chassis ID of the 5GS Bridge.

-	Traffic classes and their priorities per port as defined in IEEE 802.1Q [98].

The following parameters: independentDelayMax and independentDelayMin, how to calculate them is left to implementation and not defined in this specification.

Bridge ID of the 5GS Bridge, port numbers of the Ethernet in NW-TT could be preconfigured on the UPF. The UPF is selected for a PDU Session serving TSC based on subscribed DNN, traffic classes and VLANs. Port number of Ethernet port on the DS-TT for the PDU Session is assigned by the UPF during PDU session establishment and stored at the SMF. SMF provides the port numbers and MAC addresses of the Ethernet ports in DS-TT and NW-TT of the related PDU session to the TSN AF via PCF. If a PDU session for which SMF has reported port numbers to TSN AF is released, then SMF informs TSN AF accordingly.

The TSN AF is responsible to receive the bridge information of 5GS Bridge from 5GS, as well as register or update this information to the TSN network.

### 5.28.2	5GS Bridge configuration

In order to schedule TSN traffic over 5GS Bridge, the configuration information of 5GS Bridge is mapped to 5GS QoS within the corresponding PDU Session. The QoS parameters mapping for TSN is described in TS 23.503 [45] clause 6.1.3.23.

The configuration information of 5GS Bridge as defined in in IEEE 802.1Q [98], includes the following:

-	Bridge ID of 5GS Bridge.

-	Configuration information of scheduled traffic on ports of DS-TT and NW-TT:

-	Egress ports of 5GS Bridge, e.g., ports on DS-TT and NW-TT;

-	Traffic classes and their priorities.

NOTE:	In this Release of the specification, only support simplified IEEE 802.1Q [98], Annex Q.2 for 5GS.

The configuration information of 5GS Bridge as defined in IEEE 802.1Q [98], includes the following:

-	Chassis ID of 5GS Bridge;

-	Traffic forwarding information as defined in IEEE 802.1Q [98] clause 8.8.1:

-	Destination MAC address and VLAN ID of TSN stream;

-	Port number in the Port MAP as defined in IEEE 802.1Q [98] clause 8.8.1.

-	Configuration information per stream according to IEEE 802.1Q [98] clause 8.6.5.1:

-	Ingress port number of 5GS Bridge, i.e., ports on DS-TT/NW-TT;

-	Stream priority.

The SMF report the MAC address of the DS-TT port of the related PDU Session to TSN AF via PCF as the MAC address of the PDU Session. The association between the MAC address used by the PDU Session, 5GS Bridge ID and port number on DS-TT is maintained at TSN AF and further used to assist to bind the TSN traffic with the UE's PDU session.

In the case of provisioning traffic routing information based the information above, the TSN AF determines the MAC address used by the PDU Session for the TSN traffic based on ingress port number or egress port number, and requests to PCF influence traffic routing for TSN traffic as defined in clause 5.6.7 with the MAC address of the PDU Session, traffic class, destination MAC address and VLAN ID.

### 5.28.3	Port management information exchange in 5GS

#### 5.28.3.1	General

Bridge management information is exchanged between CNC and TSN AF. A subset of bridge management information, referred to as port management information, is related to Ethernet ports located in DS-TT or NW-TT.

5GS shall support transfer of standardized and deployment-specific port management information transparently between TSN AF and DS-TT or NW-TT, respectively inside a Port Management Information Container. Table 5.28.3.1-1 lists standardized port management information.

Table 5.28.3.1-1: Standardized port management information

| Port management information | Applicability (see Note 6) |  | Supported operations by TSN AF (see Note 1) | Reference |
| --- | --- | --- | --- | --- |
|  | DS-TT | NW-TT |  |  |
| General |  |  |  |  |
| Port management capabilities (see Note 2) | X | X | R |  |
| Bridge delay related information |  |  |  |  |
| txPropagationDelay | X | X | R | IEEE 802.1Qcc [95] clause 12.32.2.1 |
| Traffic class related information |  |  |  |  |
| Traffic class table | X | X | RW | IEEE 802.1Q [98] clause 12.6.3 and clause 8.6.6. |
| Gate control information |  |  |  |  |
| GateEnabled | X | X | RW | IEEE 802.1Q [98] Table 12-29 |
| AdminBaseTime | X | X | RW | IEEE 802.1Q [98] Table 12-29 |
| AdminControlList | X | X | RW | IEEE 802.1Q [98] Table 12-29 |
| AdminCycleTime (see Note 3) | X | X | RW | IEEE 802.1Q [98] Table 12-29 |
| AdminControlListLength (see Note 3) | X | X | RW | IEEE 802.1Q [98] Table 12-28 |
| Tick granularity | X | X | R | IEEE 802.1Q [98] Table 12-29 |
| General Neighbor discovery configuration (NOTE 4) |  |  |  |  |
| adminStatus | (x) | X | RW | IEEE 802.1AB [97] clause 9.2.5.1 |
| lldpV2LocChassisIdSubtype | (x) | X | RW | IEEE 802.1AB [97] Table 11-2 |
| lldpV2LocChassisId | (x) | X | RW | IEEE 802.1AB [97] Table 11-2 |
| lldpV2MessageTxInterval | (x) | X | RW | IEEE 802.1AB [97] Table 11-2 |
| lldpV2MessageTxHoldMultiplier | (x) | X | RW | IEEE 802.1AB [97] Table 11-2 |
| NW-TT port neighbor discovery configuration |  |  |  |  |
| lldpV2LocPortIdSubtype |  | X | RW | IEEE 802.1AB [97] Table 11-2 |
| lldpV2LocPortId |  | X | RW | IEEE 802.1AB [97] Table 11-2 |
| DS-TT port neighbor discovery configuration |  |  |  |  |
| lldpV2LocPortIdSubtype | (x) | (X) | RW | IEEE 802.1AB [97] Table 11-2 |
| lldpV2LocPortId | (x) | (X) | RW | IEEE 802.1AB [97] Table 11-2 |
| Neighbor discovery information for each discovered neighbor of NW-TT |  |  |  |  |
| lldpV2RemChassisIdSubtype | (x) | X | R | IEEE 802.1AB [97] Table 11-2 |
| lldpV2RemChassisId | (x) | X | R | IEEE 802.1AB [97] Table 11-2 |
| lldpV2RemPortIdSubtype | (x) | X | R | IEEE 802.1AB [97] Table 11-2 |
| lldpV2RemPortId | (x) | X | R | IEEE 802.1AB [97] Table 11-2 |
| TTL | (x) | X | R | IEEE 802.1AB[97] clause 8.5.4 |
| Neighbor discovery information for each discovered neighbor of DS-TT (NOTE 5) |  |  |  |  |
| lldpV2RemChassisIdSubtype | (x) | (X) | R | IEEE 802.1AB [97] Table 11-2 |
| lldpV2RemChassisId | (x) | (X) | R | IEEE 802.1AB [97] Table 11-2 |
| lldpV2RemPortIdSubtype | (x) | (X) | R | IEEE 802.1AB [97] Table 11-2 |
| lldpV2RemPortId | (x) | (X) | R | IEEE 802.1AB [97] Table 11-2 |
| TTL |  | X | R | IEEE 802.1AB [97] clause 8.5.4.1 |
| Per-Stream Filtering and Policing information |  |  |  |  |
| StreamHandleSpec | X | X | RW | IEEE 802.1Q [98] Table 12-32 |
| PrioritySpec | X | X | RW | IEEE 802.1Q [98] Table 12-32 |
| StreamGateInstanceID | X | X | RW | IEEE 802.1Q [98] Table 12-32 |
| PSFPAdminBaseTime | X | X | RW | IEEE 802.1Q [98] Table 12-33 |
| PSFPAdminControlList | X | X | RW | IEEE 802.1Q [98] Table 12-33 |
| PSFPAdminCycleTime | X | X | RW | IEEE 802.1Q [98] Table 12-33 |
| PSFPTickGranularity | X | X | R | IEEE 802.1Q [98] Table 12-33 |
| NOTE 1: R = Read only access; RW = Read/Write access. NOTE 2: Indicates which standardized and deployment-specific port management information is supported by DS-TT or NW-TT. NOTE 3: AdminCycleTime and AdminControlListLength are optional for gate control information. NOTE 4: If DS-TT supports neighbor discovery, then TSN AF sends the general neighbor discovery configuration for DS-TT Ethernet ports to DS-TT. If DS-TT does not support neighbor discovery, then TSN AF sends the general neighbor discovery configuration for DS-TT Ethernet ports to NW-TT and NW-TT performs neighbor discovery on behalf on DS-TT. NOTE 5: If DS-TT supports neighbor discovery, then TSN AF retrieves neighbor discovery information for DS-TT Ethernet ports from DS-TT. If DS-TT does not support neighbor discovery, then TSN AF retrieves neighbor discovery information for DS-TT Ethernet ports from NW-TT, which performs neighbor discovery on behalf on DS-TT. NOTE 6: X = applicable; (x) = applicable when validation and generation of LLDP frames is processed at the DS-TT; (X) = applicable when validation and generation of LLDP frames is processed centrally at NW-TT. |  |  |  |  |

Exchange of port management information between TSN AF and NW-TT or DS-TT allows TSN AF to:

1)	retrieve port management information for a DS-TT or NW-TT Ethernet port;

2)	send port management information for a DS-TT or NW-TT Ethernet port;

3)	subscribe to and receive notifications if specific port management information for a DS-TT or NW-TT Ethernet port changes.

Exchange of port management information between TSN AF and NW-TT or DS-TT is initiated by DS-TT or NW-TT to:

-	notify TSN AF if port management information has changed that TSN AF has subscribed for.

Exchange of port management information is initiated by DS-TT to:

-	provide port management capabilities, i.e. provide information indicating which standardized and deployment-specific port management information is supported by DS-TT.

TSN AF indicates inside the Port Management Information Container whether it wants to retrieve or send port management information or intends to (un-)subscribe for notifications.

#### 5.28.3.2	Transfer of port management information

Port management information is transferred transparently via 5GS between TSN AF and DS-TT or NW-TT, respectively, inside a Port Management Information Container as follows:

-	To convey port management information from DS-TT or NW-TT to TSN AF:

-	DS-TT provides a Port Management Information Container to the UE, which includes the Port Management Information container as an optional Information Element of an N1 SM container and triggers the UE requested PDU Session Modification procedure to forward the Port Management Information container to the SMF. SMF forwards the Port Management Information container and the port number of the related DS-TT Ethernet port to TSN AF as described in TS 23.502 [3] clause 4.3.3.2;

-	NW-TT provides a Port Management Information Container to the UPF, which triggers the N4 Session Level Reporting Procedure to forward the Port Management Information Container to SMF. SMF in turn forwards the container and the port number of the related NW-TT Ethernet port to TSN AF as described in TS 23.502 [3] clause 4.16.5.1. If NW-TT provides a Port Management Information Container for an Ethernet port shared by multiple PDU sessions, then UPF forwards the Port Management Information Container only for one of those PDU sessions.

-	To convey port management information from TSN AF to DS-TT or NW-TT:

-	TSN AF provides a Port Management Information Container, MAC address reported for a PDU Session and the port number of the Ethernet port to manage to the PCF, which forwards the information to SMF based on the MAC address using the PCF initiated SM Policy Association Modification procedure as described in TS 23.502 [3] clause 4.16.5.2. SMF determines whether the port number relates to a DS-TT or NW-TT Ethernet port and based on this forwards the Port Management Information Container to DS-TT or NW-TT using the network requested PDU Session Modification procedure as described in TS 23.502 [3] clause 4.3.3.2. If the port number identifies an NW-TT Ethernet port shared by multiple PDU sessions, PCF and SMF forward the Port Management Information Container only for one of those PDU sessions.

### 5.28.4	QoS mapping tables

The mapping tables between the traffic class and 5GS QoS Profile is provisioned and further used to find suitable 5GS QoS profile to transfer TSN traffic over the PDU Session. QoS mapping procedures are performed in two phases: (1) QoS capability report phase as described in clause 5.28.1, and (2) QoS configuration phase as in clause 5.28.2

(1)	The TSN AF shall be pre-configured (e.g. via OAM) with a mapping table. The mapping table contains TSN traffic classes, pre-configured bridge delays, and priority levels. Once the PDU session has been setup, and after retrieving the information related to DS-TT residence times, the TSN AF updates bridge delays per port pair and traffic class and reports the bridge delays and other relevant TSN information such as the Traffic Class Table for every port, according to the IEEE 802.1Q [X] and IEEE 802.1Qcc [95] to the CNC.

(2)	CNC distributes the TSN QoS requirements and TSN scheduling parameters to 5GS Bridge via TSN AF.

The PCF mapping table provides a mapping from TSN QoS information (see TS 23.503 [45], clauses 6.2.1.2 and 6.1.3.23) to 5GS QoS profile. Based on trigger from TSN AF, the PCF may trigger PDU session modification procedure to establish a new 5G QoS Flow or use the pre-configured 5QI for 5G QoS Flow for the requested traffic class according to the selected QoS policies and the TSN AF traffic requirements.

Figure 5.28.4-1 illustrates the functional distribution of the mapping tables.

Figure 5.28.4-1: QoS Mapping Function distribution between PCF and TSN AF

The minimum set of TSN QoS-related parameters that are relevant for mapping the TSN QoS requirements are used by the TSN AF: traffic classes and their priorities per port, bridge delays per port pair and traffic class (independentDelayMax, independentDelayMin, dependentDelayMax, dependentDelayMin), and propagation delay per port (txPropagationDelay).

NOTE:	At 5QI selection in the QoS mapping table the PCF takes into consideration the maximum burst size of TSN streams in the port traffic class.

Once the CNC has received the necessary information, it proceeds to calculate scheduling and paths. The configuration information is then set in the bridge per port and per traffic class. The most relevant information received is the scheduling for every traffic class and port of the bridge. At this point, it is possible to retrieve the TSN QoS requirements by identifying the traffic class of the port. The traffic class to TSN QoS and delay requirement mapping can be performed using the QoS mapping table in the TSN AF as specified in TS 23.503 [45]. Subsequently in the PCF, the 5G QoS Flow can be configured by selecting a 5QI as specified in TS 23.503 [45]. This feedback approach uses the reported information to the CNC and the feedback of the configuration information coming from the CNC to perform the mapping and configuration in the 5GS. The scheduling configuration information per traffic class is mapped to trigger creation/modification of a QoS Flow in 5GS.

QoS mapping table in the PCF between TSN parameters and 5GS parameters should match the delay and priority, while preserving the priorities in the 5GS. An operator enabling TSN services via 5GS can choose up to eight traffic classes to be mapped to 5GS QoS profiles.

Once the 5QIs to be used for TSN streams are identified by the PCF as specified in TS 23.503 [45], then it is possible to enumerate as many bridge port traffic classes as the number of selected 5QIs.