3GPP TSG-SA WG2 Meeting #137E	S2-2002191

# Elbonia, 24 – 27 Feburary 2020															rev of S2-1911085

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 1520 | rev | 3 | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PLMN+CAG information - minimum, maximum storage and survival of power cycle |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO, MediaTek Inc., Huawei, Nokia, Nokia Shanghai Bell, Ericsson, Deutsche Telekom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2020-02-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Note: In 24.501 subclause 4.14.3, an Editor's Note is in place, see C1-194790. The storing of PLMN + CAG information (Allowed CAG list + (optionally) CAG-only indication) was introduced in SA2#133 (in S2-1906703) where it was specifically required that the combination of PLMN + CAG information was stored even when UE is de-registered from the PLMN that provided that information. However, what is missing are:- - storage of these combinations of PLMN + CAG information shall survive power off; - the minimum (and maximum) number of such entries that the UE has to store |  |  |  |  |  |  |  |  |
| Summary of change: |  | - the combinations of PLMN + CAG information is to be kept even at power off. - A reference to TS 24.501 is added as it is assumed that any further details are left to CT1 to specify. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE will not be storing the combinations of PLMN + CAG information when powered off and not in a predictable way. Additionally, if the PLMN+CAG information is not kept when power off, then at re-power ON, there will be no CAG information to help UE select onto PNI-NPN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.30.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * * *

#### 5.30.3.3	UE configuration, subscription aspects and storage

To support CAG, the UE may be pre-configured or  (re)configured with the following CAG information, included in the subscription as part of the Mobility Restrictions:

-	an Allowed CAG list i.e. a list of CAG Identifiers the UE is allowed to access; and

-	optionally, a CAG-only indication whether the UE is only allowed to access 5GS via CAG cells (see TS 38.304 [50] for how the UE identifies whether a cell is a CAG cell);

The HPLMN may configure or re-configure a UE with the above CAG information using the UE Configuration Update procedure for access and mobility management related parameters described in TS 23.502 [3] in clause 4.2.4.2.,

The above CAG information is provided by the HPLMN on a per PLMN basis. In a PLMN the UE shall only consider the CAG information provided for this PLMN.

The UE shall store the latest available CAG information for every PLMN for which it is provided and keep it stored when the UE is de-registered or switched off, as described in TS 24.501 [47].

NOTE:	CAG information has no implication on whether and how the UE accesses 5GS over non-3GPP access.

* * * End of Change * * * *