3GPP TSG-WG SA2 Meeting #137E e-meeting 	S2-2002224

# Elbonia, February 24 – 27, 2020	(revision of S2-200xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2202 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TSN working domain and aggregation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2020-02-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - Definition of TSN working domain is missed and the terminologies used in the text need to be aligned. Currently, different terms are used in 23.501 for the “TSN domain”, e.g. TSN working domain, TSN working clock domain, TSN domain… The terms needs to be unified and defined. In TS 22.104, it uses “working clock domain” as following without definition: “Working clock domains are constrained in size. They often consist of a single machine or a set of neighbouring machines that physically collaborate. …In these cases, the application synchronizes locally within the working clock domains, allowing precise synchronization with more efficient components. ” The definition of “working clock domain” can be found in TR22.832: working clock domain: synchronization domain for a localized set of devices collaborating on a specific task or work function. Proposal to reuse the definition in TR 22.832 but rename the term to TSN working domain to minimize the changes to TS 23.501: TSN working domain: synchronization domain for a localized set of devices collaborating on a specific task or work function in a TSN network. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Propose a definition of TSN working domain and clarify . |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The definition of TSN working domain is not clear. The aggregation of TSN streams may be incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 5.27.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

3	Definitions and abbreviations

3.1	Definitions

…

Time Sensitive Communication (TSC): A communication service that supports deterministic communication and/or isochronous communication with high reliability and availability. It is about providing packet transport with QoS characteristics such as bounds on latency, loss, and reliability, where end systems and relay/transmit nodes can be strictly synchronized.

TSN working domain: Synchronization domain for a localized set of devices collaborating on a specific task or work function in a TSN network, corresponding to a gPTP domain defined in IEEE 802.1AS[104].

UDM Group ID: This refers to one or more UDM instances managing a specific set of SUPIs. An UDM Group consists of one or multiple UDM Sets.

…

# * * * * Second change * * * *

5.27.1	TSN Time Synchronization

5.27.1.1	General

For supporting TSN time synchronization, the 5GS is integrated with the external network as a TSN bridge as described in clauses 4.4.8 and 5.28.1. It shall be modelled as an IEEE 802.1AS [104] compliant entity according to TS 22.104 [105]. For TSN Synchronization, the entire E2E 5G system can be considered as an IEEE 802.1AS [104] "time-aware system". Only the TSN Translators (TTs) at the edges of the 5G system need to support the IEEE 802.1AS [104] operations. UE, gNB, UPF, NW-TT and DS- TTs are synchronized with the 5G GM (i.e. the 5G internal system clock) which shall serve to keep these network elements synchronized. The TTs located at the edge of 5G system fulfil all functions related to IEEE 802.1AS [104], e.g. (g)PTP support, timestamping, Best Master Clock Algorithm (BMCA), rateRatio. Figure 5.27.1-1 illustrates the 5G and TSN clock distribution model via 5GS.

Figure 5.27.1-1: 5G system is modelled as IEEE 802.1AS compliant time aware system for supporting TSN time synchronization

Figure 5.27.1-1 depicts the two synchronizations systems considered: the 5GS synchronization and the TSN domain synchronization, as well as the Master (M) and Slave (S) ports considered when the TSN GM is located at TSN working domain.

-	5GS synchronization: Used for NG RAN synchronization. 5G RAN synchronization is specified in TS 38.331 [28].

-	TSN domain synchronization: Provides synchronization service to TSN network. This process follows IEEE 802.1AS [104].

The two synchronization processes can be considered independent from each other and the gNB only needs to be synchronized to the 5G GM clock.

To enable TSN synchronization, the 5GS calculates and adds the measured residence time between the TTs into the Correction Field (CF) of the synchronization packets of the TSN working domain.

5.27.1.2	Distribution of timing information

5.27.1.2.1	Distribution of 5G internal system clock

The 5G internal system clock shall be made available to all user plane nodes in the 5G system. The UPF and NW-TT may get the 5G internal system clock via the underlying PTP compatible transport network with mechanisms outside the scope of 3GPP. The 5G internal system clock shall be made available to UE with signaling of time information related to absolute timing of radio frames as described in TS 38.331 [28]. The 5G internal system clock shall be made available to DS-TT by the UE.

5.27.1.2.2	Distribution of TSN clock and time-stamping

The mechanisms for distribution of TSN clock and time-stamping described in this clause are according to IEEE 802.1AS [104].

Upon reception of a downlink gPTP message the NW-TT makes an ingress timestamping (TSi) for each gPTP event (Sync) message and uses the cumulative rateRatio received inside the gPTP message payload (carried within Sync message for one-step operation or Follow_up message for two-step operation) to calculate the link delay from the upstream TSN node (gPTP entity) expressed in TSN GM time as specified in IEEE 802.1ASTM [104]. NW-TT then calculates the new cumulative rateRatio (i.e. the cumulative rateRatio of the 5GS) as specified in IEEE 802.1ASTM [104] and modifies the gPTP message payload (carried within Sync message for one-step operation or Follow_up message for two-step operation) as follows:

-	Adds the link delay from the upstream TSN node in TSN GM time to the correction field.

-	Replaces the cumulative rateRatio received from the upstream TSN node with the new cumulative rateRatio.

-	Adds TSi in the Suffix field of the gPTP packet as described in Annex H.

UPF then forwards the gPTP message to the UE via user plane (i.e. using the PDU session applicable for sending gPTP messages). Only one PDU session per UE per UPF is used for sending gPTP messages regardless of how many external TSN working domains have their clock information delivered through a given UPF serving that UE. All gPTP messages are transmitted on a QoS Flow that complies with the residence time upper bound requirement specified in IEEE 802.1AS [104].

NOTE:	The sum of the UE-DS-TT residence time and the PDB of the QoS Flow needs to be lower than the residence time upper bound requirement for a time-aware system specified in IEEE 802.1AS [104].

A UE receives the gPTP messages and forwards them to the DS-TT. The DS-TT then creates egress timestamping (TSe) for the gPTP event (Sync) messages for external TSN working domains. The difference between TSi and TSe is considered as the calculated residence time spent within the 5G system for this gPTP message expressed in 5GS time. The DS-TT then uses the rateRatio contained inside the gPTP message payload (carried within Sync message for one-step operation or Follow_up message for two-step operation) to convert the residence time spent within the 5GS in TSN GM time and modifies the payload of the gPTP message that it sends towards the downstream TSN node as follows:

-	Adds the calculated residence time expressed in TSN GM time to the correction field.

-	Removes TSi from the Suffix field.

5.27.1.3	Support for multiple TSN working domains

Each TSN working domain sends its own gPTP messages. The related Ethernet frames carry the gPTP multicast Ethernet destination MAC address and the gPTP message carries a specific PTP "domainNumber" that indicates the time domain they are referring to. The NW-TT makes ingress timestamping (TSi) for the gPTP event messages of all domains.

A UE receives gPTP messages and forwards them all to the DS-TT. The DS-TT receives the original TSN clock timing information and the corresponding TSi via gPTP messages for one or more TSN working domains. The DS-TT then makes egress timestamping (TSe) for the gPTP event messages for every external TSN working domain. Ingress and egress time stamping is based on the 5G system clock at NW-TT and DS-TT.

NOTE 1:	An end-station can select TSN timing information of interest based on the "domainNumber" in the gPTP message.

The process described in "Distribution of TSN clock and time-stamping" is thus repeated for each TSN working domain between a UE and the UPF it is connected to. When a UE is connected to multiple TSN working domains via the same UPF, then all gPTP messages from these domains are forwarded to the UE via the same PDU session.

NOTE 2:	If all TSN working domains can be made synchronous and the synchronization can be provided by the 5G clock, the TT output ports at the UPF side (NW-TT) towards the connected TSN networks propagate the 5G clock using the 802.1AS profile (i.e. the 5G system as an IEEE 802.1AS [104] compliant time-aware system).

# * * * * End of changes * * * *