3GPP TSG-SA2 Meeting #137E e-meeting	S2-2001950r04

# Elbonia, February 24 – 27, 2020

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 1582 | rev | 1 | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 23.502 Supporting for AF providing UE IP address(es) for 5G VN group PDU sessions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile, Nokia, Nokia Shanghai Bell, Deutsche Telekom? |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2020-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | External DN may need to assign specific UE IP addresses for 5G VN group PDU sessions. These UE IP addresses can be private IP addresses e.g. 192.168.1.x, which are the same IP address segment as enterprise network. The N6 PtP tunnel can be used to support the PDU packet transmission when the UE’s IP address is private IP address. In R15, the DHCP has been introduced for IP address allocation by external DN.However the UE may need to indicate to the network within the Protocol Configuration Options that the UE requests to obtain the IPv4 address with DHCPv4, which increases the complexity in UE side. And in real network, it is difficult for SMF communicating with external DN’s DHCP server. Therefore, one solution is provided in this paper that the AF may provide specific IP addresses for one 5G VN group members. And during the PDU session establishment, static IP address allocation method can be used. |  |  |  |  |  |  |  |  |
| Summary of change: |  | AF can provide speicifc UE IP ddresses for 5G VN group PDU sessions, corresponding update in 502. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | if not approved, 5G VN can not support requirements from enterprise network. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.6.3b |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | R04 This may indicate - the need for secondary authentication (as defined in TS 23.501 [2] clause 5.6) - the need for DN-AAA authorization (as defined in TS 23.501 [2] clause 5.6) - the need for SMF to request the UE IP address from the DN-AAA server If at least one of secondary authentication, DN-AAA authorization or DN-AAA UE IP address allocation is needed, the AF may provide DN-AAA server addressing information |  |  |  |  |  |  |  |  |

## <<< Start of changes >>>

#### 4.15.6.3b	5G VN group data

The 5G VN group data is described in Table 4.15.6.3b-1.

# Table 4.15.6.3b-1: Description of 5G VN group data

| Parameters | Description |
| --- | --- |
| DNN | DNN for the 5G VN group |
| S-NSSAI | S-NSSAI for the 5G VN group |
| PDU Session Type | PDU Session Types allowed for 5G VN group |
| Application descriptor | The may be multiple instances of this information; this information may be used to build URSP sent to 5G VN group members |
| Information related with secondary authentication / authorization | . This may indicate - the need for secondary authentication (as defined in TS 23.501 [2] clause 5.6) - the need for DN-AAA authorization (as defined in TS 23.501 [2] clause 5.6) - the need for SMF to request the UE IP address from the DN-AAA server If at least one of secondary authentication, DN-AAA authorization or DN-AAA UE IP address allocation is needed, the AF may provide DN-AAA server addressing information,. |

The information described in Table 4.15.6.3b-1 corresponds to 5G VN group data that an AF may provide together with External Group ID.

## <<< End of changes >>>