3GPP TSG-WG SA2 Meeting #138E	S2-2002757

# 20 - 24 April 2020, Electronic, Elbonia

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2239 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correcting I-NEF service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_CIoT |  |  |  |  |  | Date: |  |  | 2020-04-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This is a companion CR to 23.502CR2187(S2-2002756). CT3 LS S2-2002645/C3-201494 shows that I-NEF related stage 3 work could not be proceeded, therefore there is a need to revisit the information flow in §4.15.3.2.3a and the related NF services. Per 23.501, the functionalities of I-NEF are normalization and charging/accounting generation, and I-NEF identity is configured in NFs in VLPMN. =====23.501===== 6.2.5a Intermediate NEF The Intermediate NEF (I-NEF) is used for interworking between NFs in the VPLMN and the NEF. The I-NEF is optional and is only used in roaming scenarios. NFs in the VPLMN are configured with the I-NEF identity. The functionality provided by the I-NEF may include the following: - Normalization of reports according to roaming agreements between VPLMN and HPLMN…; - Generation of charging/accounting information for Monitoring In our view, (1) the NFs in the VPLMN (i.e. AMF/SMF) is able to route the event notification messages to the I-NEF based on the configured I-NEF identity in the AMF/SMF as sepcified in 6.2.5a of TS 23.501, which is similar to the way that an NF would route a message to the SCP. (2) When the AMF/SMF sends event notification message to the I-NEF, the AMF/SMF would also convey the notification endpoint info of NEF to the I-NEF. Exactly how AMF/SMF conveys the info is left for stage 3. (3) Upon receiving event notification from AMF/SMF, the I-NEF does normalization, generates charging information and then forwards the event notification message to the NEF according to the notification endpoint of NEF conveyed by AMF/SMF. Based on the above, no I-NEF service operations are needed. (4) Ninef_EventExposure_Subscribe currently performed by AMF/SMF is not used to get I-NEF events but to inform the I-NEF of the NEF’s notification endpoint info, which seems to be a misuse of SBI Subscribe/Notify. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove I-NEF Services |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No proper input to stage 3 work meaning that specification for event monitoring in roaming scenario cannot be completed for Rel-16. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.8A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS23.502 CR2187 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

| This CR's revision history: |  |
| --- | --- |

****************** START CHANGE ***************

### 7.2.8A	Void

****************** END CHANGE ***************