3GPP TSG SA WG2 Meeting #138E	S2-2003085

# Elbonia, April 20 – 23, 2020	(Revision of S2-20xxxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.316 | CR | 2043 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MBR values for wireline access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5WWC |  |  |  |  |  | Date: |  |  | 2020-04-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | BBF has clarified in an LS (S2-200xxxx) that they have no need for the RG-TMBR parameter and that they instead have defined RG-LWAC to carry applicable QoS information, such as maximum bitrates. TS 23.316 specifies that UE-AMBR is not provided to the W-AGF for wireline access and instead RG-TMBR is provided. However, it is not fully explicit in 23.316 that UE-AMBR is not applicable to wireline access, and this has led to misunderstanding. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify RG-TMBR and RG-LWAC usage for BBF access. Clarify that UE-AMBR is not applicable to wireline access. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wireline specific parameter defined that is not used by wireline networks, causing inconsistent 3GPP and BBF specifications. Unclear specification of UE-AMBR |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.5.1.1, 8.1.1, 8.1.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************** First changes ***************

#### 4.5.1.1	Total Maximum Bit Rates

For wireline access networks (W-5GAN) that do not provide dynamic resource reservation, the GBR QoS flows cannot be catered for independently of non-GBR flows. In such W-5GANs, the aggregate bitrate need to reflect both GBR and non-GBR flows instead. These reflect the characteristics of the wireline access provided for the 5G-RG or FN-RG.

Each RG may be associated with the following aggregate rate limit QoS parameter:

-	per RG Total Maximum Bit Rate (RG-TMBR).

The RG-TMBR limits the aggregate bit rate that can be expected to be provided across all GBR and Non-GBR QoS Flows of a RG. The RG-TMBR is a parameter that may be provided to the W-5GAN by the AMF based on the value of the Subscribed RG-TMBR retrieved from UDM. The RG-TMBR is measured over a TMBR averaging window which is an operator specific value.

In the case of wireline access, the AMF may provide W-AGF with Subscribed RG-TMBR in Initial UE context NGAP message (e.g. in registration and Service Request procedures) instead of Subscribed UE-AMBR.

As specified in BBF WT-470 [38], Wireline BBF Access Networks do not apply the RG-TMBR. Instead an RG may be associated with maximum bit rate parameters carried in the RG Level Wireline Access Characteristics, as described in clause 4.5.1.2 and BBF WT-470 [38].

UE-AMBR is not applicable to wireline access and is not provided by AMF to W-AGF.

The RG subscription data parameters for RG-TMBR is defined in clause 8.1.1.

*************** Next change ***************

### 8.1.1	Nudm_SubscriberDataManagement (SDM) Service

#### 8.1.1.1	General

In addition to the Subscription data types used in the Nudm_SubscriberDataManagement Service, as defined in Table 5.2.3.3.1-1 of TS 23.502 [3], the additional data types defined in Table 8.1.1.1-1 below are applicable for RGs connected to 5GC via W-5GAN.

Table 8.1.1.1-1: Wireline access specific UE Subscription data types

| Subscription data type | Field | Description |
| --- | --- | --- |
| Access and Mobility Subscription data (data needed for UE Registration and Mobility Management) | RG Level Wireline Access Characteristics | The RG level Wireline Access Characteristics parameter provides QoS information for the W-AGF, as defined in clause 4.5.1.2. This parameter is handled by the UDM as a transparent container. |
|  | RG-TMBR (NOTE) | The maximum aggregated uplink and downlink MBRs to be shared across all GBR and Non-GBR QoS Flows according to the subscription of the user. Applicable only for the wireline access network for a 5G-RG or FN-RG, as defined in clause 4.5.1.1 |
| NOTE: The RG-TMBR is not applicable to Wireline BBF Access Network. |  |  |

In the case of Wireline access, the Forbidden area information within Table 5.2.3.3.1-1 of TS 23.502 [3] may correspond to a (set of) allowed Global Line ID.

*************** End of changes ***************