3GPP SA WG2 Meeting #138E	S2-2003106

April 20-24, 2020                                                                                             was S2-2001806

# Source:	Juniper

# Title:	IEEE Compatible Ethernet Services

# Document for:	Approval

Agenda Item:	9.1

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title: IEEE 802 Compatible Ethernet Services

## Acronym: 5GCES

## Unique identifier: 	{A number to be provided by MCC at the plenary}

Potential target Release: Rel-17.

Note that this field above indicates the proposed Release at the time of submission of the WID to TSG approval. It can later be changed without a need to revise the WID. The updated target Release is indicated in the Work Plan.

## 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  | X |
| Don't know |  |  |  |  |  |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a …

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

### 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| Vertical_LAN | SA2 | 820017 | 5GS Enhanced support of Vertical and LAN Services |

### 2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
|  | N/A |  |

Dependency on non-3GPP (draft) specification: None

## 3	Justification

Current 5G Ethernet solutions (with or without 5G-LAN) specified in TS 23.501 have many documented limitations. In particular, the relation to IEEE 802.1 based Ethernet protocols is not described. It is also not described how loop-free forwarding can be achieved when the 5GS is deployed to participate as bridge(s) with within Ethernet bridged networks with general topologies.

This proposal meets TEI criteria in that it is a straightforward enhancement with significant benefits, and it is expected to take 0.5 TU.

## 4	Objective

The objective is to specify Ethernet bridging (with or without the 5G-LAN) in a more complete way.

The work item has the following objectives:

- Specify how the UPF acts as an Ethernet bridge, based on IEEE specifications. This includes the following cases:

- PDU Sessions for the DNN are anchored on the same PSA UPF, with N6 interfaces(s) to the DN

- PDU Sessions for the DNN are anchored on different PSA UPFs, each with N6 interface(s) to the DN

- PDU Sessions for the DNN are anchored on different PSA UPFs, each with N6 interface(s) to the DN and with inter-UPF N19 tunnels.

For all cases, the PDU sessions, N6 interface(s) and (if applicable) N19 tunnel(s) are treated as logical bridge interfaces. The bridge (in the UPF) is compatible to IEEE 802.1Q, including the traffic forwarding and loop prevention mechanisms specified therein.

Note: In the third case above, if there are more than two UPFs,  then a loop prevention method must be used to prevent traffic looping among UPFs.

In essence, traffic forwarding is autonomous based on the built-in bridging functionalities of UPF without the need of SMF programmed PDRs/FARs for the purpose of traffic forwarding. This is implied in 23.501 clause 5.8.2.5.3 though it is the objective of this work item to make it explicit.

NOTE: 	Even when these bridging-based options are used for UPF autonomous forwarding decisions, PDRs/FARs are still used . For example, traffic of different flows to the same destination could be forwarded towards the same member (based on MAC address lookup) but with different QoS treatment (based on PDR/FAR/QER where the PDR contains additional information identifying the flows and the FAR contains additional forwarding treatment). Other examples of still required N4 rules are: traffic buffering for a PDU Session with inactive UP and Usage reporting via URR

## 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| N/A |  |  |  |  |  |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 23.501 | Updated several clauses, in particular clauses 5.6.10.2, 5.8.2.5.3. A new clause 5.8.2.13.4 that specifies the optional alternative, plus some minor relevant changes | TSG#88 | This will be handled as “TEI17”. |

## 6	Work item Rapporteur(s)

Zhaohui (Jeffrey) Zhang, Juniper Networks, zzhang@juniper.net

## 7	Work item leadership

SA2

## 8	Aspects that involve other WGs

N/A

## 9	Supporting Individual Members

Supporting IM name

Juniper

Orange

China Mobile

KPN

Spreadtrum