SA WG2 Meeting #S2-139E	S2-2003543

01 - 12 June, 2020, Electronic, Elbonia

3GPP TSG-CT WG3 Meeting #109e	C3-202420

# E-Meeting, 16th – 24th April 2020

Title:	LS on Network Area Information in BDT Policy

Response to:

Release:	Rel-16

Work Item:	xBDT / eNA

Source:	CT3

To:	SA2

Cc:

Contact Person:

Name:	Yali Yan

Tel. Number:	+ 86 15800958428

E-mail Address:	yanyali@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None.

# 1	Overall description

CT3 analysed the recent changes in SA2 TSs to finalize work on eNA and xBDT work items and would like to clarify usage of a Network Area Information related to background data transfer policy negotiations.

As defined in clause 6.1.2.4 of TS 23.503, the AF request shall contain an ASP identifier, the volume of data to be transferred per UE, the expected amount of UEs, the desired time window, the External Group Identifier and optionally, Network Area Information, request for notification.

Clause 6.1.2.4 of TS 23.503 also specifies: "The selected background transfer policy is finally stored by the PCF in the UDR as part of the Policy Data Set. The background transfer policy contains the Background Data Transfer Reference ID, the volume of data to be transferred per UE, the expected amount of UEs, the one or more instances of the tuple (ASP id, DNN, S-NSSAI) and if the AF subscribed to notifications on changes of the negotiated BDT policy".

Above requirements are aligned with steps 2b and 12 from clause 4.16.7.2 of TS 23.502. E.g. step 12 specifies: "The H-PCF stores the Background Data Transfer Reference ID together with the new transfer policy, the corresponding network area information, optionally the information of request for notification in the UDR by invoking Nudr_DM_Update".

However, clause 6.1.2.4 of TS 23.503 also specifies: "a background transfer policy consists of a recommended time window for the background data transfer, a reference to a charging rate for this time window, network area information and optionally a maximum aggregated bitrate (indicating that the charging according to the referenced charging rate is only applicable for the aggregated traffic of all involved UEs that stays below this value)".

This requirement is aligned with clause 4.16.7.1 of TS 23.502 which specifies: "The transfer policies consist of a desired time window for the background data transfer, a reference to a charging rate for the time window, network area information, and optionally a maximum aggregated bitrate, as described in clause 6.1.2.4 of TS 23.503 [20]."

Moreover, also as defined in clause 6.1.2.4 of TS 23.503, the PCF shall determine, based on the information provided by the AF, the analytics on "Network Performance" if available and other available information (e.g. network policy and existing background transfer policies) one or more background transfer policies. The "Network Performance" analytics provided by NWDAF as defined in clause 6.6.3 of TS 23.288 also include an area subset within the requested area:

Table 6.6.3-2: Network performance predictions

| Information | Description |
| --- | --- |
| List of network performance information (1..max) | Predicted analytics during the Analytics target period |
| > Area subset | TA or Cell ID within the requested area of interest as defined in clause 6.6.1 |
| > gNB status information | Ratio of gNBs that will be up and running during the entire Analytics target period in the area subset |
| > gNB resource usage | Usage of assigned resources (CPU, memory, disk) (average, peak) |
| > Number of UEs | Number of UEs predicted in the area subset |
| > Communication performance | Ratio of successful setup of PDU Sessions |
| > Mobility performance | Ratio of successful handover |
| > Confidence | Confidence of this prediction |

CT3 asks SA2 to clarify the following questions:

Q1:	Is the network area information part of background data transfer policy or not?

Q2:	If the answer of Q1 is yes, is the network area information in the AF request the same as the network area information in each background data transfer policy, as defined in clause 6.1.2.4 of TS 23.503?

During the BDT warning notification, do the list of all candidate BDT policies which are sent to the AF apply to the same network area information in the AF request, as defined in clause 6.1.2.4 of TS 23.503?

# 2	Actions

To SA WG2 group:

ACTION: 	CT3 kindly asks SA2 to answer above questions and update their specifications if necessary.

# 3	Dates of next TSG CT WG 3 meetings

3GPP TSG CT3#110e	02nd – 11th June 2020	E-Meeting

3GPP TSG CT3#111	24th – 29th August 2020	US, US