SA WG2 Meeting #139E	S2-2003749

Elbonia, June 1- 12, 2020	(revision of S2-200xxxx)

Source:	Apple

Title:	KI #5, New Sol: UAV Authorisation revocation by UTM

Document for:	Approval

Agenda Item:	8.7

Work Item / Release:	FS_ID_UAS-SA2 / Rel-17

Abstract: This contribution proposes a solution for Key Issue #5 – UAV authorization revocation and (re)authorization failures in TR 23.754.

# 1. Introduction

We propose to include this solution for Key Issue #5 in TR 23.754.

START of CHANGE

## 6.0	Mapping Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 1 |  |  |  | X |  |  |  |
| 2 |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |
| X |  |  |  |  | X |  |  |

NEXT CHANGE (All text is new)

## 6.X	Solution #X: UAV Authorisation revocation by UTM

### 6.X.1	Introduction

This solution is for the following Key Issue #5: UAV authorization revocation and (re)authorization failures.

-	How are UAV handled in case of failed (re)authorization or revocation of authorization by the UTM, considering handling of UAV connectivity with UAV Controller and expected UAV behavior.

### 6.X.2	Functional Description

This solution introduces a method for authorisation revocation of UAV(s) initiated by UTM/TPAE by performing deregistration from the 3GPP network when the UAV(s) is deemed to be in safe place.

### 6.X.3	Procedures

#### 6.X.3.1      UAV Authorisation revocation by UTM

The procedure in Figure 6.X.3.1-1 shows a signalling flow of an UAV authorisation revocation by the 3GPP network initiated by UTM:

Figure 6.X.3.1-1: UAV Authorisation Revocation by UTM

- UAV is registered to 5GC, connected to a UAC via "Direct" or "Network-Assisted" C2 communication mode, and UAV has an active PDU session for C2-Communication.

- UTM decides to revoke the authorization of UAV(s).

NOTE 1:	UTM can decide to revoke UAV’s authorisation for various reasons. For example, repeated violations of UAV aerial in no-drone locations.

3.	UTM sends a C2-Command to the UAV(s) to revoke their authorisation. The C2-command is sent over the user plane using the PDU session that was active at Step 1. The C2-command can include the following:

a)	UAV to disconnect from UAC and C2-communication mode is changed to "UTM-navigated";

b)	Optionally, cause code why the C2-communication mode is changed;

c)	Optionally, GPS co-ordinates to navigate in case UAV is aerial. This step is needed to navigate the UAV to a safe location before deregistering from 3GPP network due to authorisation revocation; and

d)	Optionally, a flight plan to navigate to a safe location.

6.	UAV disconnects from UAC and C2-communication mode is changed to "UTM-navigated".

7.	Once the UTM decides that UAV is in a safe location, UTM sends a Deregistration request to AMF via NEF/AF. The Deregistration request contains the UAV identity (Remote ID) and a cause code.

8.	AMF sends the Deregistration request message to UAV(s) with a cause code (e.g. Illegal UE, Illegal ME or a new cause code specific to UAV authorisation revocation).

9.	UAV(s) sends Deregistration Accept message.

NOTE: Steps 7-9 are optional. The UAV could stay registered to the 3GPP network.

### 6.X.4	Impacts on existing entities and interfaces

-	AMF - UTM communication needs to be defined.

-	Optionally, a new 5GMM cause code can be defined for authentication revocation of UAV(s).

### 6.X.5 	Evaluation

Editor's note:	This clause provides an evaluation of the solution.

End of CHANGES