3GPP TSG-SA/WG2 Meeting #139E	S2-2003905

# Elbonia, 01 - 12 June, 2020

Source:	China Unicom

Title:	KI #18, New Sol: Pre-analytics based solution for Real-Time communication with NWDAF

Document for:	Agreement

Agenda Item:	8.1

Work Item / Release:	FS_eNA_Ph2 / Rel-17

Abstract of the contribution: This contribution proposes a new solution to resolve key issue#18: Enhancement for real-time communication with NWDAF.

1	Introduction

This solution addresses key issue #18: Enhancement for real-time communication with NWDAF.

Based on the procedures defined in TS 23.288 [x], an NWDAF analytic process may include the following steps:

- NWDAF service consumer subscribe at NWDAF to be notified on analytics information by using Nnwdaf_AnalyticsSubscription service, or requests to get from NWDAF analytics information by using Nnwdaf_AnalyticsInfo service.

- NWDAF collects data from the data sources (e.g. NF such as AMF, SMF, PCF, and AF; OAM) as a basis of the computation of network analytics.

- NWDAF derives requested analytics information based on the data collected from the data source(s).

- NWDAF provides requested analytics information to the NWDAF service consumer by using Nnwdaf_AnalyticsInfo_Request Response or Nnwdaf_AnalyticsSubscription_Notify service operation.

The existing procedures for data collection (step 2 as mentioned above) are triggered by the request from NWDAF service consumer and require the NWDAF to subscribe to the data sources in order to be notified for data collection on a related event (s) (except for the data collection from NRF). And the data analytics takes place after the completion of the corresponding data collection. The observation of the existing mechanism so far is that:

-	The data collection and analytics derivation cannot be done before receiving an analytics request from the NWDAF service consumer.

-	The operation of data/event subscription may take extra time for NWDAF to wait the subscription conditions to be met in the data source(s).

For preventing the extra time caused by the existing mechanism, this solution proposes to introduce a new type of NWDAF: Real-Time communication capable NWDAF (RT-NWDAF) which can process data pre-collection and pre-analytics before receiving the analytics request from the NWDAF service consumer. An RT-NWDAF should start data collection for specific Analytics ID(s) when it completes the NF service Registration procedure as specified in clause 4.17.1, TS 23.502 [x], and continuously derive the analytics results in order to respond to the analytics requester NF in as soon as possible.

2	Proposal

It is proposed to agree the following changes in TR 23.700-91.

FIRST CHANGE

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | 11 |  |  |  |
| 2 | 4 |  |  |  |
| 3 | 15 |  |  |  |
| 4 | 12 |  |  |  |
| X | 18 |  |  |  |

NEXT CHANGE (all new text)

6.x	Solution #X: Pre-analytics based solution for Real-Time communication with NWDAF

6.x.1	Description

Editor's note:	Describe the solutions. (sub) clause(s) may be added to capture details, input data, output analytics, procedural flow etc. if necessary.

6.x.1.1	General

This solution addresses key issue #18: Enhancement for real-time communication with NWDAF.

Based on the procedures defined in TS 23.288 [x], an NWDAF analytic process may include the following steps:

- NWDAF service consumer subscribe at NWDAF to be notified on analytics information by using Nnwdaf_AnalyticsSubscription service, or requests to get from NWDAF analytics information by using Nnwdaf_AnalyticsInfo service.

- NWDAF collects data from the data sources (e.g. NF such as AMF, SMF, PCF, and AF; OAM) as a basis of the computation of network analytics.

- NWDAF derives requested analytics information based on the data collected from the data source(s).

- NWDAF provides requested analytics information to the NWDAF service consumer by using Nnwdaf_AnalyticsInfo_Request Response or Nnwdaf_AnalyticsSubscription_Notify service operation.

The existing procedures for data collection (step 2 as mentioned above) are triggered by the request from NWDAF service consumer and require the NWDAF to subscribe to the data sources in order to be notified for data collection on a related event (s) (except for the data collection from NRF). And the data analytics takes place after the completion of the corresponding data collection. The observation of the existing mechanism so far is that:

-	The data collection and analytics derivation cannot be done before receiving an analytics request from the NWDAF service consumer.

-	The operation of data/event subscription may take extra time for NWDAF to wait the subscription conditions to be met in the data source(s).

For preventing the extra time caused by the existing mechanism, this solution proposes to introduce a new type of NWDAF: Real-Time communication capable NWDAF (RT-NWDAF) which can process data pre-collection and pre-analytics for some specific Analytics ID(s) before receiving the analytics request from the NWDAF service consumer. An RT-NWDAF should start data collection for specific Analytics ID(s) when it completes the NF service Registration procedure as specified in clause 4.17.1, TS 23.502 [x], and continuously derive the analytics results in order to respond to the analytics requester NF as soon as possible.

Editor's note:	Real-time communication may only apply to some of the specific Analytic IDs. Which specific Analytic IDs are applicable to real-time communication is FFS.

RT-NWDAF should send the real-time communication capability and the Analytics ID(s) which applicable to real-time communication to the NRF as part of its NF Profile during the NF service Registration procedure as specified in clause 4.17.1, TS 23.502 [x]. The NRF should take the real-time communication capability information into account during the NWDAF discovery procedure performed by NWDAF service consumer.

During the NWDAF discovery procedure (as specified in clause 4.17.4 or 4.17.9, TS 23.502 [x]), the NWDAF service consumer may indicate whether it expects real-time communication with NWDAF for specific Analytics ID(s) in the service discovery request. Based on the information corresponding to the real-time communication expectation received from the NWDAF service consumer, the NRF selects a suitable RT-NWDAF for the NWDAF service consumer. In case of delegate service discovery, the indication of whether real-time communication with NWDAF is needed is included in the request sent from the NWDAF service consumer to the SCP. The NWDAF service consumer then establishes the Analytics Request procedure (as specified in clause 6.1.2, TS 23.288) to the selected RT-NWDAF.

NOTE:	When establishing a real-time communication with a NWDAF, the NWDAF service consumer should only use the Analytics Request procedure as specified in clause 6.1.2, TS 23.288 [x].

Upon receiving the Analytics Request from the NWDAF service consumer, the RT-NWDAF provides the requested analytics information to the NWDAF service consumer based on the pre-analytics result without collecting data from the data source(s) for this individual Analytics Request.

6.x.1.2	RT-NWDAF register

The following impacts are applicable to clause 4.17.1, TS 23.502 [x] when an RT-NWDAF becomes operative for the first time:

-	Step 1: RT-NWDAF sends Nnrf_NFManagement_NFRegister Request message to NRF, includes the real-time communication capability information and corresponding Analytics ID(s) applicable to real-time communication as part of its NF Profile.

6.x.1.3	RT-NWDAF discovery

The following impacts are applicable to clause 4.17.4, TS 23.502 [x] when a NWDAF service consumer requests the NRF for the discovery of an RT-NWDAF:

-	Step 1: The NWDAF service consumer sends Nnrf_NFManagement_NFDiscover Request message to NRF, includes an indication which indicates it expects real-time communication with NWDAF for specific Analytics ID(s).

The following impacts are applicable to clause 4.17.9, TS 23.502 [x] when a NWDAF service consumer requests for the discovery of an RT-NWDAF with delegated service discovery:

-	Step 1: The NWDAF service consumer sends the service request message to SCP, includes an indication which indicates it expects real-time communication with NWDAF for specific Analytics ID(s).

6.x.1.4	RT-NWDAF based data pre-collection and pre-analytics

Figure 6.x.1.4-1: RT-NWDAF based Data pre-collection and pre-analytics procedure

0.	RT-NWDAF successfully registered in NRF.

1.	RT-NWDAF subscribes to the data source for necessary data/event reporting by sending Data collection Subscribe Request. In cases of different scenarios, the Data collection Subscribe procedure may be:

-	Nnf_EventExposure_Subscribe in case of the data source is an NF.

-	Nnef_EventExposure_Subscribe in case of the data source is an AF. In this case, the service data subscribe request should be forwarded from NEF to AF by invoking Naf_EventExposure_Subscribe service operation.

-	Nnrf_NFManagement_NFStatusSubscribe in case of the data source is an NRF.

-	Subscribe (Input) in case of the data source is an OAM.

The RT-NWDAF should additionally include a continuous data reporting indication in the Data collection Subscribe Request to indicate the data source to report the subscribed data/event continuously to the RT-NWDAF until the RT-NWDAF unsubscribes the data/event reporting. The immediate reporting flag may be used by the RT-NWDAF, if available, to obtain the current status of the subscribed event.

NOTE:	In case of the data source is an NRF, the RT-NWDAF may invoke Nnrf_NFDiscovery service before invoking Nnrf_NFManagement_NFStatusSubscribe to obtain the current NF information from the NRF.

The data source responds the request by sending Data collection Subscribe Response to the RT-NWDAF.

2.	Data source notifies to the RT-NWDAF the subscribed data/event continuously based on the continuous data reporting indication received from RT-NWDAF in Data collection Subscribe Request. The data/event notification can be done by:

-	Nnf_EventExposure_Notify in case of the data source is an NF.

-	Naf_EventExposure_Notify in case of the data source is an AF. In this case, the service data notification should be forwarded from NEF to RT-NWDAF by invoking Nnef_EventExposure_Notify service operation.

-	Nnrf_NFManagement_NFStatusNotify in case of the data source is an NRF.

-	Notification (notifyFileReady) in case of the data source is an OAM.

3.	The RT-NWDAF processes pre-analytics for the Analytics ID(s) applicable to real-time communication based on the data/event reported by the data source. The RT-NWDAF should keep updating the analytics result of specific Analytics ID(s) when receiving new notification from the data source.

4.	The RT-NWDAF sends Data collection Unsubscribe Request to data source when it decides not to collect data from the data source. The Data collection Unsubscribe procedure may be:

-	Nnf_EventExposure_Unsubscribe in case of the data source is an NF.

-	Nnef_EventExposure_Unsubscribe in case of the data source is an AF. In this case, the service data subscribe request should be forwarded from NEF to AF by invoking Naf_EventExposure_Unsubscribe service operation.

-	Nnrf_NFManagement_NFStatusUnsubscribe in case of the data source is an NRF.

-	Unsubscribe operation in case of the data source is an OAM.

The data source stops reporting data/event to the RT-NWDAF and responds the request by sending Data collection Unsubscribe Response to the RT-NWDAF.

6.x.1.5	Analytics Request

Figure 6.x.1.5-1: Network data analytics Request to an RT-NWDAF

1.	The NWDAF service consumer requests analytics information by invoking Nnwdaf_AnalyticsInfo_Request service operation, and includes real-time communication flag per Analytics ID(s) in the request message. The Nnwdaf_AnalyticsInfo_Request message may be forwarded by an NEF to the RT-NWDAF in case of the NWDAF service consumer is an AF.

2.	If the RT-NWDAF cannot provide the requested analytics from the pre-analytics results derived based on the pre-collected data (e.g. analytics for predictions.), it should derive the requested analytics base on the trained data model and the Analytics Reporting Information included in the Nnwdaf_AnalyticsInfo_Request message.

NOTE:	The trained data model is derived by the RT-NWDAF itself during the RT-NWDAF based data pre-collection and pre-analytics procedure or shared from other NWDAFs relies on the conclusion of KI#19.

3.	The NWDAF responds with analytics information to the NWDAF service consumer.

6.x.2	Impacts on services, entities and interfaces

Editor's note:	This clause lists impacts to services and interfaces.

RT-NWDAF:

-	Supports to include the real-time communication capability and corresponding Analytics ID(s) applicable to real-time communication as part of its NF Profile during the NF service Registration Procedure.

-	Supports to process continuous data collection after the NF service Registration procedure without receiving analytics request from NWDAF service consumer.

-	Supports to process pre-analytics for specific Analytics ID(s) continuously after obtaining the necessary data.

NRF:

-	Supports to maintain the NF Profile of RT-NWDAF including the real-time communication capability.

-	Supports to discover a RT-NWDAF based on corresponding indication received from the NWDAF service consumer for real-time communication with NWDAF for specific Analytics ID(s).

NWDAF service consumers:

-	Supports to include an indication in the NF service discovery request which indicates that real-time communication with NWDAF for specific Analytics ID(s) is needed.

-	Supports to include real-time communication flag per Analytics ID(s) in the Nnwdaf_AnalyticsInfo_Request message.

END OF CHANGES