SA WG2 Meeting #139E	S2- 2004082

1 June – 12 June 2020

Source:	Samsung

Title:	KI #12, New Sol: NWDAF-assisted RFSP index selection

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_ph2/ Rel-17

Abstract of the contribution: This contribution proposes a new solution for NWDAF assisted RFSP selection.

- Discussion

This is a solution for Key issue #12: NWDAF-assisted RFSP policy.

In the 5G era, there could be various radio access technologies co-existence in a same region. Each different RAT/frequency may provide different communication characteristics to the user in terms of bandwidth, latency, coverage, and power consumption. Therefore, it is desirable to guide a user to use best RAT/frequency according to the usage type to manage radio resources to maximize the efficiency.

The objective of this solution is to help keeping the entire network with proper resources without degrading of services. To archive this objective, it is essential to figure out the current and future radio demand per RAT/frequency. If RAT/frequency is figured out with low or no demand in current or near future

As described in Use Case #1, MNO able to 1) inter-RAT selection between NR and E-UTRA, and 2) frequency selection such as low and high frequency. This contribution proposed a solution to allow 5GC to select or lead UE’s RAT/frequency based on RFSP index selection mechanism. According to the TS23.501, RFSP index could be used 1) to derive UE specific cell reselection priorities to control idle mode camping, and 2) to decide on redirecting active mode UEs to different frequency layers or RATs. The solution provides statistics/predictions to PCF to decide proper RFSP index to lead UEs into desirable RAT/frequency by NWDAF. The solution proposals are below.

Proposal. NWDAF provides statistics/predictions on the number of users and data volume per RAT and frequency band.

2.	Proposal

It is proposed to include the added text into the TR 23.700-91 as a solution for use case #1 and key issue #X.

* * * * First Change (all new text) * * * *

6.0	Mapping Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | 11 |  |  |  |
| 2 | 4 |  |  |  |
| 3 | 15 |  |  |  |
| 4 | 12 |  |  |  |
| X | 12 |  |  |  |

* * * * Second Change (all new text) * * * *

6.X	Solution X: RAT/Frequency usage analytics

6.X.1	Description

This is a solution for Key issue #12: NWDAF-assisted RFSP policy.

In the 5G era, there could be various radio access technologies co-existence in a same region. Each different RAT/frequency may provide different communication characteristics to the user in terms of bandwidth, latency, coverage, and power consumption. Therefore, it is desirable to guide a user to use best RAT/frequency according to the usage type to manage radio resources to maximize the efficiency.

The objective of this solution is to help keeping the entire network with proper resources without degrading of services. To archive this objective, it is essential to figure out the current and future radio demand per RAT/frequency. If RAT/frequency is figured out with low or no demand in current or near future

As described in Use Case #1, MNO able to 1) inter-RAT selection between NR and E-UTRA, and 2) frequency selection such as low and high frequency. This contribution proposed a solution to allow 5GC to select or lead UE’s RAT/frequency based on RFSP index selection mechanism. According to the TS23.501, RFSP index could be used 1) to derive UE specific cell reselection priorities to control idle mode camping, and 2) to decide on redirecting active mode UEs to different frequency layers or RATs.

The solution provides statistics/predictions to PCF to decide proper RFSP index to lead UEs into desirable RAT/frequency by NWDAF. NWDAF provides analytics information about the current/future usage of RAT/frequency for a specific area. For example, in region A, 20 E-UTRA users and 5 NR users, and 3 users are using low frequency NR and 2 users are using high frequency NR. Based on this analytics, PCF or OAM can decide to manage base stations or radio resources by figuring out the usage demand per RAT/frequency. This solution is based on the selection of RFSP index per UE, not impact to any existing RAN or OAM mechanism.

6.X.1.1	Information for the support of RFSP index selection

The consumer of these analytics shall indicate in the request or subscription:

-	Analytics Id set to "Radio/Frequency usage";

-	The Target of Analytics Reporting: Area of Interest;

-	Analytics Filter Information: one or more combinations of the following Analytics Filters: S-NSSAI, RAT type;

-	An Analytics target period that indicates the time window for which the statistics or predictions are requested; and

-	In a subscription, the Notification Correlation Id and the Notification Target Address.

6.X.2	Input Data

The input data for the analytics is described in Table 6.X.2-1.

Table 6.X.2-1. Input data for Radio/Frequency usage

| Information | Source | Description |
| --- | --- | --- |
| S-NSSAI | AMF | Identifies network slice |
| Location | AMF | The location of UE |
| RFSP index | AMF | Currently configured RSFP index for a UE |
| QoS flow Bit Rate | SMF | The observed bit rate (UL/DL) |
| Packet transmission | SMF | The observed number of packet transmission (UL/DL) |

Editor’s Note: It is FFS whether other data is needed or not, and how to collect input data for the analytics.

6.X.3	Output Analytics

The output analytics of NWDAF is defined in Table 6.X.1.1-1.

Table 6.X.1.1-1: Radio/Frequency usage statistics

| Information | Description |
| --- | --- |
| S-NSSAI (optional) | Identifies the Network Slice for which analytics information is provided. |
| Total number of user | The total number of users |
| Number of user per RAT type | The number of users per specific RAT type (e.g. NR, E-UTRA) |
| Number of user per frequency | The number of users per specific RAT type (e.g. low, high) |
| Total data volume | The total data volume on RAN |
| Data volume per RAT type | The data volume per RAT type (e.g. NR, E-UTRA) |
| Data volume per frequency | The data volume per frequency (e.g. low, high) |

Table 6.X.1.1-1: Radio/Frequency usage prediction

| Information | Description |
| --- | --- |
| S-NSSAI (optional) | Identifies the Network Slice for which analytics information is provided. |
| Predicted Total number of user | The total number of users |
| Predicted Number of user per RAT type | The number of users per specific RAT type (e.g. NR, E-UTRA) |
| Predicted Number of user per frequency | The number of users per specific RAT type (e.g. low, high) |
| Predicted Total data volume | The total data volume on RAN |
| Predicted Data volume per RAT type | The data volume per RAT type (e.g. NR, E-UTRA) |
| Predicted Data volume per frequency | The data volume per frequency (e.g. low, high) |

6.X.4	Procedure

Figure 6.X.1.2-1: Procedure to support RFSP index selection

Step 0. AMF and PCF create an association to handle UE’s AM policy using Npcf_AMPolicyControl_Create.

Step 1. PCF decide to adjust UE’s RFSP index for internal management objective such as energy saving.

NOTE: 	Configuring and internal actions of PCF to internal management objective is out of scope.

Step 2. PCF request to subscribe “RAT/Frequency usage” analytics to figure the current/future time to NWDAF. Target of reporting could be area of interest.

Step 3. The NWDAF collects necessary data from NFs/OAM, and derive “RAT/Frequency” analytics for the request.

Step 4. The NWDAF notify the analytics outputs listed in Table 6.X.1.1-1.

Step 5. PCF derive new RFSP index suitable for the internal objective in step 1 for the UEs.

Step 6. PCF deploy the RFSP index to AMF using AM policy control to UEs.

6.X.5	Impacts on services, entities and interfaces

NWDAF:

- The NWDAF collect data from NFs/OAM, and analyse RAT/Frequency usage analytics.

PCF:

Receives the RAT/Frequency usage analytics from the NWDAF and consider it for deciding RFSP index.

* * * * End of change * * * *