SA WG2 Meeting #139e 	S2-2004384

# 01-12 June 2020, Online

Source:	Alibaba Group.

Title:	Solution for UE onboarding and remote provisioning

Document for:	Approval

Agenda Item:	8.2

Work Item / Release:	FS_eNPN / Rel-17

Abstract of the contribution: This contribution proposes a solution for UE onboarding and provisioning.

Discussion

According to the requirement of Key issue #4 defined in TR 23.700 and the requirement defined in TS 22.263: “The 5G system shall support a secure mechanism for a network operator of an NPN to remotely provision the non-3GPP identities and credentials of a uniquely identifiable and verifiably secure IoT device.” The solution for UE onboarding and remote provisioning should cover both the 3GPP credentials and the non-3GPP credentials cases. It requires the solution to have enough flexibility that can work in both cases.

Device onboarding is a well-studied topic by the standards community and the industry. For example, IETF RATS (Remote Attestation Procedures) working group [1] has developed a flexible architecture and related protocols that can be used for UE onboarding. Leveraging existing and well-designed architectures for UE onboarding have the following advantages:

1. Better ecosystem support for adoption.

The stakeholders of the UE onboarding ecosystem include chipset manufactures, device manufactures, third-party service providers, NPN operators, PLMN operators etc.

Successfully adopting of the NPN UE onboarding and provisioning standard requires all of the stakeholders in the ecosystem implement their production using the same architecture and the same protocols. Most of those stakeholders in the UE onboarding ecosystem have been working in IETF RATS working group and it is easier for them to adopt an existing well-designed architecture and standard that are defined by IETF. Leveraging existing well-designed architecture and standard defined by IETF will help the industry to adopt the 3GPP NPN UE onboarding and remote provisioning standards.

2. Maximum the flexibility.

The UE onboarding and provision solution should have the flexibility to work for 3GPP credentials, non-3GPP credentials cases and should support different deployment scenarios. The IETF RATS architecture (“Remote Attestation Procedures Architecture” [2]) provides enough flexibility to fulfill those requirements.

The architecture of this solution uses the concept of the attestation architecture that is defined in IETF draft: “Remote Attestation Procedures Architecture” [2]. The UE functions as the “Attester” role, the provisioning server functions as the “Relying Party” role and the DCS (Default Credential Server) functions as the “Verifier” role that is defined in IETF draft “Remote Attestation Procedures Architecture” [2].

This solution also allows the provisioning server to dynamically generate and provision credentials to the UE and the correspondent network entity that used to store UE’s credentials after successful onboarding. Enabling the provisioning server to dynamically generate credentials provides maximum flexibility for deployment. For example, it allows the SNPN operator to purchase commercial off-the-shelf IoT UE without preconfiguring credential for both the UE side and the SNPN network side. This will simplify the SNPN deployment.

[1] IETF RATS working group: https://datatracker.ietf.org/wg/rats/

[2]: IETF draft “Remote Attestation Procedures Architecture”: https://tools.ietf.org/html/draft-ietf-rats-architecture-02

Proposal

It is proposed to adopt the following solution into TR 23.700-07.

# * * * * First change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for next generation new services and markets".

[3]	3GPP TS 22.263: " Service requirements for Video, Imaging and Audio for Professional Applications (VIAPA)".

[4]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[5]	3GPP TS 23.122: "Non-Access-Stratum (NAS) functions related to Mobile Station in idle mode".

[6]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[7]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[8]	IETF draft: “Remote Attestation Procedures Architecture”. ( draft-ietf-rats-architecture-04)

[9]	IETF draft: “The Entity Attestation Token (EAT)”. (draft-ietf-rats-eat-03)

# * * * * Second change * * * *

## 6.0	Mapping Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 |
| 1 | X |  |  |  |  |  |
| 2 | X |  |  |  |  |  |
| 3 | X |  |  |  |  |  |
| 4 | X |  |  |  |  |  |
| 5 |  |  |  | X |  |  |
| 6 |  |  |  | X |  |  |
| 7 |  |  |  | X |  |  |
| x |  |  |  | X |  |  |

# * * * *Third change （new text）* * * *

6.X	Solution #X: Solution for UE onboarding and remote provisioning

6.X.1	Introduction

This solution addresses key issue#4 “UE Onboarding and remote provisioning”.

6.X.2	Functional Description

Figure 6.X.2-1: UE Onboarding and provisioning architecture

Figure 6.X.2-1 illustrates the architecture of UE onboarding and provisioning.

This solution uses the device attestation architecture that is defined in IETF draft “Remote Attestation Procedures Architecture”. The UE functions in the “Attester” role, the provisioning server functions as the “Relying Party” role and the DCS (Default Credential Server) functions as the “Verifier” role that defined in IETF draft “Remote Attestation Procedures Architecture”.

The solution has the following assumption:

1. UE: The manufacturer provisions universally unique identity (Universal Entity ID) into the UE during manufacturing.

The manufacturer may also provision secret attestation key material (AKM) into the UE during manufacturing which is used to sign the UE’s attestation token and the DCS (verifier) can verify the UE’s attestation token using the correspondent AKM that is pre-configured in DCS.

The UE may have many different execution environments operating with different security levels to store the AKM and other information such as TEE (Trusted Execution Environment), eSE (embedded Secure Element) etc.

2. DCS (Default Credential Server): The DCS is configured with the correspondent attestation key material (AKM) which is used to verify UE’s attestation token. The DCS functions as the “verifier” as defined in IETF draft “Remote Attestation Procedures Architecture”.

3. Provisioning Server: Provisioning server functions the relaying party as defined in IETF draft “Remote Attestation Procedures Architecture”.

6.X.3	Procedures

Figure 6.X.3-1 shows the high-level procedure of the solution.

Figure 6.X.3-1: Option 2: Onboarding and provisioning Procedures

Step 1: Onboarding - attestation.

The UE establishes the network connectivity that can communicate with the provisioning server. This is achieved by defining a new registration type which allows the onboarding SNPN network to provide limited connectivity PDU session that is only allowing the UE to access the provisioning server.

After establishing the network connectivity for onboarding, the UE starts the onboarding procedure.

The UE sends attestation token which contains the universal UE identity and signing by attestation key material to the provisioning server by application layer protocol. One option of the protocol is based on CoAP. The provisioning server functions as the relaying party defined in IETF draft “Remote Attestation Procedures Architecture” and forwards the attestation token to the DCS. The DCS verifies the attestation token’s signing using the attestation key material.

Step 2: Onboarding - provisioning.

If the UE completes the onboarding-attestation procedure successfully in step 1:

The provisioning server stores the UE’s credential used for the SNPN authentication. How the UE’s credentials created/configured in the provisioning server is out of scope.

Editor's note: The detailed format of UE’s credential for desired SNPN authentication is FFS.

Step 2.1, 2.2: The provisioning server pushes the UE’s credentials to the network entity that stores the UE’s credentials for authentication. The desired SNPN stores UE’s credentials in UDM, the provisioning server pushes the UE’s credentials to the SNPN’s UDM via the NEF.

Editor's note: The detailed service interface/API between the provisioning server and the NEF is FFS.

Step 2.3: The provisioning server pushes the UE’s credentials to the UE.

Step 3: Onboarding -  UE selects the desired SNPN and authentication.

The UE selects the desired SNPN and starts the authentication procedure using the credentials that provisioned in Step 2. The SNPN selection may be based on the broadcast information sent by the SNPN.

Editor’s Note: It is FFS how the SNPN selects the desired SNPN.

6.X.4	Impacts on existing entities and interfaces

Provisioning Server/NEF:

The service interface between the provisioning server and the NEF, the service interface between the NEF and the UDM need to be defined.

UE and AMF:

The new registration type which allows the onboarding SNPN network to provide limited PDU session connectivity that is only allowing the UE to access the provision server need to be defined.

# * * * * End of changes * * * *