SA WG2 Meeting #139E	S2-2003749r02

Elbonia, June 1- 12, 2020	(revision of S2-200xxxx)

Source:	Apple

Title:	KI #5, New Sol: UAV Authorisation revocation by UTM via C2-Command

Document for:	Approval

Agenda Item:	8.7

Work Item / Release:	FS_ID_UAS-SA2 / Rel-17

Abstract: This contribution proposes a solution for Key Issue #5 – UAV authorization revocation and (re)authorization failures in TR 23.754.

# 1. Introduction

We propose to include this solution for Key Issue #5 in TR 23.754.

START of CHANGE

## 6.0	Mapping Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 1 |  |  |  | X |  |  |  |
| 2 |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |
| X |  |  |  |  | X |  |  |

NEXT CHANGE (All text is new)

## 6.X	Solution #X: UAV Authorisation revocation by UTM via C2-Command

### 6.X.1	Introduction

This solution is for the following Key Issue #5: UAV authorization revocation and (re)authorization failures.

-	How are UAV handled in case of failed (re)authorization or revocation of authorization by the UTM, considering handling of UAV connectivity with UAV Controller and expected UAV behavior.

### 6.X.2	Functional Description

This solution introduces a method for authorisation revocation of UAV(s) initiated by UTM by performing deregistration from the 3GPP network when the UAV(s) is deemed to be in safe place.

### 6.X.3	Procedures

#### 6.X.3.1      UAV Authorisation revocation by UTM

The procedure in Figure 6.X.3.1-1 shows a signalling flow of an UAV authorisation revocation by the 3GPP network initiated by UTM:

Figure 6.X.3.1-1: UAV Authorisation Revocation by UTM

- UAV is registered to 5GC, connected to a UAC via user plane over an active PDU session for C2-Communication.

Editor’s Note: Details of the C2 application layer protocols are outside the scope of 3GPP.

- UTM decides to revoke the authorization of UAV(s).

NOTE 1:	UTM can decide to revoke UAV’s authorisation for various reasons. For example, repeated violations of UAV aerial in no-drone locations.

3.	UTM sends a C2-Command to the UAV(s) to revoke their authorisation. The C2-command is sent over the user plane using the PDU session that was active at step 1.

4.	UAV disconnects from UAC.

Editor’s Note: 	How the solution ensures that the UAV is actually disconnected from the UAV controller is FFS. The solution assumes that at application layer there are mechanisms to enforce this.

5.	Once the UTM decides that UAV is in a safe location, UTM sends a Deregistration request to AMF via NEF/AF. The Deregistration request contains the UAV identity (Remote ID) and a cause code.

Editor’s Note: how the Remote ID is used as UAV identity by the 3GPP system is FFS.

6.	AMF sends the Deregistration request message to UAV(s) with a cause code (e.g. Illegal UE, Illegal ME or a new cause code specific to UAV authorisation revocation).

7.	UAV(s) sends Deregistration Accept message.

NOTE: Steps 5-7 are optional. The UAV could stay registered to the 3GPP network.

Editor’s Note: The need for de-registering the UAV is FFS.

### 6.X.4	Impacts on existing entities and interfaces

-	AMF - UTM communication needs to be defined.

-	Optionally, a new 5GMM cause code can be defined for deregistration of UAV(s).

Editor’s Note: 	The need for a 5GMM cause code is FFS.

### 6.X.5 	Evaluation

Editor's note:	This clause provides an evaluation of the solution.

End of CHANGES