SA WG2 Meeting #139E (e-meeting)						S2-2xxxx

- June 01 - 12, 2020, Elbonia

Title:	[Draft] ReplyLS on service area restriction for CIoT 5GS optimization

Response to:	LS (S2-20xxxx) on service area restriction for CIoT 5GS optimization

Release:	Release 16

Work Item:	5G_CIoT

Source:	SA WG2

To:	SA WG1, CT WG1

Cc:

Contact Person:

Name:	Lalith Kumar

Tel. Number:

E-mail Address:	lalith.kumar@samsung.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	S2-2xxxxx

1. Overall Description:

SA2 thanks CT1 for the LS on service area restriction for CIoT 5GS optimization.

CT1 has asked the following question:

Is the UE using CP CIoT or UP CIoT optimization allowed to send or receive data in a non-allowed area, including exception data?

SA2 Answer:

The service area restriction applies to both Control Plane CIoT 5GS Optimisations and User Plan CIoT 5GS Optimisations. Based on local policy, the network may page the UE in Non-Allowed Area, and once the UE is in CM-CONNECTED state, the network may send DL data and UE may send UL data (as it can be acknowledgement of DL data).

SA WG2 would like to ask SA WG1:

Can UE send MO exception data in non-allowed area?

2. Actions:

TO SA WG1:

ACTION: 	SA WG2 kindly asks SA WG1 to clarify the service requirements for MO exception data.

To CT WG1

ACTION: 	SA WG2 kindly asks CT WG1 to take the above information into account.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #140		24 – 28 Aug 2020		Sophia-Antipolis, FR

TSG-SA2 Meeting #141		12 – 16 Oct 2020		Tallinn, Estonia