3GPP TSG-WG SA2 Meeting #139E e-meeting 	S2-2004029r03

Elbonia, June 1 – 12, 2020	(revision of S2-200xxxx)

Source:	China Mobile, AsiaInfo, CATT

Title:	KI #2, New Sol, Federated Learning among Multiple NWDAF Instances

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_Ph2 / Rel-17

Abstract: This contribution proposes a solution for Key Issue#2: Multiple NWDAF instances and Key Issue #19: Trained data model sharing between multiple NWDAF instances based on Federated Learning.

1. Discussion

This solution is related to the Key Issue#2: Multiple NWDAF instances and Key Issue #19: Trained data model sharing between multiple NWDAF instances.

Considering the actual deployment for the feature of eNA R16, it may still faces some major challenges as follows:

-	In 5GS, data exists in the form of isolated islands, which means it is difficult for NWDAF to centralize the data from different domains (e.g. UE, RAN, TN, CN and AF).

-	Data privacy and security have become a worldwide issue so it is also difficult for NWDAF to collect UE level network data, especially directly collect data from UE.

-	If the NWDAF is deployed in a centralized manner and implemented by the 3rd party, the network data may be exposed to the 3rd party, which may lead to the network data leakage or mis-used.

In this sense, Federated Learning (also called Federated Machine Learning) could be a possible solution, in which there is no need for raw data transferring (e.g. centralized into single NWDAF) but only need for ML model or ML model parameter sharing among NWDAFs.

As shown in Figure 1, the main idea of Federated Learning is to build globally optimized machine-learning models based on data sets that are distributed in different domains or network functions or UEs. A Client NWDAF (e.g. deployed in a domain or network function or UE) locally trains the local ML model with its own data and share it or its ML parameters to the server NWDAF. With local models or model parameter from different Client NWDAFs, the Server NWDAF could form a global or optimal model and send it back to the Client NWDAFs for inference.

Figure 1: Example architecture for a Federated Learning system

Imagine that we have N data owners F1,…,FN (e.g. network functions), all of whom wish to train a machine learning model by consolidating their respective data D1,…,DN. In eNA R16, the method is to put all data together into NWDAF and use D=D1∪…DN to train a model MSUM. A federated learning system is a learning process in which the data owners collaboratively train a model MFED, in which process any data owner Fi does not expose its data Di to the NWDAF. In addition, the accuracy of MFED, denoted as VFED should be very close to the performance of MSUM, VSUM. Formally, let δ be a non-negative real number, if

VFED-VSUM<δ

We then say the federated learning algorithm has δ -accuracy loss.

This solution tries to involve the idea of Federated Learning into the NWDAF-based architecture, which aims to investigate the following aspects:

-	Registration and discovery of multiple NWDAF instances that support Federated Learning;

-	How to share the ML models or ML model parameters during the Federated Learning training procedure among multiple NWDAF instances;

2. Text Proposal

It is proposed to add the following changes in TR 23.700-91 v0.3.0.

# * * * * First change * * * *

6.X	Solution #X: Federated Learning among Multiple NWDAF Instances

6.X.1	Description

This is a solution for the Key Issue#2: Multiple NWDAF instances and Key Issue #19: Trained data model sharing between multiple NWDAF instances.

6.X.1.1	General

As shown in Figure 6.X.1-1, multiple NWDAF will be deployed in a big PLMN, so maybe it is difficult for NWDAF to centralize all the raw data that are distributed in different Areas. However, it is desired or reasonable for the NWDAF distributed in an Area to share its model or data analytics with others NWDAFs.

Figure 6.X.1-1: Hierarchical NWDAF deployment in a PLMN

Federated Learning (also called Federated Machine Learning) could be a possible solution to handle the issues such as data privacy and security, model training efficiency, and so on, in which there is no need for raw data transferring (e.g. centralized into single NWDAF) but only need for model sharing.

The main idea of Federated Learning is to build machine-learning models based on data sets that are distributed in different network functions. A Client NWDAF (e.g. deployed in a domain or network function) locally trains the local ML model with its own data and share it to the server NWDAF. With local ML models from different Client NWDAFs, the Server NWDAF could aggregate them into a global or optimal ML model or ML model parameters and send them back to the Client NWDAFs for inference.

This solution tries to involve the idea of Federated Learning into the NWDAF-based architecture, which aims to investigate the following aspects:

-	Registration and discovery of multiple NWDAF instances that support Federated Learning;

-	How to share the ML models or ML model parameters during the Federated Learning training procedure among multiple NWDAF instances.

6.X.1.2	General procedure for Federated Learning among Multiple NWDAF Instances

Figure 6.x.1.2-1: General procedure for Federated Learning among Multiple NWDAF Instances

1~3. Client NWDAF registers its NF profile into NRF.

Editor’s note: Which information are required for the NF profile of client NWDAF is FFS.

4~6. Server NWDAF discovers one or multiple Client NWDAF instances which could be used for Federated Learning via the NRF to get IP addresses of Client NWDAF instances.

7a: Each client NWDAF communicates licensing conditions for its data and training infrastructure to participate in the federated learning task. These conditions can be based on policies set-up based on how sensitive the data is, how much compute is expected to be needed to perform local training, who will get the use the trained model, how revenues obtained through the use of the model will be shared, etc.

7b: Server NWDAF selects which NWDAF clients will participate based on its desired license model and.

7c. Server NWDAF sends analytics request to the Client NWDAFs that participate in the Federated model according to steps 7a and 7b including some parameters (such as data type list, maximum response time window, etc.) to help the local model training for Federated Learning.

Editor's note: 	How to realize the interface and parameters between Server NWDAF and Client NWDAF is FFS.

8. Each Client NWDAF collects its local data by using the current mechanism in clause 6.2, TS 23.288 [5].

9. During Federated Learning training procedure, each Client NWDAF trains local ML model based on its own data and the parameters received from Server NWDAF, and reports the local ML model information (e.g. volume of the local dataset, parameters of the local ML model) to the Server NWDAF.

10. The Server NWDAF aggregates all the local ML model information to derive the aggregated model information.

11. The Server NWDAF sends the aggregated ML model information to each Client NWDAF.

12. Each Client NWDAF then updates the local ML model based on the aggregated model information distributed by the Server NWDAF.

NOTE: The step 9~12 should be repeated until the training termination condition (e.g. maximum number of iterations) is fulfilled.

13. After the training procedure is finished, the globally optimal ML model or ML model parameters could be distributed to the Client NWDAFs for the inference.

6.X.1.3	Procedure for usage of Federated Learning in Abnormal Behaviour

Figure 6.x.1.3-1: Procedure for usage of Federated Learning in Abnormal Behaviour

1. A Server NWDAF determines to train an Abnormal Classifier using the input data defined in Table 6.7.5.2-1, TS 23.288 [5]). The Server NWDAF discovers two Client NWDAFs i.e. Client NWDAF 1 in Area A, Client NWDAF 2 in Area B that are participate in the license model as defined in steps 7 in figure 6.x.1.2-1.

2. Client NWDAF 1 and Client NWDAF 2 collect raw data from SMF and AMF, respectively. The data collection procedures from AMF and SMF are defined in step 2~3, clause 6.7.5.4, TS 23.288 [5].

3~4. Client NWDAF 1 and Client NWDAF 2 collect Exception information from AF, respectively. The Exception information is defined in clause 6.7.5.2, TS 23.288 [5].

Repeat from step 5-8 until the training termination condition (e.g. maximum number of iterations) is fulfilled:

5. During Federated Learning training procedure, based on the local raw data, Client NWDAF 1 and Client NWDAF 2 locally train local Abnormal Classifier, respectively. Or, Client NWDAF 1 and Client NWDAF 2 updates the local Abnormal Classifier based on the aggregated Abnormal Classifier information distributed by the Server NWDAF in step 7.

6. Client NWDAF 1 and Client NWDAF 2 reports the local Abnormal Classifier information (e.g. volume of the local dataset, parameters of the Abnormal Classifier) to the Server NWDAF.

7. Server NWDAF aggregates all the local Abnormal Classifier information to derive the aggregated Abnormal Classifier information.

8. Server NWDAF sends the aggregated Abnormal Classifier information to Client NWDAF 1 and Client NWDAF 2.

9. After the training procedure is finished, the globally optimal Abnormal Classifier or Abnormal Classifier parameters could be distributed to the Client NWDAF 1 and Client NWDAF 2 for the inference.

6.X.2	Impacts on Existing Nodes and Functionality

NWDAF:

-  Registration into NRF;

As a server NWDAF,

-	Client NWDAF instances discovery;

-	Initial Federated Learning parameters determination and distribution;

-  Collection of local ML model information from the Client NWDAF instances;

-	ML Model aggregation and distribution to the Client NWDAF.

As a Client NWDAF,

-	Local dataset collection;

-	Local model training and reporting to the Server NWDAF;

-	Receive aggregated model information from Server NWDAF and perform local ML model update.

- 	Report the licensing conditions.

NRF:

-	Server NWDAF and Client NWDAF registration and discovery.

# * * * * End of changes * * * *