SA WG2 Meeting #S2-139E	S2-2004558

June 01 - 12, 2020, Elbonia	(revision of S2-2004182)

Source:	CATT

Title:	KI #13, New Sol: Triggers for requesting analytics

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_ph2 / Rel-17

Abstract of the contribution: This contribution proposes a new solution for Key Issue #13 "Triggering conditions for analytics".

- Discussion

In Key Issue #13 "Triggering conditions for analytics", it is proposed to study how analytics can be triggered from the analytics consumer perspective or from the NWDAF perspective, including: "

-	which types of triggers can be used by analytics consumers to request for or subscribe to analytics;

-	whether triggers need to be defined for data collection by NWDAF and which types of triggers can be used by NWDAF to start collecting data;

-	whether triggers need to be defined for analytics generation by NWDAF and which types of triggers can be used by NWDAF to build analytics;

-	whether configuration for these triggers is needed;

-	whether changes to 5Gs architecture and services are needed to allow for such triggers and configuration. "

This contribution mainly addresses the triggers which can be used by analytics consumers to request for or subscribe to analytics.

In general, an NF may request for or subscribe to the analytics information from the NWDAF:

-	based on the triggers (e.g. event reports or requests) from the other NF(s).

-	based on local event, e.g. the detected UE behaviour or network performance.

-	based on the analytics information received. That is, the NF may decide to further request for or subscribe to the analytics information (e.g. with different Analytics filters, Target of Analytics Reporting or Analytics IDs) from the NWDAF, upon receiving the analytics information provided by the NWDAF for existing subscription(s) or previous request(s).

For different NFs, the possible triggers are analysed as follows:

PCF:

As specified in TS 23.503, policy decisions based on network analytics allow PCF to perform policy decisions taking into account the analytics information provided by the NWDAF. The following Analytics IDs are relevant for Policy decisions: "Load level information", "Service Experience", "Network Performance" and "Abnormal behaviour".

The PCF may request for or subscribe to the analytics information from the NWDAF:

-	based on the Policy Control Request from the AMF or the SMF.

Upon receiving the Policy Control Request from the AMF or the SMF, the PCF may request the analytics from the NWDAF in order to update policy and charging control decision.

-	based on the analytics information received.

Upon receiving the analytics on "Load level information" which indicates the load level of a network slice reached the threshold for a high load level, the PCF may decide to request for or subscribe to the analytics information on "Abnormal behaviour" to detect whether there are any exceptions regarding the behaviours of the UEs in this network slice.

SMF:

The SMF may request for or subscribe to the analytics information:

-	based on UE access and mobility event reports from the AMF.

When the "number of UEs served by the AMF and located in Area Of Interest" reported by the AMF reaches a predefined threshold, the SMF may subscribe to the analytics on "UE communication" or "Network Performance" with the Analytics Filter including the Area of Interest, to observe the communication performance of the UE(s) or the network in the specific area.

-	based on Policy Control Request Triggers or updated policy and charging control decision from the PCF.

When the SMF receives the Policy Control Request Trigger of "Usage report" from the PCF, the SMF may subscribe to the analytics information on "Abnormal behaviour", to detect whether there are any exceptions on the PDU session or the Monitoring key specific resources consumed by a UE as specified in TS 23.503.

-	based on local events.

If the SMF detects the number of PDU session establishment or release reaches a threshold in a specific area, the SMF may request for or subscribe to the analytics information on "Abnormal behaviour" to detect whether there are any exceptions regarding the behaviours of the UEs in this area.

AMF:

The AMF may request for or subscribe to the analytics information:

-	based on event reporting triggers from the SMF/NEF.

When the AMF receives the event reporting trigger from the SMF/NEF on "UE loss of communication", the AMF may subscribe to the analytics information on "Abnormal behaviour" to detect whether there are any mobility related exceptions.

-	based on local events.

When the AMF detects frequent mobility re-registration of one or more UEs, the AMF may subscribe to the analytics on "Abnormal behaviour" in order to trace the mobility trend of the UE(s) and take appropriate actions.

# 2. Proposal

It is proposed to include the following changes in TR 23.700-91.

***** Start of Change (all new text) *****

## 6.X	Solution for Key Issue #13: Triggers for requesting analytics

### 6.X.1	Description

Editor's note:	Describe the solutions. (sub) clause(s) may be added to capture details, input data, output analytics, procedural flow etc. if necessary.

This solution addresses Key Issue #13 "Triggering conditions for analytics", especially on the aspect of triggers which can be used by analytics consumers to request for or subscribe to analytics from the NWDAF.

In general, an NF may request for or subscribe to the analytics information from the NWDAF:

-	based on the triggers (e.g. event reports or requests) from the other NF(s).

-	based on local event, e.g. the detected UE behaviour or network performance.

-	based on the analytics information received. That is, the NF may decide to further request for or subscribe to the analytics information (e.g. with different Analytics filters, Target of Analytics Reporting or Analytics IDs) from the NWDAF, upon receiving the analytics information provided by the NWDAF for existing subscription(s) or previous request(s).

Specifically, for different NFs, the possible triggers and example scenarios can be as follows:

PCF:

The PCF may request for or subscribe to the analytics information from the NWDAF:

-	based on Policy Control Request from the AMF or SMF.

-	based on analytics information received.

Table 6.X.1-1: Triggers used by PCF to request for or subscribe to analytics

| Triggers | Examples |
| --- | --- |
| Policy Control Request from the AMF or SMF | the PCF may request the analytics from the NWDAF in order to update policy and charging control decision. |
| analytics information received | Upon receiving the analytics on "Load level information" which indicates the load level of a network slice reached the threshold for a high load level, the PCF may request for or subscribe to the analytics information on "Abnormal behaviour" to detect whether there are any exceptional UE behaviours in this network slice. |

SMF:

The SMF may request for or subscribe to the analytics information from the NWDAF:

-	based on UE access and mobility event reports from the AMF.

-	based on Policy Control Request Triggers or updated policy and charging control decision from the PCF.

-	based on local events.

Table 6.X.1-2: Triggers used by SMF to request for or subscribe to analytics

| Triggers | Examples |
| --- | --- |
| UE access and mobility event reports from the AMF | When the "number of UEs served by the AMF and located in Area Of Interest" reported by the AMF reaches a predefined threshold, the SMF may subscribe to the analytics on "UE communication" or "Network Performance" with the Analytics Filter including the Area of Interest, to observe the communication performance of the UE(s) or the network in the specific area. |
| Policy Control Request Triggers or updated policy and charging control decision from the PCF | When the SMF receives the Policy Control Request Trigger of "Usage report" from the PCF, the SMF may subscribe to the analytics information on "Abnormal behaviour", to detect whether there are any exceptions on the PDU session or the Monitoring key specific resources consumed by a UE as specified in TS 23.503. |
| local events | If the SMF detects the number of PDU session establishment or release reaches a threshold in a specific area, the SMF may request for or subscribe to the analytics information on "Abnormal behaviour" to detect whether there are any exceptional UE behaviours in this area. |

AMF:

The AMF may request for or subscribe to the analytics information from the NWDAF:

-	based on event reporting triggers from the SMF/NEF.

-	based on local events.

Table 6.X.1-3: Triggers used by AMF to request for or subscribe to analytics

| Triggers | Examples |
| --- | --- |
| event reporting triggers from the SMF/NEF | When the AMF receives the event reporting trigger from the SMF/NEF on "UE loss of communication", the AMF may subscribe to the analytics information on "Abnormal behaviour" to detect whether there are any mobility related exceptions. |
| local events | When the AMF detects frequent mobility re-registration of one or more UEs, the AMF may subscribe to the analytics on "Abnormal behaviour" in order to trace the mobility trend of the UE(s) and take appropriate actions. |

### 6.X.2	Input Data

### 6.X.3	Output Analytics

### 6.X.4	Procedures

### 6.X.5	Impacts on services, entities and interfaces

Editor's note:	This clause lists impacts to services and interfaces.

***** End of Change *****