3GPP TSG SA WG2 Meeting #139E	S2-2004924

# Elbonia, August 19 – September 01, 2020	(Revision of S2-20xxxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.214 | CR | 0075 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to CUPS Session/Bearer deactivation procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, AT&T, T-Mobile USA |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16, CUPS |  |  |  |  |  | Date: |  |  | 2020-08-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | With MME initiated dedicated bearer deactivation or PDN GW initiated bearer deactivation, TS 23.314 states that PGW-C requests PGW-U to stop counting and stop forwarding packets for the affected bearer(s). However, at that time, SGW-U can still forward uplink payloads to PGW-U and record those packets in CDRs for settlement, which means there will be subscriber charging inconsistencies between PGW-U and SGW-U. Similarly, when SGW-C receives Delete Session Request, TS 23.314 states that SGW-C triggers an Sx Session Modification Request to the SGW-U to discard downlink packets reveived from PGW-U for the affected bearers and discard uplink packets received from eNodeB for the affected bearers. However, at this moment, PGW-U can still forward downlink packets to SGW-U and record those packets in CDRs for charging, which means there will be subscriber charging inconsistencies between PGW-U and SGW-U. Furthermore, the references to 23.060 in Figure 6.3.4.1.2-2 seems not correct, since “step (A)” and “step 1” are actually the same step in 23.060 Figure 77. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For MME initiated dedicated bearer deactivation or PDN GW initiated bearer deactivation, PGW-C triggers an Sx PFCP Modification Request to the PGW-U to stop counting and stop forwarding only downlink packets for the affected bearers, but continue to forward and measure uplink packets. When SGW-C receives Delete Session Request, SGW-C triggers an Sx PFCP Modification Request to the SGW-U to only discard uplink packets received from eNodeB for the affected bearers, but continue to forward and measure downlink packets. Updated references to 23.060 in Figure 6.3.4.1.2-2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PGW uplink and downlink charging is inconsistent with SGW measurements. May lead to unnecessary investigations into settlement discrepancies. Incorrect references to steps in 23.060. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1.6, 6.3.1.7, 6.3.4.1.2, 6.3.4.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First change * * * *

#### 6.3.1.6	Procedures with release of PDN connection

This clause defines interactions between the CP and UP function during the following procedures:

-	UE-initiated Detach procedure for E-UTRAN (TS 23.401 [2] clause 5.3.8.2.1).

-	UE-initiated Detach procedure for GERAN/UTRAN with ISR activated (TS 23.401 [2] clause 5.3.8.2.2).

-	MME-initiated Detach procedure (TS 23.401 [2] clause 5.3.8.3).

-	SGSN-initiated Detach procedure with ISR activated (TS 23.401 [2] clause 5.3.8.3A).

-	HSS-initiated Detach procedure (TS 23.401 [2] clause 5.3.8.4).

-	UE or MME requested PDN disconnection (TS 23.401 [2] clause 5.10.3).

During the above procedures following is the nature of interactions between the CP and UP function:

-	The SGW-C terminates the Sx session at the SGW-U.

-	The PGW-C terminates the Sx session at the PGW-U.

Figure 6.3.1.6-1: Interaction between CP and UP function during release of PDN connection

1.	Procedure as listed in this step is initiated as specified in the relevant clauses of this specification. The relevant steps of the procedure as specified in the figure above are executed.

2.	The SGW-C sends an Sx Session Modification Request to the SGW-U. SGW-C shall indicate SGW-U to stop counting uplink packets for the affected bearers. SGW-C shall also indicate the SGW-U to discard uplink packets received from eNodeB for the affected bearers.

3.	The SGW-U sends an Sx Session Modification Response to the SGW-C.

4.	The relevant steps of the procedure as specified in the figure above are executed.

5.	The PGW-C sends an Sx Session Termination Request to the PGW-U.

6.	The PGW-U sends an Sx Session Termination Response to the PGW-C confirming the successful termination of the Sx session. Additionally, if the usage reporting is configured by the PGW-C (as specified in clause 5.3.2) the PGW-U shall provide the usage report(s) for the PDN connection (as specified in clause 5.3.3) to the PGW-C.

7.	The relevant steps of the procedure as specified in the figure above are executed.

8.	The SGW-C sends an Sx Session Termination Request to the SGW-U.

9.	The SGW-U sends an Sx Session Termination Response to the SGW-C confirming the successful termination of the Sx session. Additionally, if the usage reporting is configured by the SGW-C (as specified in clause 5.3.2) the SGW-U shall provide the usage report(s) for the PDN connection (as specified in clause 5.3.3) to the SGW-C.

* * * Second change * * * *

#### 6.3.1.7	Procedures with modification of bearer

This clause defines interactions between the CP and UP function during the following procedures:

-	Dedicated bearer activation (TS 23.401 [2] clause 5.4.1).

-	PDN GW initiated bearer modification with bearer QoS update (TS 23.401 [2] clause 5.4.2.1).

-	PDN GW initiated bearer modification without bearer QoS update (TS 23.401 [2] clause 5.4.3).

-	PDN GW initiated bearer deactivation (TS 23.401 [2] clause 5.4.4.1).

-	HSS Initiated Subscribed QoS Modification (TS 23.401 [2] clause 5.4.2.2).

-	MME Initiated Dedicated Bearer Deactivation (TS 23.401 [2] clause 5.4.4.2).

During the above procedures following is the nature of interactions between the CP and UP function:

-	The PGW-C modifies the Sx session at the PGW-U.

-	The SGW-C modifies the Sx session at the SGW-U.

Figure 6.3.1.7-1: Interaction between CP and UP function during modification of bearer

1.	Procedure as listed in this step is initiated as specified in the relevant clauses of this specification. The relevant steps of the procedure as specified in the figure above are executed.

2.	The PGW-C sends an Sx Session Modification Request to the PGW-U. For "Dedicated bearer activation", PGW-C indicates PGW-U to allocate F-TEID for the new dedicated bearer. For "PGW/MME initiated bearer deactivation procedure", PGW-C shall indicate PGW-U to stop counting and stop forwarding downlink packets for the affected bearer(s).

3.	The PGW-U sends an Sx Session Modification Response to the PGW-C confirming the successful modification of the Sx session.

4.	The relevant steps of the procedure as specified in the figure above are executed.

5.	The SGW-C may send an Sx Session Modification Request to the SGW-U. For "Dedicated bearer activation", SGW-C indicates SGW-U to allocate F-TEID for the new dedicated bearer.

6.	The SGW-U sends an Sx Session Modification Response to the SGW-C confirming the successful modification of the Sx session.

7.	The relevant steps of the procedure as specified in the figure above are executed.

8.	The SGW-C sends an Sx Session Modification Request to the SGW-U. For dedicated bearer activation procedure, the SGW-C shall include the F-TEIDu of the activated bearer received from the eNodeB. For PDN GW/HSS initiated bearer modification with bearer QoS update procedure, SGW-C provides updated QoS parameters to SGW-U. For PDN GW/MME initiated bearer deactivation procedure, SGW-C shall indicate SGW-U to remove the PDR(s) for the affected bearer(s).

If the default bearer belonging to a PDN connection is deactivated, the SGW-C sends an Sx Session Termination Request to the SGW-U.

9.	The SGW-U sends an Sx Session Modification Response or an Sx Session Termination Response to the SGW-C confirming the successful modification or termination of the Sx session.

10.	The relevant steps of the procedure as specified in the figure above are executed.

11.	The PGW-C sends an Sx Session Modification Request to the PGW-U. For dedicated bearer activation procedure, the PGW-C shall include the F-TEIDu of the activated bearer received from the SGW. For PDN GW/HSS initiated bearer modification with bearer QoS update procedure, PGW-C provides updates according to the changed QoS parameters and TFT (if applicable) to PGW-U. For "PDN GW initiated bearer modification without bearer QoS update" procedure, PGW-C provides updates according to the changed APN-AMBR and/or TFT to PGW-U. For PDN GW/MME initiated bearer deactivation procedure, PGW-C shall indicate PGW-U to remove the PDR(s) for the affected bearer(s).

If the default bearer belonging to a PDN connection is deactivated, the PGW-C sends an Sx Session Termination Request to the PGW-U.

12.	The PGW-U sends an Sx Session Modification Response or an Sx Session Termination Response to the PGW-C confirming the successful modification or termination of the Sx session.

13.	The relevant steps of the procedure as specified in the figure above are executed.

* * * Third change * * * *

##### 6.3.4.1.1	Procedures with SGSN terminating S4 interface

This clause defines interactions between the CP and UP function during the following procedures related to the deactivation of a PDN connection:

Group 1:

-	Combined GPRS / IMSI Attach procedure:

-	Including steps specific for S4 SGSN: Combined GPRS / IMSI Attach procedure, Delete Bearer by the new SGSN, using S4, and Combined GPRS / IMSI Attach procedure, Delete Bearer by the old SGSN, using S4.

-	MS-Initiated Combined GPRS / IMSI Detach Procedure.

-	SGSN-Initiated GPRS Detach Procedure.

-	HLR-Initiated GPRS Detach Procedure:

-	Including steps specific for S4 SGSN: SGSN interaction during Detach Procedure when using S4.

-	MS Initiated Bearer Deactivation Procedure for A/Gb mode (with Teardown Ind).

-	MS Initiated Bearer Deactivation Procedure for Iu mode (with Teardown Ind).

-	SGSN-initiated Bearer Deactivation Procedure (with Teardown Ind):

-	Including steps specific for S4 SGSN: MS-and SGSN Initiated PDN connection Deactivation Procedure using S4.

Group 2:

-	GGSN-initiated PDP Context Deactivation Procedure (with Teardown Ind):

-	Including steps specific for S4 SGSN: PDN GW-initiated Bearer Deactivation Procedure using S4, part 1 and part 2.

During the above procedures, following is the nature of interactions between the CP and UP function:

-	The SGW-C releases the existing Sx session from SGW-U.

-	The PGW-C releases the existing Sx session from PGW-U.

-	The SGW-C modifies the existing Sx session for SGW-U.

-	The PGW-C modifies the existing Sx session for PGW-U.

Figure 6.3.4.1.1-1: Interaction between CP and UP function with PDN connection deactivation Group 1

1.	Procedure as listed in this step is initiated as specified in the relevant clauses of this specification. The relevant steps of the procedure as specified in the figure above are executed.

2a.	The SGW-C sends an Sx Session Modification Request to the SGW-U to modify the Sx session. SGW-C shall request SGW-U to stop counting uplink packets for the affected bearers. SGW-C shall also request the SGW-U to discard uplink packets received from eNodeB for the affected bearers.

2b.	The SGW-U sends an Sx Session Modification Response to the SGW-C confirming the modification of the Sx session.

2c.	The relevant steps of the procedure as specified in the figure above are executed.

3a.	The PGW-C may send an Sx Session Termination Request to the PGW-U to release the Sx session.

3b.	The PGW-U sends an Sx Session Termination Response to the PGW-C confirming the release of the Sx session.

3c.	The relevant steps of the procedure as specified in the figure above are executed.

3d.	The SGW-C may send an Sx Session Termination Request to the SGW-U to release the Sx session.

3e.	The SGW-U sends an Sx Session Termination Response to the SGW-C confirming the release of the Sx session.

4.	The relevant steps of the procedure as specified in the figure above are executed.

5a.	The SGW-C may send an Sx Session Modification Request to the SGW-U in detach and Bearer Deactivation procedures. SGW-C shall indicate SGW-U to stop counting uplink packets for the affected bearers. SGW-C shall also indicate the SGW-U to discard uplink packets received from eNodeB for the affected bearers.

5b.	The SGW-U sends an Sx Session Modification Response to the SGW-C.

6.	The relevant steps of the procedure as specified in the figure above are executed.

7a.	The PGW-C may send an Sx Session Termination Request to the PGW-U to release the Sx session.

7b.	The PGW-U sends an Sx Session Termination Response to the PGW-C confirming the release of the Sx session.

8.	The relevant steps of the procedure as specified in the figure above are executed.

9a.	The SGW-C may send an Sx Session Termination Request to the SGW-U to release the Sx session.

9b.	The SGW-U sends an Sx Session Termination Response to the SGW-C confirming the release of the Sx session.

10.	The relevant steps of the procedure as specified in the figure above are executed.

Figure 6.3.4.1.1-2: Interaction between CP and UP function with PDN connection deactivation Group 2

1.	Procedure as listed in this step is initiated as specified in the relevant clauses of this specification. The relevant steps of the procedure as specified in the figure above are executed.

2a.	The PGW-C sends an Sx Session Modification Request to the PGW-U when it initiates bearer deactivation procedure. PGW-C shall indicate PGW-U to stop counting downlink packets for the affected bearers. PGW-C shall also indicate the PGW-U to discard downlink packets received for the affected bearers.

2b.	The PGW-U sends an Sx Session Modification Response to the PGW-C.

3a.	void.

3b.	void.

4.	The relevant steps of the procedure as specified in the figure above are executed.

5a.	The SGW-C sends an Sx Session Termination Request to the SGW-U to release the Sx session.

5b.	The SGW-U sends an Sx Session Termination Response to the SGW-C confirming the release of the Sx session.

6.	The relevant steps of the procedure as specified in the figure above are executed.

7a.	The PGW-C sends an Sx Session Termination Request to the PGW-U to release the Sx session.

7b.	The PGW-U sends an Sx Session Termination Response to the PGW-C confirming the release of the Sx session.

8.	The relevant steps of the procedure as specified in the figure above are executed.

* * * Fourth change * * * *

##### 6.3.4.1.2	Procedures with SGSN terminating Gn/Gp interface

This clause defines interactions between the CP and UP function during the following procedures related to the deactivation of a PDN connection:

Group 1:

-	Combined GPRS / IMSI Attach procedure.

-	MS-Initiated Combined GPRS / IMSI Detach Procedure.

-	SGSN-Initiated GPRS Detach Procedure.

-	HLR-Initiated GPRS Detach Procedure.

-	MS Initiated PDP Context Deactivation Procedure for A/Gb mode (with Teardown Ind).

-	MS Initiated PDP Context Deactivation Procedure for Iu mode (with Teardown Ind).

-	SGSN-initiated PDP Context Deactivation Procedure (with Teardown Ind).

Group 2:

-	GGSN-initiated PDP Context Deactivation Procedure (with Teardown Ind).

During the above procedures, following is the nature of interactions between the CP and UP function:

-	The PGW-C releases the existing Sx session from PGW-U.

Figure 6.3.4.1.2-1: Interaction between CP and UP function with PDN connection deactivation Group 1

1.	Procedure as listed in this step is initiated as specified in the relevant clauses of this specification. The relevant steps of the procedure as specified in the figure above are executed.

2a.	The PGW-C may send an Sx Session Termination Request to the PGW-U to release the Sx session.

2b.	The PGW-U sends an Sx Session Termination Response to the PGW-C confirming the release of the Sx session.

3.	The relevant steps of the procedure as specified in the figure above are executed.

4a.	The PGW-C may send an Sx Session Termination Request to the PGW-U to release the Sx session.

4b.	The PGW-U sends an Sx Session Termination Response to the PGW-C confirming the release of the Sx session.

5.	The relevant steps of the procedure as specified in the figure above are executed.

Figure 6.3.4.1.2-2: Interaction between CP and UP function with PDN connection deactivation Group 2

1.	Procedure as listed in this step is initiated as specified in the relevant clauses of this specification. The relevant steps of the procedure as specified in the figure above are executed.

2a.	The PGW-C sends an Sx Session Modification Request to the PGW-U when it initiates bearer deactivation procedure. PGW-C shall indicate PGW-U to stop counting downlink packets for the affected bearers. PGW-C shall also indicate the PGW-U to discard downlink packets received for the affected bearers.

2b.	The PGW-U sends an Sx Session Modification Response to the PGW-C.

3.	The relevant steps of the procedure as specified in the figure above are executed.

4a.	The PGW-C sends an Sx Session Termination Request to the PGW-U to release the Sx session.

4b.	The PGW-U sends an Sx Session Termination Response to the PGW-C confirming the release of the Sx session.

5.	The relevant steps of the procedure as specified in the figure above are executed.

* * * End of changes * * * *