3GPP TSG-SA WG2 Meeting #140e 	S2- 2004959

Electronic meeting, 2020‑08‑19 -- 2020‑09‑02

Source:	Ericsson

Title:KI#3	Way forward for KI#3

Document for:	Agreement

Agenda Item: 	8.3

Work Item / Release:	FS_enh_EC /Rel-17

Abstract of the contribution: This contribution proposes solutions41, 43, 48 and 49 as the way forward for KI#3

# 1	Introduction

This DP discusses the way forward for KI#3

# 2	Discussion

## 2.1	KI description

The following is an excerpt from the KI#3 description in 23.748:

With edge computing deployment, it is expected that a set of edge computing functions or edge application servers running on edge hosting environment will need to interact with the 5GS to access to 5GS functionality and information, and/or to provide information to 5GS for the provisioning of connectivity services supporting edge computing. The interaction for exposure of network information between the 5GS and the edge computing functions need to be studied.

As part of the study, latency of network exposure needs to be considered. Current network exposure mechanism in 5GS is designed based on NEF and other control plane NFs, e.g. AMF, SMF, PCF etc. For applications deployed in edge hosing environments, the Edge Application Servers or Application Functions may be locally deployed, but in current Rel-16, some Control Plane NFs involved in network exposure, e.g. NEF and PCF, are likely deployed centrally to avoid frequently relocation. This may result in a less than efficient network exposure path in terms of latency.

For some network information to be exposed, the long exposure latency is tolerable. However, some real time network information, e.g. network congestion condition or real-time user path latency, can change very frequently. If this information needs to be delivered to Application servers or Application Functions timely, undesirable latency may make the information obsolete cause applications to adjust their behaviour (e.g. adjust the resolution of video stream, or switch levels of driving automation) based on out-of-date network information.

Examples of existing QoS information that may need to be exchanged quickly between network and Application Functions (e.g. Edge Application Servers) include:

1.	The AF may subscribe to receive QoS congestion condition notifications.

2.	The AF may request 5GC to monitor QoS status (e.g. over-the-air and/or end-to-end data path) and receive QoS measurement reports.

This key issue addresses exposure of information to Application Functions deployed in the edge (e.g. Edge Application Servers), including:

-	Which information that have already existed in Rel-16 needs to be exposed with low latency to the edge computing functions by the 5GS?

-	How does the 5GC determine whether a network information need to be exposed with low latency?

-	How to expose the network information to the application functions deployed in the edge with low latency?

-	Whether and how to maintain the exposure when the UE moves out of the coverage of NF(s) supporting the exposure?

NOTE 1:	This study does not discuss solution related to NG-RAN exposing information to edge computing functions or servers via direct reference points.

NOTE 2:	This study does not disclose RAN private data (e.g. RAN details) to the edge computing functions or servers.

## 2.2	Solutions

| Tdoc | Title | Contributer | Solution # |
| --- | --- | --- | --- |
| S2-2004438 | Network Information Provisioning using the IP path. | Ericsson | 41 |
| S2-2004426 | Providing selected radio information to an App requiring it. | Nokia, Nokia Shanghai Bell | 42 |
| S2-2004427 | Low Latency exposure API by using the distributed CAPIF framework feature | Sony | 43 |
| S2-2004428 | Network Information Exposure to Local AF with Low Latency. | Tencent | 44 |
| S2-2004429 | Using AS or NAS message notify UE's application layer | OPPO | 45 |
| S2-2003771 | Local NEF Deployment for network information exposure to Local AF with Low Latency. | China Mobile,AT&T | 46 |
| S2-2004430 | User Plane based Network Information Provisioning . | Tencent | 47 |
| S2-2004431 | QoS monitoring information exposure based on unstructured data transmission mechanism. | CATT | 48 |
| S2-2004432 | Network Information Provisioning to EAS with low latency based on User Plane | Huawei, HiSilicon | 49 |

### 2.2.1	Solution #41: Network Information Provisioning using the IP path

Origin of information: queue length in RAN

Exposure of information: in band in IP, using ECN, and feedback loops in IP transport layer or RTP

The solution uses ECN (Explicit Congestion Notification) in IP. RAN supports ECN according to 38.300.

### 2.2.2	Solution #42: Providing selected radio information to an App requiring it

Origin of information: RAN as an MnS provider

Exposure of information: AF request and reporting via to NEF locally deployed

### 2.2.3	Solution #43: Low latency exposure API solution.

Origin of information: UPF, what information that can be provided is unclear

Exposure of information: AF request via CAPIF (NEF), reporting directly from UPF to AF

The solution uses the CAPIF framework. AF requests QoS information from 5GC and reporting comes directly from UPF.

### 2.2.4	Solution #44: Network Information Exposure to Local AF with Low Latency

Origin of information: RAN as an MnS producer

Exposure of information: AF request and reporting via to NEF locally deployed

Solution uses a locally deployed NEF acting as a MnS consumer. MnS producer seem to be RAN. MnS (Management Service) offers capabilities for management and orchestration of network and service.

### 2.2.5	Solution #45: Using AS or NAS message notify UE’s application layer

Origin of information: RAN using notification control

Exposure of information: Via NAS to UE application and to application server via application signalling

The solution uses Notification control send from RAN to SMF via SMF if required QoS cannot be fulfilled. SMF modifies the QoS flow and by this the UE is informed. The UE can inform AS if needed via the e2e application specific signalling.

### 2.2.6	Solution #46: Local NEF Deployment for network information exposure to Local AF with Low Latency.

Origin of information: UPF, what information that can be provided is unclear

Exposure of information: AF request (subscribes) to locally deployed NEF. NEF reports to AF. NEF has direct interface to UPF.

The solution uses event subscription via NEF directly to UPF. UPF reports to AF via NEF. UPF is found by via PCF and SMF.

### 2.2.7 	Solution #47: User Plane based Network Information Provisioning

Origin of information: UPF, what information that can be provided is unclear

Exposure of information: Requests and reports via option field in header of TCP or IP, or data field ICMP ECHO

The solution uses in band requests from AS to UPF. Reports of events is sent in band from UPF to AS. The information in requests and reports are transported in the IP or TCP header’s options field or in ICMP ECHO data

### 2.2.8 	Solution #48: QoS monitoring information exposure based on unstructured data transmission mechanism

Origin of information: QoS monitoring in UPF

Exposure of information: AF request to PCF. Reports provided by UPF via N6 tunnelling to AF

The solution uses QoS monitoring (23.501 clause 5.33.3.2) to get QoS situation. The AF requests QoS monitoring from PCF. PCF request SMF which request UPF to monitor and send result via unstructured N6 tunnel to the AF.

### 2.2.9 	Solution #49: Network Information Provisioning to EAS with low latency

Origin of information: QoS monitoring in UPF

Exposure of information: Request from AF to NEF. Reporting from UPF to locally deployed NEF.

The solution uses QoS monitoring (23.501 clause 5.33.3.2) to get QoS situation. The AF would subscribe to QoS monitoring to NEF. NEF indicates monitoring required to PCF. PCF request SMF which request UPF to monitor and send result via N4 to local NEF.

## 2.3	Analysis

There are 4 general question to be solved in this KI#3:

1)	Which information that have already existed in Rel-16 needs to be exposed with low latency to the edge computing functions by the 5GS?

2)	How does the 5GC determine whether a network information need to be exposed with low latency?

3)	How to expose the network information to the application functions deployed in the edge with low latency?

4)	Whether and how to maintain the exposure when the UE moves out of the coverage of NF(s) supporting the exposure?

For 1), 5GS can currently generate QoS information in 3 ways:

a)	Using notification control(23.501 clause, 5.7.2.4). Here RAN informs SMF if QoS parameter(s) (GFBR, PDB, PER) for a GBR QoS flow cannot be fulfilled. Along with a request for notification control, SMF can send alternative QoS profiles to RAN with a priority order, and RAN will indicate which one of these it can fulfil if the requested QoS profile cannot be fulfilled. SMF may indicate to PCF what SMF gets from RAN. SMF may also indicate to UE the new situation using NAS signalling.

b)	QoS monitoring (23.501 clause. Here PCF requests SMF to do QoS monitoring. SMF initiate QoS monitoring towards UPF. UPF and RAN cooperate to measure the latency of a QoS flow. Both GBR and non-GBR QoS flows may be monitored. Only latency can be measured.

c)	AQM (Active Queue Management) in the schedular in RAN either by dropping packets or using Explicit Congestion Control (38.300 clause 12.2 and RFC 3168). When indication comes, the receiver of the ECN/Packet drop will need to feedback the information to the sender. Both GBR and non-GBR are supported. Only congestion is can be indicated, which implicitly indicates latency.

Since RAN is the best source of getting information related to the dynamic QoS situation, i.e. information needed for KI#3, solution for KI#3 can only use what currently are available provided by RAN, which are the 3 ways described above. Any other method is disqualified since it will have RAN impact.

a) and b) requires active requests/subscription by and AF, whereas c) does not.

Solution 42 and 43 uses the management and orchestration framework. It is not clear how these solutions can fulfil the question 2) regarding low latency. It is not clear why this would be faster than getting information via the control plane NFs as of a) or b). Neither is it clear that wanted information can be provided without RAN impacts, i.e. it has not been proven that any specification from SA5 provides this. Therefor it is proposed that not go further with solutions 42 and 43.

Solution 45 uses notification control (i.e. method a)) and then if QoS changes UE is informed via NAS, and UE relays to AS via application specific means. It is not clear how this can be low latency and why this would be better than for an AF to use Npcf services for this information from PCF. Therefore, it is proposed not to go further with solution 45.

47 use in band information in IP. 47 assumes that UPF can provide QoS information, but it does not state how UPF will obtain the QoS information. Furthermore, it introduces a new “protocol” transported in IP/TCP headers or ICMP ECHO, but the questions are what better information than ECN such a protocol would provide and what impact it may have on OS layer in servers and UE? Therefore, it is proposed not to go further with solution 47

Solutions 43 and 46 states that UPF generates the QoS information, but it is not clear how UPF can get this information, and thus cannot be a solution question 1)

Solutions 48 and 49 uses QoS monitoring (method b)) in 5GC to answer question 1). It is proposed that QoS monitoring is the bases for answering question 1) for the case where and AF uses exposure interface for this information, and thus bases for normative phase (for question 1).

Solution 43, 46, 48, and 49 proposed some direct reporting from UPF to the AF/AS perhaps via NEF. Thus, they may fulfil the latency requirement of reporting required by the KI #3, thus question 3) may be answered. Solutions 43, 48 and 49 are similar in their way of exposure, both for requests/subscription of QoS info (latency) and for reporting. However, exactly which protocol to be use for reporting needs to be carefully evaluated so that the latency requirements can be fulfilled. It is proposed that 43, 48 and 49 are used as bases for further work for how to expose QoS information.

Of solution 43, 48 and 49, only 49 has an embryo to solve question 4. It is proposed that further work is needed to solve question 4

Solution 41 solves all questions 1), 2) 3) and 4). It does not have any impact on the 5GC system and RAN supports ECN. It is proposed that it is documented in the normative phase.

# 3	Proposal

Text to be added to 23.748

**********************  START CHANGES   *********************

# 7	Overall Evaluation

Editor's note:	This clause will provide evaluation of different solutions.

## 7.x	Interim conclusion of KI #3

## 7.x.1	Analysis

There are 4 general question to be solved in this KI#3:

1)	Which information that have already existed in Rel-16 needs to be exposed with low latency to the edge computing functions by the 5GS?

2)	How does the 5GC determine whether a network information need to be exposed with low latency?

3)	How to expose the network information to the application functions deployed in the edge with low latency?

4)	Whether and how to maintain the exposure when the UE moves out of the coverage of NF(s) supporting the exposure?

For 1), 5GS can currently generate QoS information in 3 ways:

a)	Using notification control (TS 23.501 [2], clause 5.7.2.4). Here RAN informs SMF if QoS parameter(s) (GFBR, PDB, PER) for a GBR QoS flow cannot be fulfilled. Along with a request for notification control, SMF can send alternative QoS profiles to RAN with a priority order, and RAN will indicate which one of these it can fulfil if the requested QoS profile cannot be fulfilled. SMF may indicate to PCF what SMF gets from RAN. SMF may also indicate to UE the new situation using NAS signalling.

b)	QoS monitoring (23.501 clause. Here PCF requests SMF to do QoS monitoring. SMF initiate QoS monitoring towards UPF. UPF and RAN cooperate to measure the latency of a QoS flow. Both GBR and non-GBR QoS flows may be monitored. Only latency can be measured.

c)	AQM (Active Queue Management) in the schedular in RAN either by dropping packets or using Explicit Congestion Control (TS 38.300[16] clause 12.2 and RFC 3168). When indication comes, the receiver of the ECN/Packet drop will need to feedback the information to the sender. Both GBR and non-GBR are supported. Only congestion is can be indicated, which implicitly indicates latency.

Since RAN is the best source of getting information related to the dynamic QoS situation, i.e. information needed for KI#3, solution for KI#3 can only use what currently are available provided by RAN, which are the 3 ways described above. Any other method is disqualified since it will have RAN impact.

a) and b) requires active requests/subscription by and AF, whereas c) does not.

Solution 42 and 43 uses the management and orchestration framework. It is not clear how these solutions can fulfil the question 2) regarding low latency. It is not clear why this would be faster than getting information via the control plane NFs as of a) or b). Neither is it clear that wanted information can be provided without RAN impacts, i.e. it has not been proven that any specification from SA5 provides this. Therefor it is proposed that not go further with solutions 42 and 43.

Solution 45 uses notification control (i.e. method a)) and then if QoS changes UE is informed via NAS, and UE relays to AS via application specific means. It is not clear how this can be low latency and why this would be better than for an AF to use Npcf services for this information from PCF. Therefore, it is proposed not to go further with solution 45.

47 use in band information in IP. 47 assumes that UPF can provide QoS information, but it does not state how UPF will obtain the QoS information. Furthermore, it introduces a new “protocol” transported in IP/TCP headers or ICMP ECHO, but the questions are what better information than ECN such a protocol would provide and what impact it may have on OS layer in servers and UE? Therefore, it is proposed not to go further with solution 47

Solutions 43 and 46 states that UPF generates the QoS information, but it is not clear how UPF can get this information, and thus cannot be a solution question 1)

Solutions 48 and 49 uses QoS monitoring (method b)) in 5GC to answer question 1). It is proposed that QoS monitoring is the bases for answering question 1) for the case where and AF uses exposure interface for this information, and thus bases for normative phase (for question 1).

Solution 43, 46, 48, and 49 proposed some direct reporting from UPF to the AF/AS perhaps via NEF. Thus, they may fulfil the latency requirement of reporting required by the KI #3, thus question 3) may be answered. Solutions 43, 48 and 49 are similar in their way of exposure, both for requests/subscription of QoS info (latency) and for reporting. However, exactly which protocol to be use for reporting needs to be carefully evaluated so that the latency requirements can be fulfilled. It is proposed that 43, 48 and 49 are used as bases for further work for how to expose QoS information.

NOTE:	Which protocol to use is a stage 3 issue

Of solution 43, 48 and 49, only 49 has an embryo to solve question 4. It is proposed that further work is needed to solve question 4

Solution 41 solves all questions 1), 2) 3) and 4). It does not have any impact on the 5GC system and RAN supports ECN. It is proposed that it is documented in the normative phase

### 7.x.2	Way forward

The proposed way forward is that solutions 41, 43, 48 and 49 are the base to solve KI#3.

Solution 41 does not have any impact on current 5GC and RAN and should be documented in normative phase.

Solutions 43, 48, and 49 are used as bases for exposure and QoS monitoring as described in solutions 48 and 49 are used as the means of providing latency information.

Further work is needed to solve: “how to maintain the exposure when the UE moves out of the coverage of NF(s) supporting the exposure”

**********************  NEXT CHANGES   *********************

# 9	Conclusions

## 9.x	Conclusions regarding solutions for Key Issue #3

The proposed way forward is that solutions 41, 43, 48 and 49 are the base to solve KI#3.

Solution 41 does not have any impact on current 5GC and RAN and should be documented in normative phase.

Solutions 43, 48, and 49 are used as bases for exposure and QoS monitoring as described in solutions 48 and 49 are used as the means of providing latency information.

Further work is needed to solve: “how to maintain the exposure when the UE moves out of the coverage of NF(s) supporting the exposure”

**********************  NEXT CHANGES   *********************

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System".

[3]	3GPP TS 23.502: "Procedures for the 5G System".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[5]	3GPP TR 23.758: "Study on application architecture for enabling Edge Applications".

[6]	3GPP TS 22.261: "Service requirements for the 5G system".

[7]	IETF RFC 7871: "Client Subnet in DNS Queries".

[8]	IETF RFC 1546: "Host Anycasting Service".

[9]	IETF RFC 1034: "Domain Names - implementation and specification".

[10]	IETF RFC 7858: "Specification for DNS over Transport Layer Security (TLS)".

[11]	IETF RFC 8484: "DNS Queries over HTTPS (DoH)".

[12]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

[13]	IETF RFC 792: "INTERNET CONTROL MESSAGE PROTOCOL".

[14]	IETF RFC 7157: "IPv6 Multihoming without Network Address Translation".

[15]	IETF RFC 1122: "Requirements for Internet Hosts - Communication Layer".

[16]	3GPP TS 38.300: "NR; Overall description; Stage-2".

[17]	IETF RFC 8311: "Relaxing Restrictions on Explicit Congestion Notification (ECN) Experimentation".

[18]	IETF draft draft-ietf-tsvwg-l4s-arch-06: " Low Latency, Low Loss, Scalable Throughput (L4S) Internet Service: Architecture".

[19]	IETF draft draft-ietf-tsvwg-ecn-l4s-id-10: "Identifying Modified Explicit Congestion Notification (ECN) Semantics for Ultra-Low Queuing Delay (L4S)".

[20]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs".

[21]	3GPP TS 33.122: "Security aspects of Common API Framework (CAPIF) for 3GPP northbound APIs".

[22]	3GPP TS 32.421: "Subscriber and equipment trace; Trace concepts and requirements".

[23]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[24]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[25]	IETF RFC 7556: "Multiple Provisioning Domain Architecture".

[26]	IETF RFC 6731: "Improved Recursive DNS Server Selection for Multi-Interfaced Nodes".

[27]	IETF RFC 8684: "TCP Extensions for Multipath Operation with Multiple Addresses".

[28]	IETF RFC 7681: "Email Exchange of Secondary School Transcripts".

[29]	IETF RFC 4191: "Default Router Preferences and More-Specific Routes".

[30]	3GPP TS 29.503: "5G System; Unified Data Management Services; Stage 3".

[30]	3GPP TS 29.244: "Interface between the Control Plane and the User Plane nodes".

[xx]	IETF RFC 3168: The Addition of Explicit Congestion Notification (ECN) to IP

**********************  END CHANGES   *********************