SA WG2 Meeting #140E e-meeting	S2-2004987

Aug 19 – Sep 01, 2020, Elbonia

Title:	[draft] LS on new PQI support for PC5 communication

Response to:	-

Release:	Rel-17

Work Item:	FS_5G_ProSe

Source:	[OPPO]

To:	RAN1

Cc:	RAN2

Contact person:	Jianhua Liu

liujianhua@oppo.com

Send any reply LS to: 3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

SA2 is studying the PC5 direct communication on the SID of ProSe, which is covering Public Safety and commercial cases . SA2 has assumption the existing V2X mechanism is reused, on top of that, SA tends to introduce the following new PQIs for Public Safety and commercial in Table 1 and Table 2. SA2 kindly ask RAN WG whether these new PQIs can be supported in AS layers.

Table 1: New PQIs for Public Safety

| PQI Value | Resource Type | Default Priority Level | Packet Delay Budget | Packet Error Rate | Default Maximum Data Burst Volume | Default Averaging Window | Example Services |
| --- | --- | --- | --- | --- | --- | --- | --- |
| New value#1 | GBR | 1 | 150 ms | 10-2 | N/A | 2000 ms | Mission Critical user plane Push To Talk voice (e.g. MCPTT) |
| New value#2 |  | 2 | 200 ms | 10-2 | N/A | 2000 ms | Non-Mission-Critical user plane Push To Talk voice |
| New value#3 |  | 2 | 200 ms | 10-3 | N/A | 2000 ms | Mission Critical Video user plane |
| New value#4 | Non-GBR | 1 | 120 ms | 10-6 | N/A | N/A | Mission Critical delay sensitive signalling (e.g. MC-PTT signalling) |
| New value#5 |  | 6 | 400 ms | 10-6 | N/A | N/A | Mission Critical Data (e.g. example services are the same as 5QI 6/8/9) |

Table 2: New PQIs for commercial service

| PQI Value | Resource Type | Default Priority Level | Packet Delay Budget | Packet Error Rate | Default Maximum Data Burst Volume | Default Averaging Window | Example Services |
| --- | --- | --- | --- | --- | --- | --- | --- |
| New value#1 | Delay Critical GBR | 5 | 5ms | 10-4 | 2000 bytes | 2000 ms | Interactive service - consume VR content with high compression rate via tethered VR headset (100Mbps) |
| New value#2 |  | 6 | 10ms | 10-4 | 20000bytes | 2000 ms | interactive service - consume VR content with low compression rate via tethered VR headset (10Gbps) |

# 2	Actions

To RAN1, RAN2

ACTION: SA2 kindly ask RAN WG1 whether these new PQIs can be supported in AS layers.

# 3	Dates of next TSG SA WG 2 meetings

SA WG2 Meeting #141E 	12th – 23rd October 2020