3GPP TSG-WG SA2 Meeting #140E	S2-2005012

Elbonia, Aug 19 – Sep 01, 2020	(revision of S2-20xxxx)

Source:	Qualcomm Incorperation

Title:	Update of solution #6: NWDAF decompose architecture

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_Ph2 / Rel-17

Abstract: This contribution proposes an update to solution#6.

# 1. Introduction

This contribution proposes an update to solution#6: NWDAF decompose architecture.

# 2. Text Proposal

It is proposed to capture the following update in TR 23.700-91.

* * * * Start of Changes * * * *

## 6.6	Solution #6: NWDAF decompose architecture

### 6.6.1	Description

#### 6.6.1.1	General

This solution addresses Key Issue#1: Logical decomposition of NWDAF and possible interactions between logical functions.

#### 6.6.1.2	Architecture

The NWDAF is composed by the following functions:

1).	ML training function: used to train the model;

2).	Analytics function: used to do the data analytics for the consumer and expose the analytics result to the consumer;

3). NWDAF Data Repository: the data storage entities that stores the data collected by NWDAF from other 5G NFs.

NOTE 1:	The NWDAF data repository is addressed by the Key issue 11: Increasing efficiency of data collection.

#### 6.6.1.3	Initial ML Model Training

The ML training function is used for initial ML model training, the initial ML model can be provided by a ML designer or local configuration. The ML designer is a designer environment along with ML toolkit to create the initial ML model which can be operated either by a 3rd party, or by the operator.

NOTE 2:	The ML designer function is out of 3GPP scope, how the ML designer provides the initial ML model to ML training function is out of 3GPP scope.

NWDAF Data Repository provides training data to the ML training function for the initial model training.

#### 6.6.1.4	Data analytics

The successful trained ML models are published to the Analytics function with the related analytics IDs, the Analytics Function can register the supported analytics IDs to NRF. If any analytics request / subscribe is received from the consumer, the Analytics function will collect the data from NWDAF Data Repository or trigger the NWDAF Data Repository to collect data from other entites, performs the data analytics according to the supported ML model and exposure the output to the consumer.

NOTE 3:	The Analytics request / subscribe procedure is initiated from consumer to the Analytics function is already supported in Rel-16. The data collection from NWDAF Data Repository is related to Key issue 11: Increasing efficiency of data collection.

There are two options about how the Analytics function receives the published trained ML models.

Option 1: ML model published via ML designer

Figure 6.6.1.4-1 proposed a NWDAF decompose architecture in which the ML designer publishs the trained ML model to the Analytics Function.

Figure 6.6.1.4-1: ML model publishes the trained ML model to the Analytics Function

Step 1 and 2 in Figure 6.6.1.4-1 refer to the initial ML model training described in clause 6.6.1.3. After the initial ML model training success, the ML training function sends the trained model to the ML designer and the ML designer publishes the ML model with the corresponding Analytics ID to the Analytics Function. ML designer may publish the trained model to different Analytics function s.

NOTE 4:	How ML designer sends the trained ML model to Analytics functions is to addressed by Key Issue #19: Trained data model sharing between multiple NWDAF instances.

Option 2: ML model published by ML training function

Step 1 and step 2 in Figure 6.6.1.4-2 refer to the initial ML model training described in clause 6.6.1.3 (same as option 1). After initial ML model training success, the ML training function publishes the trained ML model and the corresponding Analytics ID to Analytics function. The trained ML model may publish the trained model to different Analytics functions.

NOTE 5:	How ML training function sends the trained ML model to Analytics functions is to addressed by Key Issue #19: Trained data model sharing between multiple NWDAF instances.

Figure 6.6.1.4-2: ML training function  publishes ML model

### 6.6.2	Procedures

#### 6.6.2.1	Initial ML Model Training

Figure 6.6.2.1-1 is the call flow of Initial ML Model Training.

Figure 6.6.2.1-1: Initial ML Model Training call flow

0. The initial ML model maybe received from ML Designer or based on local configuration. This step is out of 3GPP scope.

1. ML Training Function sends the Initial ML model training data collection request (Analytics ID) to the NWDAF Data Repository. The Analytics ID identify the Initial ML model.

2. NWDAF Data Repository may trigger the data collection from other entities and provides all the input data that related to the requested Analytics ID to ML Training Function.

3. ML Training Function performs the initial model training.

#### 6.6.2.2	ML model publish procedure

For option 1 ML training function sends the trained ML model to ML designer, and ML designer publishes the ML model to different Analytics Function. The interaction between ML training function and ML designer and ML designer to Analytics Function are out of 3GPP scope.

For option 2, a standard interface between ML Training Function and Analytics Function are supported for ML model publish procedure.

After the ML model training, the ML Training Function can register the supported ML Model to the NRF. Analytics Function may query the ML Training function for a specific ML model from NRF.

Figure 6.6.2.2-1 ML model publish procedure

The ML model publish procedure may be triggered either by operator configuration in ML Training Function or request from Analytics Function(s).

1a. Analytics Function sends the ML Model Analytics (ML Model ID, Analytics ID) to ML Traninig Function.

1. ML Training Function publishes the trained ML model to Analytics Function(s).

2. The Analytics Function(s) send the ACK to ML training Function.

NOTE 6:	The parameters in step 1 and 1a are related to Key Issue #19: Trained data model sharing between multiple NWDAF instances.

### 6.6.3	Impacts on services, entities and interfaces

NWDAF

-	Decompose NWDAF into different functions

* * * * End of Changes * * * *