3GPP TSG-SA2 Meeting #140-E	S2-2005066

Elbonia, August 19 – September 1, 2020

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0177 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Service experience analytics discrimination |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA |  |  |  |  |  |  | Date: |  | 2020-08-13 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Current specification does not allow to discriminate service analytics subscriptions/requests for an Application and a Network Slice, but service consumers may be interested in only one of the two types of service experience analytics, or both. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The CR proposes several changes to allow NWDAF to determine which of the two analytics types is being requested/subscribed to. It also introduces changes to the output analytics tables for service experience to allow for such differentiaton. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is no way for NWDAF to discriminate between service experience analytics for an Application and for a Network Slice. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.1, 6.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of Changes * * * *

### 6.4.1	General

This clause specifies how NWDAF can provide Observed Service Experience (i.e. average of observed Service MoS and/or variance of observed Service MoS indicating service MOS distribution for services such as audio-visual streaming as well as services that are not audio-visual streaming such as V2X and Web Browsing services) analytics, in the form of statistics or predictions, to a service consumer.

The Observed Service Experience analytics may provide one or both of the following:

-	Service Experience for a Network Slice: Service Experience for a group of UEs or any UE for a given Application or a set of Applications or any Application (i.e. all Applications) in a Network Slice;

-	Service Experience for an Application: Service Experience for a UE or a group of UEs or any UE in an Application or a set of Applications.

Therefore, Observed Service experience may be provided individually per UE or group of UEs, or globally, averaged per Application or averaged across a set of Applications on a Network Slice.

The service consumer may be an NF (e.g. PCF), or the OAM.

The consumer of these analytics shall indicate in the request or subscription:

-	Analytics Id set to "Service Experience";

-	The Target of Analytics Reporting: one or more SUPI(s) or Internal Group Identifier(s), or "any UE";

-	Analytics Filter Information as defined in Table 6.4.1-1; and

-	optionally, maximum number of objects and maximum number of SUPIs;

Table 6.4.1-1: Analytics Filter Information related to the observed service experience

| Information |  | Description | Mandatory |  |
| --- | --- | --- | --- | --- |
|  |  |  | App | NS |
| Application ID (0...max) (NOTE 1) |  | The identification of the application(s) for which the analytics information is subscribed or requested. | Y | N |
| S-NSSAI |  | Identifies the Network Slice for which analytics information is subscribed or requested. | N | Y |
| NSI ID(s) |  | Identifies the Network Slice instance(s) for which analytics information is subscribed or requested. | N | N |
| Area of Interest (NOTE 2) |  | Identifies the Area (i.e. set of TAIs), as defined in TS 23.501 [2] where the analytics information is subscribed or requested. | Y | Y |
| Media/application bandwidth |  | Identifies the Media/application bandwidth requirement of the application. | N | N |
| DNN |  | DNN to access the application. | N | N |
| DNAI | Identifier of a user plane access to one or more DN(s) where applications are deployed as defined in TS 23.501 [2] |  | N | N |

NOTE 1:	If no Application ID is provided, the rest of Analytics Filter information applies to any application (i.e. all applications) in the Network Slice.

NOTE 2:	Area of Interest is not mandatory for Application service experience if Target of Analytics Reporting is not set to "any UE".

-	An Analytics target period that indicates the time window for which the statistics or predictions are requested;

-	In a subscription, the Notification Correlation Id and the Notification Target Address.

The NWDAF shall notify the result of the analytics to the consumer as specified in clause 6.4.3.

NWDAF collects the network data from AF (directly or via NEF) and from other 5GC NF(s) in order to calculate and provide statistics and predictions on the observed service experience to a consumer NF or to OAM.

Based on the Analytics Filter information and the Target of Analytics reporting provided by the service consumer in the analytics subscription or request, NWDAF determines whether service experience analytics should be delivered for i) Application(s), ii) Network Slice, iii) both Application(s) and Network Slice.

If service experience for both Application(s) and Network Slice is desired but the Target of Analytics reporting or Analytics Filter information values (e.g. Area of Interest) need to be different, separate subscriptions/requests may be provided by the service consumer.

* * * * Next Change * * * *

### 6.4.3	Output Analytics

The NWDAF services as defined in the clause 7.2 and 7.3 are used to expose the analytics.

-	Service Experience statistics information is defined in Table 6.4.3-1.

-	Service Experience predictions information is defined in Table 6.4.3-2.

Table 6.4.3-1: Service Experience statistics

| Information | Description |
| --- | --- |
| S-NSSAI | Identifies the Network Slice for which analytics information is provided. |
| Slice instance service experiences (0…max) | List of observed service experience information for each Network Slice instance. |
| > NSI ID | Identifies the Network Slice instance within the Network Slice. |
| > Network Slice instance service experience | Service experience across Applications on a Network Slice instance over the Analytics target period (average, variance). |
| > SUPI list (0..SUPImax) | List of SUPI(s) for each slice instance service experience. |
| > Ratio | Estimated percentage of UEs with similar service experience (in the group, or among all UEs). |
| > Spatial validity | Area where the Network Slice service experience analytics applies. |
| > Validity period | Validity period for the Network Slice service experience analytics as defined in clause 6.1.3. |
| Application service experiences (0..max) | List of observed service experience information for each Application. |
| > S-NSSAI | Identifies the Network Slice of the Application |
| > Application ID | Identification of the Application. |
| > Service Experience Type | Type of Service Experience analytics, e.g. on voice, video, other. |
| > Service Experience | Service Experience over the Analytics target period (average, variance). |
| > SUPI list (0..SUPImax) | List of SUPI(s) for each application service experience. |
| > Ratio | Estimated percentage of UEs with similar service experience (in the group, or among all UEs). |
| > Spatial validity | Area where the Application service experience analytics applies. |
| > Validity period | Validity period for the Application service experience analytics as defined in clause 6.1.3. |

Table 6.4.3-2: Service Experience predictions

| Information | Description |
| --- | --- |
| S-NSSAI | Identifies the Network Slice for which analytics information is provided. |
| Network Slice instance service experiences (0…max) | List of observed service experience information for each Network Slice instance. |
| > NSI ID | Identifies the Network Slice instance within the Network Slice. |
| > Network Slice instance service experience | Service experience across Applications on a Network Slice instance over the Analytics target period (average, variance). |
| > SUPI list (0..SUPImax) | List of SUPI(s) for each slice instance service experience. |
| > Ratio | Estimated percentage of UEs with similar service experience (in the group, or among all UEs). |
| > Spatial validity | Area where the Network Slice service experience analytics applies. |
| > Validity period | Validity period for the Network Slice service experience analytics as defined in clause 6.1.3. |
| > Probability assertion | Confidence of this prediction. |
| Application service experiences (0..max) | List of predicted service experience information for each Application. |
| > S-NSSAI | Identifies the Network Slice of the Application |
| > Application ID | Identification of the Application. |
| > Service Experience Type | Type of Service Experience analytics, e.g. on voice, video, other. |
| > Service Experience | Service Experience over the Analytics target period (average, variance). |
| > SUPI list (0..SUPImax) | List of SUPI(s) for each application service experience. |
| > Ratio | Estimated percentage of UEs with similar service experience (in the group, or among all UEs). |
| > Spatial validity | Area where the Application service experience analytics applies. |
| > Validity period | Validity period for the Application service experience analytics as defined in clause 6.1.3. |
| > Probability assertion | Confidence of this prediction. |

NOTE 1:	Based on the service experience analytics discrimination, NWDAF may provide the output analytics entries in Tables 6.4.3-1 and 6.4.3-2 corresponding to Network Slice, Application, or both.

NOTE 2:	If multiple Network Slice instances are not deployed for the S-NSSAI or NSI IDs are not available, only one Network Slice instance service experience entry is provided. In that case, the NSI ID is not provided and the Network Slice instance service experience indicates the service experience for the S-NSSAI.

NOTE 3:	The SUPI list and Ratio in the service experience information for an application can be omitted, if the corresponding parameter(s) is/are provided and with the same value(s) in the service experience information for the slice instance which the application belongs to.

The number of Service Experiences and SUPIs are limited respectively by the maximum number of objects and the Maximum number of SUPIs provided as part of Analytics Reporting Information by the NWDAF Service Consumer.

* * * * End of Changes * * * *