SA WG2 Meeting #S2-140E	S2-2005127

19 Aug - 2 Sep, 2020, Elbonia

Source: 	Ericsson

Title: 	Evaluation and conclusion for KI#3

Document for: 	Approval

Agenda Item: 	8.4

Work Item / Release:	FS_eNS_Ph2 / Rel-17

Abstract of the contribution: This paper proposes the evaluation and conclusion for KI#3.

# Discussion

This is a proposal for interim evaluation and interim conclusions for KI#3 that will be a starting point and need to be improved when Editor´s Notes in the different solutions, possible new solutions and overall conclusions are reached.

The evaluation criteria are taken from the proposal from rapporteur during the conference call for eNS_ph2.

# Proposal

Add the following changes to TR 23.700-20.

*** BEGIN CHANGES ***

# 7	Evaluation

Editor's note:	This clause will provide a general evaluation of the solutions.

## 7.x	Key Issue #3: Limitation of data rate per network slice in UL and DL per UE

Editor's note:	This clause will provide some interim evaluation based on solutions #13, #20, #21, #22 that will need further updates to address e.g. roaming aspects.

The criteria for evaluation include analysis of the system architectural impacts, signalling overhead, administration and configuration overheads and backward compatibility.

System architectural impacts

- Solution#21 introduces a new NF controls the Slice-MBR value without impacting other NFs other than SMF that needs to contact NSQ per each PDU session establishment and modification in addition to the PCF, UDM, DN-AAA. The need to define a new NF to control QoS parameters is not justified, since QoS control is part of the PCF functionality.

- Solution #22 needs confirmation from RAN on the feasibility of the solution.

- Solution#20 addresses both KI#3 and KI#5.

- All solutions impact the existing NF services to a certain extent, so this is not considered as a differentiation factor.

Signalling overhead

- Solution #21 adds some additional signalling for each PDU session establishment, and new triggers for the PDU session modification.

- Solution#13, #20, #22 does not add new signalling to the PDU session establishment.

Administration and configuration overheads

- Solution#20 method 1 does not add any new requirements on policy data or subscription data. It requires an SLA in place on the Slice-MBR value for a UE.

- Solution #13, #21, #22 adds new requirements on subscription data to store the Slice-MBR.

The administration and configuration overheads are not a differentiator factor.

Backwards compatibility

- Solution #22 proposes to enforce the Slice-MBR value in RAN, this will cause wrong charging to the end user, since the DL traffic that is charged in the UPF may be later discarded in RAN, then the user will be charged for traffic that has never being delivered to the UE. This is considered to be a non-backward compatible solution since it cannot address charging properly.

- Solution #13, #20, #21 are backward compatible.

# 8	Conclusions

Editor's note:	This clause will capture conclusions for Key Issue #<X>.

## 8.x	Key Issue #3: Limitation of data rate per network slice in UL and DL per UE

Editor's note:	This clause will provide some interim conclusion based on solutions #13, #20, #21, #22 that will need further updates to address e.g. roaming aspects.

It is concluded not to progress solution #22 due to the wrong charging.

It is concluded not to define a new NF for Slice-MBR control due to the signalling overhead, and the NF overlap with the role of the PCF.

*** END CHANGES ***