SA WG2 Meeting #S2-140E	S2-2005150

19 August - 02 September 2020, Electronic, Elbonia	(revision of S2-200xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2351 | rev | - | Current version: | 16.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Modify the CN tunnel allocation step for SR procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ETSUN |  |  |  |  |  | Date: |  |  | 2020-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In TS23.502 clause 4.23.12.8.2 UE Triggered Service Request with I-SMF insertion/change/removal ,the CN Tunnel Info for EPS bearer in the PGW-U+UPF is provided to I-SMF in Nsmf_PDUSession_Update Response in step 20c. However based on stage 3 TS 29.502g40, “epsBearerInfo” is not included in the PduSessionUpdatedData. In stead the “epsBearerInfo” is included in the PduSessionCreatedData and VsmfUpdateData which are sent from PGW-C+SMF to I-SMF. There are two options to resolve this issue. Option 1) Add epsBearerInfo in PduSessionUpdatedData in stage3. Option 2) the PGW-C+SMF provides the CN Tunnel Info for EPS bearer in the PGW-U+UPF to I-SMF in step 8c Nsmf_PDUSession_Create Response Considering a)the R16 stage 3 has been frozen and b)alignment with handover procedures in clause 4.23.12.8 and clause 4.23.12.8.6 in which the CN Tunnel Info for EPS bearer is sent in the Nsmf_PDUSession_Create Response message, it is proposed to adopt optin 2). |  |  |  |  |  |  |  |  |
|  |  | . |  |  |  |  |  |  |  |  |
| Summary of change: |  | In the UE Triggered Service Request with I-SMF insertion/change/removal procedure, the PGW-C+SMF provides the CN Tunnel Info for EPS bearer in Nsmf_PDUSession_Create Response in step 8c. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Mis-alignment with stage 3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.23.12.8.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

4.23.12.8.2	UE Triggered Service Request with I-SMF insertion/change/removal

The following impact is applicable for UE triggered Service Request with I-SMF insertion in clause 4.23.4.3:

-	In step 8a, when an I-SMF is inserted, if EBI(s) have been allocated before but the PGW-C+SMF has not prepared the CN Tunnel Info for each EPS bearer, the PGW-C+SMF requests the PGW-U+UPF to allocate the CN Tunnel for each EPS bearer for PDU Session(s). PGW-U+UPF allocates the PGW-U tunnel info for the EPS bearer and sends it to the PGW-C+SMF.

-	In step 8c, the PGW-C+SMF provides also CN Tunnel Info for each EPS bearer to the I-SMF.

NOTE:	The CN Tunnel Info for each EPS bearer provided to the I-SMF is to prepare for UE mobility to EPC network so that the I-SMF does not need interact with the PGW-C+SMF to get the EPS bearer context(s) at mobility to EPC.

END OF CHANGES