3GPP TSG-WG SA2 Meeting #140E e-meeting 	S2-2005364

Elbonia, August 19 – September 01, 2020	(revision of S2-200xxxx)

Source:	Huawei, HiSilicon

Title:	KI#1: Update solution #22 to resolve ENs and support multi-homing PDU session

Document for:	Approval

Agenda Item:	8.3

Work Item / Release:	FS_enh_EC / Rel-17

Abstract: This contribution proposes to remove ENs of solution #22 and update solution#22 to support multi-homing PDU session.

# 1. Discussion

This pCR proposes the following changes:

- Addressing the following Editor’s Notes.

- Editor’s Note: It’s FFS whether the LDNSR can be integrated into SMF or UPF.

- Editor's note: This forwarding depends on the SA WG2 decision on how to support LDNSR (in SMF, in UPF, as a stand-alone entity).

3 options are available: Option 1: LDNSR as standalone NF; Option 2: LDNSR in SMF; Option 3: LDNSR in UPF.

On option 1: Solution #3 gives an example on standalone LDNSR (called DNS AF in solution 3) and interaction between LDNSR and network. The following disadvantages are observed:

  - Multiple interactions between LDNSR and BSF, PCF are needed for the LDNSR finding the appropriate SMF. In case LDNSR is out of 3GPP trusted domain, NEF is also needed in the procedure.

  - Some further enhancements (e.g. Token) may be used to avoid BSF, PCF or NEF are involved for each DNS query. However, this need to introduce UE-specific contexts (e.g. mapping between UE IP address and Token) on the LDNSR. This requests further mechanisms on LDNSR to decide how to maintain/release the UE contexts when UE IP address changes or released.

On Option 2: The following disadvantages are observed:

  - The volume of DNS query is huge (it can be about 0.1% of total traffics from UE in some typical scenarios). If LDNSR is located in SMF and UPF has no capability on handling DNS messages, then the LDNSR need to forward all of the DNS query to SMF then causes heavy load on N4 interfaces.

  - DNS server is usually visited via user plane path. Hence the SMF need to be configured with an user plane interface towards C-DNS/L-DNS servers. Although it may be possible to configure transport interface, it’ll put extra requirements on operator’s transport network.

Based on above discussion, it is proposed to integrate the LDNSR with UPF.

- Editor's note:	It's FFS whether both above options of DNS configuration on UE need to be supported.

Considering the requirement for encrypted DNS and the handling of LDNSR to redirect DNS query to L-DNS (see option 2a), it is proposed that the DNS query is terminated at the LDNSR, i.e., the determination address of DNS query is the address of LDNSR. Therefore, the LDNSR is configured as DNS server to the UE during PDU session establishment by SMF via PCO. With that, this Editor's note can be removed.

- Editor's note:	the text above assumes that the LDNSR keeps a context related with the PDU Session of the UE where it keeps track of the DNS requests sent by a UE in a PDU Session. It is FFS how this can be done when the LDNSR is not naturally associated with the PDU Session (in case the LDNSR would not be located in SMF or UPF). Even if the DNS queries are from same source IP, it may still be possible that the IP address has been reallocated to another UE.

According to the above discussion, it is proposed to integrate the LDNSR with UPF. Therefore, this Editor's note can be removed.

- Editor’s Note: Whether and how SMF indicates LDNSR whether to contact SMF based on e.g. UE location is FFS.

  - Currently, the procedure shows that the LDNSR query the SMF and get the ECS option or info for L-DNS selection after the LDNSR receives the DNS query. It is not optimized if for every DNS query, the query is required.

  - It is proposed to optimize the operation via the following mechanism:

    - For Option 1, the SMF provides LDNSR with IP address for Local DN that is accessible from the UE’s current location. The SMF may updates the IP address to LDNSR when UE moves to a new location. The LDNSR, when receiving the DNS query, adds the IP address into ECS option of the DNS message without asking SMF.

    - For Option 2, the SMF provides LDNSR with L-DNS address that is accessible from the UE’s current location. The SMF may updates the L-DNS address sent to LDNSR when UE moves to a new location. The LDNSR queries the L-DNS corresponding to the DNAI when it receives DNS query without asking SMF.

- Editor’s Note: The conditions for LDNSR to send EAS IP address after receiving DNS response to SMF is FFS.

  - Currently, there is no condition on when should the LDNSR send the DNS response to SMF

  - It is proposed to optimize the signalling as following:

    - The SMF provides rules to LDNSR to filter which DNS response need to be informed to SMF. In case LDNSR is  in UPF, the rules will be PDR:

      - The detection rule of the PDR request the LDNSR detect EAS IP address in DNS response message

      - URR is included in PDR, the URR indicates the UPF to report to SMF in case the EAS IP address matches with IP range associated with a Local DN for which the ULCL/L-PSA has not been inserted yet.

  - The solution reduces signalling required to report EAS IP address/FQDN to SMF.

- Adding the support of multi-homing PDU session.

In this solution, after the ULCL insertion, the 5GC can assist the EAS discovery by instructing the ULCL to steer the DNS query to the local DN, considering the IP address @remote PSA cannot represent the UE location.

After the BP insertion, a new IPv6 prefix @local PSA that is representative of UE location is assigned to UE using IPv6 Router Advertisement message. Optionally the SMF may indicate the UE to contact with DHCP server to get the DNS-related configuration information, which is used to select DNS server for a target FQDN (RFC 6731). Also, the SMF sends IPv6 multi-homed routing rule along with the IPv6 prefix to the UE to influence the selection of the source Prefix (RFC 4191).

Therefore, after the BP insertion, how to handle subsequent DNS queries depends on the information configured on the UE, which will influence the multi-homing UE to select the DNS server and source prefix.

Centralized

DNS server

SMF

Localized

DNS server

UE

UPF

(PSA-1)

BP

UPF (PSA-2)

DNS Query

(Source: IP@PSA-2, Destination: L-DNS server)

DNS Query

(Source: IP@PSA-1, Destination: C-DNS server)

DNS Query

(Source: IP@PSA-2, Destination: C-DNS server)

Figure-1: DNS query for different options

# 2. Text Proposal

It is proposed to capture the following changes in the TR 23.748.

# * * * * First change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System".

[3]	3GPP TS 23.502: "Procedures for the 5G System".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[5]	3GPP TR 23.758: "Study on application architecture for enabling Edge Applications".

[6]	3GPP TS 22.261: "Service requirements for the 5G system".

[7]	IETF RFC 7871: "Client Subnet in DNS Queries".

[8]	IETF RFC 1546: "Host Anycasting Service".

[9]	IETF RFC 1034: "Domain Names - implementation and specification".

[10]	IETF RFC 7858: "Specification for DNS over Transport Layer Security (TLS)".

[11]	IETF RFC 8484: "DNS Queries over HTTPS (DoH)".

[12]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

[13]	IETF RFC 792: "INTERNET CONTROL MESSAGE PROTOCOL".

[14]	IETF RFC 7157: "IPv6 Multihoming without Network Address Translation".

[15]	IETF RFC 1122: "Requirements for Internet Hosts - Communication Layer".

[16]	3GPP TS 38.300: "NR; Overall description; Stage-2".

[17]	IETF RFC 8311: "Relaxing Restrictions on Explicit Congestion Notification (ECN) Experimentation".

[18]	IETF draft draft-ietf-tsvwg-l4s-arch-06: " Low Latency, Low Loss, Scalable Throughput (L4S) Internet Service: Architecture".

[19]	IETF draft draft-ietf-tsvwg-ecn-l4s-id-10: "Identifying Modified Explicit Congestion Notification (ECN) Semantics for Ultra-Low Queuing Delay (L4S)".

[20]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs".

[21]	3GPP TS 33.122: "Security aspects of Common API Framework (CAPIF) for 3GPP northbound APIs".

[22]	3GPP TS 32.421: "Subscriber and equipment trace; Trace concepts and requirements".

[23]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[24]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[25]	IETF RFC 7556: "Multiple Provisioning Domain Architecture".

[26]	IETF RFC 6731: "Improved Recursive DNS Server Selection for Multi-Interfaced Nodes".

[27]	IETF RFC 8684: "TCP Extensions for Multipath Operation with Multiple Addresses".

[28]	IETF RFC 7681: "Email Exchange of Secondary School Transcripts".

[29]	IETF RFC 4191: "Default Router Preferences and More-Specific Routes".

[30]	3GPP TS 29.503: "5G System; Unified Data Management Services; Stage 3".

[30]	3GPP TS 29.244: "Interface between the Control Plane and the User Plane nodes".

[x]	IETF RFC 4861: "Neighbor Discovery for IP version 6 (IPv6)".

[y]	IETF RFC 8106: "IPv6 Router Advertisement Options for DNS Configuration".

# * * * * Second change * * * *

## 6.22	Solution #22: DNS based EAS discovery supporting session breakout

This solution is for Key Issue #1 on DNS based EAS discovery supporting session breakout.

### 6.22.1	Description

#### 6.22.1.1	Deployment assumption of the solution

Centralized DNS (C-DNS) server is centrally deployed by MNO or 3rd party and responsible for resolving the UE DNS queries into a suitable (Edge) Application Server IP address.

Localized DNS (L-DNS) resolvers/servers may be locally deployed within edge hosting environment, and responsible for resolving the UE DNS queries into a suitable EAS IP address within the Local DN. The L-DNS resolvers/servers may or may not have connectivity with C-DNS server depending on the deployment. The L-DNS resolvers/servers may be deployed by MNO or 3rd parties.

NOTE 1: The C-DNS server and/or L-DNS resolvers/servers may use an anycast address.

NOTE 2: The C-DNS server or L-DNS resolvers/servers may contact any other DNS servers for recursive queries, which is out of control of 5GS and not described in the paper. The C-DNS server or L-DNS resolvers/servers can be different from the authoritative DNS server for the target domain name.

Figure 6.22.1.1-1: Target deployment scenario of the solution

The solution requires a new Functionality, an enhanced DNS Forwarder here referred to as “LDNSR”. LDNSR supports Edge AS Discovery using DNS using knowledge of the 5GC connectivity of the UE. The LDNSR is part of remote PSA UPF.

#### 6.22.1.2	PDU session establishment

When the UE sets up a PDU session a PSA is allocated: this PSA may be a central PSA.

ULCL/BP/local PSA insertion can be triggered by user traffic ("dynamic ULCL/BP/Local PSA insertion") or by other means before user traffic ("pre-established ULCL/BP/Local PSA insertion"):

-	Pre-established ULCL/BP/Local PSA insertion can be done before the UE sends out actual DNS queries or traffic to be locally routed, e.g. at PDU Session establishment or be triggered by AF influence on routing related PCC rules. SMF sets up filters in ULCL/BP for steering to Local PSA of both the edge application traffic and the DNS messages towards L-DNS.

When the L-DNS resolvers/servers does not have connectivity with C-DNS servers, specific mechanisms need to nevertheless support DNS translation for domains not corresponding to the edge resources (DNS requests not targeting the edge application domain). Alternative options are further described in clause 6.22.2.

NOTE 4: 	Encrypted DNS traffic requires L3/L4 filters in ULCL/BP (e.g. DNS Query destination IP address is used) for the steering. Same filters can be used at all locations if L-DNS uses Anycast IP address.

-	Dynamic ULCL/BP/Local PSA insertion is triggered by the DNS messages for EAS FQDN resolution. It requires LDNSR involvement and different modes are possible. LDNSR behaviour is described below.

NOTE 4a:	The pre-established option can be combined with options using LDNSR if it is not wanted that all DNS traffic is handled in the L-DNS in deployment.

#### 6.22.1.3	DNS resolution with Pre-established UL CL/BP/L-PSA

5G core functions (PCF, UDR) can be configured by AF traffic influence routing with the list of DNAIs and IP addresses where the edge application servers are deployed and the traffic filters corresponding to the service to be steered. These apply to all services to be steered including DNS messages. The traffic filters may include the IP address or FQDN of the Local DN deployed service, L-DNS IP address.

Traffic can be steered based on ULCL/BP filters for all DNS messages (DoH, DoT, Do53), and traffic to applications (to EAS).

In the configuration phase, traffic filters for each Local DN deployed service and the DNAI are configured using AF traffic influenced routes. PCF stores the list of traffic filters for each DNAI. This uses 3GPP Rel-16 mechanisms

During PDU session establishment, traffic filters are installed in ULCL/BP using 3GPP Rel-16 mechanisms.

The UE may be configured with DNS resolver IP address per UE interface(e.g. per (DNN, S-NSSAI) using PCO, or use application specific DNS configurations (DoH, VPN), This may further be refined by other solutions.

The pre-established traffic filters are used to steer DNS requests to the closest Local DN or DNAI for EAS discovery. The L-DNS may add an edns-client-subnet (ECS) EDNS0 option before using recursive requests to reach an authoritative DNS server for the target domain.

#### 6.22.1.4	DNS resolution before and after Dynamic UL CL/L-PSA insertion

As for the case of DNS resolution with Pre-established UL CL/BP /L-PSA and L-DNS, the LDNSR is configured as DNS server to the UE during PDU session establishment by SMF via PCO.

LDNSR have visibility of the DNS (DoH, DoT, Do53) parameters.

The EAS discovery can happen in two phases:

-	Phase 1: Before Dynamic UL CL/BP/L-PSA insertion

Option 1: LDNSR receives UL DNS query inserts an ECS option, which includes an IP address/prefix obtained from SMF, into the DNS query and sends to C-DNS. The ECS option is related with the user location and requested FQDN.

Option 2a: LDNSR receives UL DNS query, and it forwards the UL DNS Query to a L-DNS. The L-DNS address is related with the user location and requested FQDN.

Option 2b: LDNSR receives UL DNS query determines the L-DNS address but cannot send traffic to the L-DNS directly. This is further described in clause 6.22.2

For above options, the SMF is notified by the LDNSR and triggers UL CL/L-PSA insertion . In order to support different scenarios, LDNSR may notify SMF based on, EAS IP address in DNS response and/or FQDN.

As an optimization and if DNS request is sent in clear, SMF may optionally configure UL CL to local route some subsequent DNS queries based on FQDNs supported by the Local DN/DNAI. This corresponds to option 3 described as part of phase 2 below.

-	Phase 2: After Dynamic UL CL/BP/L-PSA is inserted

Unless configured to be locally routed (see option 3), LDNSR receives subsequent DNS Queries.

For queries related to that same FQDN from same UE, LDNSR can either redirect the query to L-DNS or follow the procedure as described in Options 1&2 above (e.g. when previous decisions should be reconsidered).

For requests related to new FQDNs, the process is as described in Phase 1. The information of the already inserted UL CL/BP/ Local PSA can also be considered in Option 1 & 2 above.

When LDNSR is configured to apply redirect for an FQDN, ULCL has also been configured by SMF to locally divert to Local PSA the DNS Queries to L-DNS (e.g. on Destination IP address).

Option 3: If in Phase 1 SMF configured a FQDN query can be local routed on the UL CL, then the subsequent DNS queries for the FQDN will be locally routed to the Local DNS Resolver. The Local DNS Resolver will

a)	Either send the DNS query to a L-DNS Server(this can be done by various way listed in sol 4 and sol 20), so that the L-DNS Server can add ECS option based on its own configuration (sol 4/ sol 20) and do recursive DNS resolution if the L-DNS Server cannot resolve the EAS IP address.

b)	Or forward the query to C-DNS server, which assume that the deployment should ensure DNS query packet has topologically correct UE source address when DNS resolver gets it(sol 5: NAT at UPF-PSA rewrites the UE address to an address referring to the L-PSA (and the reverse on the downstream packet))

NOTE 4c: Options 3 assumes that either ULCL/BP steering is based on L3/L4 information or ULCL has visibility of the DNS traffic.

The above options for the EAS discovery using LDNSR can be described in Figure 6.22.1.4-1.

C-DNS server

UE

L-DNS resolver

SMF

(1)

LDNSR

Remote PSA

ULCL/

BP

(2)

Local PSA

Option 1

Option 2

Option 3

(3)

Obtain ECS option

Trigger ULCL insertion

L-DNS
Server

Figure 6.22.1.4-1: Options for the EAS discovery using LDNSR for PDU session breakout

### 6.22.2	Procedures

L-DNS server/resolver

L-DNS server/resolver

UE

ULCL/

BP

Local PSA-2

PSA-1

SMF

C-DNS server

2. DNS query

6. DNS response

LDNSR

3. Interaction with SMF

Option 3

Option 1

4a. DNS query/response

4b. Notify SMF

5a.  ULCL/BP insertion and local PSA selection

PDU Session Establishment with PSA1

Option 2a

4c. DNS query/response

4d. Notify SMF

Traffic Filter

1. DNS Query Triggered

Option 2b

DNS query/response

7. DNS Query

7. DNS Query

PRE-ESTABLISHED ULCL/L-PSA & FILTER

DYNAMIC ULCL/L-PSA & FILTER

5b. Router Advertisement

L-DNS server/

Resolver

DNS Response

Figure 6.22.2-1 DNS resolving in session breakout scenario

The AF provides EAS deployment information to UDR via Nnef_TrafficInfluence, This includes FQDNs, IP addresses/prefixes, Local DNS resolver/server IP address per DNAI. The SMF gets the information from PCF (PCC rule) during PDU session establishment. During PDU Session Establishment procedure, the address of C-DNS server or LDNSR is provided by SMF via PCO to UE. Steps 1 - 6 are processed for each DNS request if UL CL/BP is not inserted or if UL CL/BP is configured to route DNS query to remote PSA.

For the “pre-established” ULCL/BP/L-PSA scenario, the SMF performs UL CL/BP/L-PSA selection and insertion based on UE location and sets up uplink filter rule on UL CL/BP. Steps 1, 7 are processed for each DNS request if UL CL/BP is configured to route DNS query to Local PSA.

The SMF may provision to the LDNSR during PDU session establishment or during UE mobility:

Option 1: The IP address corresponding to DNAI available to UE’s location.

Option 2a: The address of L-DNS server serving the DNAI available to UE’s location.

Option 2b: Indicates LDNSR to forward DNS query with FQDN(s) corresponding to DNAI which has no direct connectivity to LDNSR.

The SMF may configure LDNSR to query IP address or L-DNS server address corresponding to FQDN(s). In this case, the LDNSR sends FQDN in received DNS query to SMF and SMF provides IP address or L-DNS server address to LDNSR.

The SMF also provides the IP range(s) that the LDNSR needs to notify SMF if the IP address of EAS in DNS response is within the IP range(s).

1.	The UE sends a DNS query including the requested FQDN.

2. 	If an ULCL/BP exists and the DNS query matches the uplink filter rule on UL CL/BP to route the DNS query to Local PSA , then step 3 to 6 are skipped. Otherwise the PSA1 UPF forwards the received DNS query to the LDNSR.

3.	If the SMF indicated the LDNSR to contact SMF for the FQDN, the LDNSR contacts SMF and provides FQDN and then:

Option 1: the LDNSR retrieves IP address corresponding to the DNAI selected by SMF based on UE location and requested FQDN. or

Option 2a (with direct connectivity between LDNSR and L-DNS): the LDNSR  retrieves UE location and  the address of L-DNS server serving the DNAI selected by the SMF.

Option 2b (without direct connectivity between LDNSR and L-DNS): the LDNSR requests the SMF to forward the DNS query via the user plane (after step 5). The SMF determines the DNAI and L-DNS address based on the UE location and requested FQDN.

4a-4b:	Option 1: The LDNSR adds the IPv4 subnet or address or IPv6 prefix provisioned by SMF during PDU session establishment or during UE mobility or retrieved from SMF in step 3 as ECS option as specified in RFC 7871 and sends it to C-DNS server. The C-DNS returns the DNS response including EAS IP address.

4c-4d:	Option 2a (with direct connectivity between LDNSR and L-DNS): the LDNSR sends DNS query to the L-DNS server provisioned by SMF during PDU session establishment or during UE mobility or retrieved from SMF in step 3 and get the DNS response including the EAS IP address.

For both 4a-4b and 4c-4d, After receiving the DNS response, based on the IP range(s) provided by SMF, the LDNSR may provide SMF with IP address of EAS and FQDN if the IP address of EAS in DNS response is within the IP range(s) indicated by SMF. Then, SMF decides whether to trigger the insertion of the ULCL/Local PSA .

5a.	The SMF decides DNAI and performs UL CL/L-PSA selection and set up. The DNAI and UL CL/Local PSA2 is selected based on the information provided by LDNSR to ensure the selected local PSA and EAS are corresponding to same DNAI.

Option 2b: If the LDNSR requested the SMF to forward the DNS query in step 3, the SMF sends the DNS query to the address of the local DNS server via the Local PSA2. The SMF configures the Local PSA2 to forward the DNS response to the SMF. When receiving the DNS response, the SMF forwards it to the LDNSR. After receiving the DNS response, the LDNSR provides the SMF with IP address of EAS. The SMF may perform update of the ULCL/L-PSA based on this information. When receiving subsequent DNS queries, the LDNSR may provide the SMF with the new IP address of EAS and the SMF may perform update of the ULCL/L-PSA based on this information.

Optionally, the SMF may configure the L-DNS address in ULCL to be diverted to L-PSA. In order to redirect subsequent DNS queries directly to the L-DNS, the LDNSR acts as an iterative DNS server and redirects the DNS Query to the L-DNS selected for the UE Location by responding with a referral to the UE. This triggers the UE to send a new DNS query towards the L-DNS.

As an optimization, the LDNSR may redirect subsequent DNS queries for the FQDN to the L-DNS. For that, the traffic to that L-DNS needs to be configured in ULCL to be diverted to L-PSA.

As an optimization, the SMF may configure uplink forwarding rules on UL CL to route the traffic and subsequent DNS queries for FQDNs corresponding to the DNAI to the Local PSA2.

5b. In the case of IPv6 multi-homing, the SMF notifies the UE of the availability of the new IP prefix @ PSA2 using an IPv6 Router Advertisement message (RFC 4861 [x]). Optionally, in this RA message, the SMF may notify UE the new address of DNS server (RFC 8106 [y]) and indicate the UE to contact with DHCP server to get the DNS-related configuration information (RFC 4861 [x]), which is used to select DNS server for a target FQDN (RFC 6731[26]).

Also, the SMF sends IPv6 multi-homed routing rule along with the IPv6 prefix to the UE to influence the selection of the source Prefix for the subsequent DNS queries(RFC 4191 [29]) as described in TS 23.501 [2] clause 5.8.2.2.2. The SMF may re-configure the UE for the Source IP prefix @ PSA1.

6.	The LDNSR sends DNS response to UE via PSA1.

7.	In the case of IPv6 multi-homing, if the DNS-related information is configured on UE, the UE decides the DNS server to use for the subsequent DNS queries. Otherwise, the UE uses the DNS server address provided in the PCO during PDU session establishment. Further, the UE selects the source IP prefix based on the IPv6 multi-homed routing rule provided by SMF.

Option 3a and “pre-established” case: Based on the uplink forwarding rules, if an ULCL exists and the query matches the uplink filter rule on UL CL, the UL CL route the DNS query to local PSA2, and then local N6 routing forwards to Local DN. If no filter is present in ULCL, the DNS request is forwarded to the C-DNS.

The DNS query may be routed to a local DNS resolver which then resolves the DNS Query or forwards it for resolution. It may derive an ECS option (using locally configured value or the source address/prefix of the received DNS request), then send the DNS query to A-DNS server using the ECS option.

Alternatively, the DNS query may be routed to C-DNS server via Local N6 or DN. The deployment should ensure the DNS query packet has topologically correct UE source address when DNS resolver gets it, e.g. via NAT in Local DN.

The local DNS resolver or C-DNS server sends DNS response to the UE by retracing the forward path.

Option 3b:

a. The local DNS resolver modifies the packet’s destination IP address (corresponding to a default DNS server) to that of the L-DNS and stores the original IP address (Default-DNS-IP) and the packet’s source IP address (corresponding to UE’s IP address) to its own (i.e. the local DNS resolver’s) IP address and stores the original source IP address (UE-IP) for later processing.

b. The local DNS resolver then forwards the modified DNS request to the L-DNS and processes as follows:

-	If the L-DNS can resolve the IP address for the requested FQDN of EAS, it responds to the local DNS resolver with the desired IP address of the local EAS.

-	If the L-DNS cannot resolve the IP address for the requested FQDN of EAS and the L-DNS is not connected to the C-DNS, it responds to the local DNS resolver with DNS Response without including the IP address of EAS or indicating that it could not resolve the FQDN.

-	If the L-DNS cannot resolve the IP address for the requested FQDN of EAS but it is connected to a C-DNS, it communicates with the C-DNS to recursively resolve the EAS IP address.

c. The local DNS resolver receives a DNS Response including empty value:

-  The local DNS resolver forwards the original DNS request (the one that had originally arrived from the UL CL/BP UPF) back to the UL CL/BP UPF.

-  The UL CL/BP UPF forwards the original DNS Request from local DNS resolver to the remote PSA UPF via N9.

NOTE 7: Option 1 and 2 can be still applicable in Phase 2 if the requested FQDN is not configured to be local routed on the UL CL or the DNS traffic is encrypted.

# * * * * Third change * * * *

### 6.22.3	Impacts on services, entities and interfaces

UE:

- In case of IPv6 multi-homing, the UE receives the DNS-related configuration information for the PDU Session and selects the DNS server to be used for DNS queries of a specific FQDN.

SMF:

- Configure UE with the address of LDNSR as the DNS server during PDU session establishment via PCO.

- Optionally, the SMF provisions to the LDNSR during PDU session establishment or during UE mobility:

- Option 1: The IP address corresponding to DNAI available to UE’s location.

- Option 2a: The address of L-DNS server serving the DNAI available to UE’s location.

- Option 2b: Indicates LDNSR to forward DNS query with FQDN(s) corresponding to DNAI which has no direct connectivity to LDNSR.

- Optionally, the SMF configures the LDNSR with FQDN(s) and indicate the LDNSR to contact SMF for the received FQDN. In this case, the SMF returns to the LDNSR the IP address or L-DNS server address corresponding to FQDN.

- Dynamically inserts ULCL and local PSA and optionally configures the traffic routing rule to ULCL/BP for subsequent DNS queries targeting FQDNs supported by the DNAI.

- In case of IPv6 multi-homing, the SMF indicates the UE to contact with DHCP server to get the DNS-related configuration information for the PDU Session.

UPF:

UPF as an ULCL:

- Optionally, the ULCL is configured with uplink forwarding rules to route the traffic and subsequent DNS queries for FQDNs corresponding to the DNAI to the Local PSA2.

UPF as an LDNSR:

- Optionally, the LDNSR is provisioned with the following information by SMF during PDU session establishment or during UE mobility:

- Option 1: The IP address corresponding to DNAI available to UE’s location.

- Option 2a: The address of L-DNS server serving the DNAI available to UE’s location.

- Option 2b: Indicates LDNSR to forward DNS query with FQDN(s) corresponding to DNAI which has no direct connectivity to LDNSR.

- Optionally, the LDNSR is configured with FQDN(s) by the SMF and indicated to contact SMF for the received FQDN. In this case, the LDNSR obtains the IP address or L-DNS server address corresponding to FQDN from SMF.

- The handling of DNS message in the LDNSR:

- Option 1: the LDNSR adds ECS option in DNS query message.

- Option 2a: the LDNSR redirects the DNS query to the L-DNS.

- Option 2b: the LDNSR forwards the DNS query to the SMF and receives the DNS response from SMF.

- The LDNSR is provided with the IP range(s) and indicated by SMF to notify the IP address of EAS in DNS response to SMF if the IP address of EAS in DNS response matches the IP range(s).

# * * * * End of changes * * * *