# 3GPP TSG-WG SA2 Meeting #139E	S2-2005418

# Elbonia, June 01 – 05, 2020 	(revision of S2-200xxxx)

Source:	CATT

Title:	KI #3B, Sol #7: Update to resolve ENs

Document for:	Approval

Agenda Item:	8.5

Work Item / Release:	FS_IIoT / Rel-17

Abstract of the contribution: This contribution proposes to resolve ENs in solution #7.

Proposal

The following change is proposed for TR 23.700-20.

<<< Start of changes >>>

6.7	Solution #7: Exposure of Time Synchronization

6.7.1	Introduction

For Key Issue #3B: Exposure of time synchronization, a solution is needed that allows "any AF that has knowledge of deterministic application requirements" to "be able to request TSC services from the 5GS and as authorized, be notified of pertinent network events." This solution provides a mechanism to do that whereby the AF may learn 5GS capabilities to support time synchronization, request time synchronization with specified requirements, and supply information that can be used to optimize and configure time synchronization procedure for connected devices.

The solution is applicable for IP and Ethernet types of PDU Sessions.

The solution is applicable for the following synchronization methods:

1)	assuming use of a gPTP client (IEEE 802.1AS [6] Time Aware System) or PTP client (IEEE 1588-2008 [13]), e.g. including leveraging the DS-TT and NW-TT capabilities defined in Rel-16 with IEE802.1AS protocol support.

2)	assuming use of a gPTP client (IEEE 802.1AS [6] Time Aware System) or PTP client (IEEE 1588-2008 [13]) in UPF/NW-TT, where the time source is provided by the 5GS, and UPF/NW-TT is configured to create the (g)PtP message for conveying the timing information (time sync methods as defined in Rel-16).

3)	assuming use of the 5GS time source by 5GS; where the 5G-AN provides a 5GS reference time to the UE via 3GPP radio layer and UE may provide it to the applications or devices behind the UE by implementation specific means out of scope of 3GPP.

4)	assuming use of the 5GS time source by 5GS; where the 5G-AN provides a 5GS reference time to the UE via 3GPP radio layer and DS-TT provides PTP messages for conveying the timing information to devices behind the UE.

This solution can be used together with e.g. Solution #1 for the time source to locate in a TSN node behind the DS-TT/UE.

To enable the above capabilities, this solution proposes to enhance External Exposure of Network Capability (see TS 23.501 [2], clause 5.20), specifically for Provisioning and Monitoring capabilities.

To cover a wide range of possible time synchronization requirements for TSC applications, the AF requests is supplemented with requirements for synchronization. When a requirement for a specific synchronization parameter is provided in the AF request, the PCF may dynamically replace default values with values that meet the specific requirement provided by the AF.

6.7.2	Functional Description

The following capabilities are proposed:

-	The AF requests Time Synchronization service via External Parameter Provisioning, supplying the NEF with requirements for one or more of a time synchronization method, supported (g)PTP versions, Timing Domain, grandmaster priorities, required synchronization accuracy (both in terms of required gPTP rate for each DN port, synchronization accuracy in microseconds per device link, and number of connected gPTP slaves in case 5GS provides the Timing Domain). The AF also provides Target UE(s) Identification or Spatial Validity Condition (i.e. to target a geographical area), AF Identification. Note GPSI may be applied to identify the individual UE in a manner like that used for AF influence on Traffic Routing (see TS 23.501 [2], clause 5.6.7). Then:

-	Time synchronization method 1 (the time source is located in the DN or device side):

Timing Domain is provided from the DN or Device Side and gPTP or PTP method is applied,The PCF may, according to PCC rule authorization, choose a 5QI based on the QoS Reference and dynamically set the PDB and/or MDBV according to requirements for gPTP protocol (e.g. required number of synchronization clients and gPtP messaging rate). The 5GS acts as a PTP Boundary Clock. For Ethernet type PDU sessions, both PTP over UDP/IP and gPTP over Ethernet are applicable. For IP type PDU sessions, PTP over UDP/IP is applicable. In this case, neither the NW-TT port or DS-TT port is a (g)PTP grandmaster.

NOTE 1:	It is assumed that 5G internal system clock is made available to all user plane nodes in the 5G system (e.g. to UEs via SIB/RRC messages).

-	Time synchronization method 2 (the time source is provided by the UPF/NW-TT in 5GS via gPTP or PTP):

Timing Domain is to be provided from the 5GS (or the AF and the 5GS shares the same Timing Domain) The PCF may, according to PCC rule authorization, choose a 5QI based on the QoS Reference and dynamically set the PDB and/or MDBV according to requirements for gPTP protocol. Further, the PCF may, according to PCC rule authorization, choose a gPTP message generation rate for the UPF/NW-TT adaptively. If the AF does not provide a required gPTP message rate, the 5GS may determine a gPTP message rate based on 5GS constraints. The 5GS acts as a (g)PTP grandmaster. For Ethernet type PDU sessions, both PTP over UDP/IP and gPTP over Ethernet are applicable. For IP type PDU sessions, PTP over UDP/IP is applicable. The PTP grandmaster resides in NW-TT.

-	Time synchronization method 3 (the time source is provided by the 5GS and 5G-AN provides a 5GS reference time to the UE via 3GPP radio layer; UE provides time information to attached devices using implementation specific means):

Timing Domain is not applicable. A PCC rule for time synchronization is not required. The method is applicable for both IP type and Ethernet type PDU sessions.

-	Time synchronization method 4 (the time source is provided by the DS-TT in 5GS via gPTP or PTP):

Timing Domain is to be provided from the 5GS (or the AF and the 5GS shares the same Timing Domain). The 5GS acts as a (g)PTP grandmaster. For Ethernet type PDU sessions, both PTP over UDP/IP and gPTP over Ethernet are applicable. For IP type PDU sessions, PTP over UDP/IP is applicable. The PTP grandmaster resides in DS-TT. Solution #X: "PTP GM support by DS-TT" describes the details for the PTP grandmaster in DS-TT. In case the DS-TT cannot act as a GM, then the GM is activated in NW-TT. NEF can be the coordinator to activate/deactivate the PTP functionalities in DS-TT and NW-TT.

Editor's note:	Need to provide additional information (e.g. required accuracy) received from the AF to the UE/DS-TT using PMIC for any of the above methods is FFS.

-	If the AF does not provide a required synchronization accuracy over the air interface, the 5GS may determine a synchronization accuracy based on 5GS constraints.

NOTE 2:	this is intended for a wide range of applications, ex. motion control use-cases, VIAPA, smart grid applications where requirements for synchronization accuracy and deployment scenario may be very diverse.

-	The 5GS exposes the following to aid the AF in formulating a request for Time Synchronization that will be acceptable to the 5GS:

-	5GS Support for Time Synchronization to 5GS.

-	supported the activation/deactivation/modification of Time Synchronization service.

-	supported time synchronization methods (i.e. method(s) 1, 2, 3, 4).

-	For method 2, additional information is exposed, including grandmaster clock quality and clock identity that are provided by UPF/NW-TT.

-	supported (g)PTP versions:

-	IEEE 802.1AS [6] (i.e. gPTP), and/or

-	PTP over UDP/IPv4 according to IEEE 1588-2008 [13] Annex D, and/or

-	PTP over UDP over IPv6 according to IEEE 1588-2008 [13] Annex E.

-	Minimum Time Synchronization accuracy supported.

-	Minimum gPTP or PTP message generation supported, and maximum number of clients that can be supported at the minimum gPTP or PTP message rate.

To support notification, the additional events may be added to the list of monitoring capabilities specified for the NEF in TS 23.502 [3] (see clause 4.15.3.1).

6.7.3	Procedures

The following procedure is used to expose 5GS information to aid the AF in formulating a request for Time Synchronization in 5GS.

Figure 6.7.3-1: Time Synchronization capability exposure towards AF

The following procedure is used by the AF to request activation/deactivation/modification of the Time Synchronization service in 5GS.

Figure 6.7.3-2: Activate/Deactivate/Modify UE's Time Synchronization Service

0.	A PDU session is established between UE/DS-TT and UPF/NW-TT.

1.	An AF sends a Time Sync Service Request indicating the target UE and the clock domain to the PCF via NEF to activate/deactivate/modify UE's time synchronization service.

If the request is target for multiple UEs or for future PDU session(s), the NEF stores the time synchronization service information to UDR, and the PCF subscribes the synchronization service information stored as DataSet “Application Data” and Data Subset “Service specific information” with the specific S-NSSAI and DNN in the UDR. The subscribed information may consist of Announce information reported by DS-TT/NW-TT, UE sync capability and AF request information. The PCF(s) receive(s) a Nudr_DM_Notify notification of data change from the UDR. Optionally, the PCF may indicate new PDU session needs to be immediately triggered by UE based on local policy.

2.	When a request for time synchronization service activation/deactivation/modification is received by the PCF, it determines the time synchronization service policy that includes time synchronization method and allowed synchronization requirements, but if the time synchronization service is already in the target state due to previous request from another AF and the policy is not changed, then steps 3-6 are skipped.

3.	The PCF notifies the SMF via Npcf_SMPolicyControl_UpdateNotify message about the time synchronization service activation/deactivation/modification;

4.	The SMF triggers the SM Policy Association establishment/modification procedure to create a QoS flow (or release the QoS flow) for transmitting the clock domain's (g)PTP message.

5a.	The SMF notifies the UPF/NW-TT about the activation/deactivation/modification of the time synchronization service through the N4 session level modification message, if needed.

5b.	The SMF notifies the UE/DS-TT about the activation/deactivation/modification of the time synchronization service, if needed.

Editor's note:	Whether the SMF notifies UE/DS-TT using Port Management Information Container or via a specific indication from SMF to UE is FFS.

6a.	Upon reception of the notification in step 5, UPF/NW-TT performs consequent behaviours to support time synchronization of corresponding GM, for example if the time source for the Time Sync service requested to be activated is in the UPF/NW-TT, then the UPF will start to create and distribute the (g)PTP messages of the 5G GM to the UE, and (re)configure the port role, if needed.

6b.	Upon reception of the notification in step 5, UE/DS-TT starts or stops to handle the (g)PTP messages as indicated in the notification and (re)configure the port role, if needed.

7.	If PCF receive Time Sync service request from NEF, the PCF responses the NEF and the NEF responses the AF with the Time Sync service request result.

6.7.4	Impacts on services, entities and interfaces

AF:

-	Provides the parameters to the NEF to configure the Time Synchronization service.

-	Provides the priorities for grandmaster in 5GS (DS-TT or NW-TT) for use during BMCA procedure.

NOTE:	The priorities can be used to ensure that the time source provided by 5GS (DS-TT or NW-TT) is selected as a grandmaster towards the devices outside 5GS.

NEF:

-	Exposes the 5GS capabilities to support Time Synchronization service. Authorizes the Time Synchronization service request from the AF..

UE:

-	Distributes the 5G reference time to DS-TT and optionally to applications or devices behind the UE (by mechanism out of scope of 3GPP), for IP and Ethernet types of PDU Sessions.

DS-TT:

-	Indicates a support for PTP/UDP/IP residence time calculation in Port Management Information container to the network.

-	When activated by NEF, calculates and adds the measured residence time between the NW-TT and DS-TT into the Correction Field (CF) of the PTP time synchronization messages conveyed over UDP/IP, applies to IP and Ethernet types of PDU Sessions.

-	When activated by NEF, (if the time source is located behind the DS-TT as in Solution #1): Inserts a timestamp to the PTP time synchronization messages conveyed over UDP/IP, applies to IP and Ethernet types of PDU Sessions.

-	When activated by NEF, (if the time source is located in the DS-TT): generates PTP time sync messages (with the indicated PTP version, domain number(s), sending rate, etc), applies to IP and Ethernet types of PDU Sessions. Solution #X: "PTP GM support by DS-TT" describes the details for the PTP grandmaster in DS-TT.

-	Reports Announce information to NEF.

UPF/NW-TT:

-	When activated by NEF, (if the time source is located in the DN): Inserts a timestamp to the PTP time synchronization messages conveyed over UDP/IP, applies to IP and Ethernet types of PDU Sessions.

-	When activated by NEF, (if the time source is located in the NW-TT): generates PTP time sync messages (with the indicated PTP version, domain number(s), sending rate, etc), applies to IP and Ethernet types of PDU Sessions.

-	Forwards the PTP time synchronization messages conveyed over UDP/IP in DL direction to the PDU Sessions, applies to IP and Ethernet types of PDU Sessions.

-	Reports Announce information to NEF.

UDR:

-	Stores synchronization service information.

PCF:

-	Determines the time synchronization service policy.

<<< End of changes >>>