SA WG2 Meeting #140E (e-meeting)	S2-2005576

# August 19th – September 2nd, 2020, Elbonia		                             (revision of S2-20yyyyy)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2383 | rev | - | Current version: | 16.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NSAAA re-authentication by AAA-S |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS |  |  |  |  |  | Date: |  |  | 13-08-2020 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | When the AAA-S initiates re-authentication or revocation, the UE is not always registered in 5GS or the S-NSSAI is not registered. To ensure that the NSSAA is executed in case the UE later registers to the specific S-NSSAI, the AMF updates the NSSAA status in the UE context. |  |  |  |  |  |  |  |  |
| Summary of change: |  | AMF logic added to update the NSSAA status in case AAA-S invokes re-authentication or revokation procedure as to ensure a new NSSAA is executed in case the UE tries to register the S-NSSAI again. Also, in the Figure 4.2.9.3-1, Step 3c is added and step 5 corrected, in figure 4.2.9.4-1 step 3c is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is no mechanism to ensure NSSAA is executed when UE tries to register an S-NSSAI after revocation or re-authentication procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.9.3; 4.2.9.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** BEGIN CHANGES ***

#### 4.2.9.3	AAA Server triggered Network Slice-Specific Re-authentication and Re-authorization procedure

Figure 4.2.9.3-1: AAA Server initiated Network Slice-Specific Re-authentication and Re-authorization procedure

1.	The AAA-S requests the re-authentication and re-authorization for the Network Slice specified by the S-NSSAI in the AAA protocol Re-Auth Request message, for the UE identified by the GPSI in this message. This message is sent to a AAA-P, if the AAA-P is used (e.g. the AAA Server belongs to a third party), otherwise it is sent directly to the NSSAAF.

2.	The AAA-P, if present, relays the request to the NSSAAF.

3a-3b.	NSSAAF gets AMF ID from UDM using Nudm_UECM_Get with the GPSI in the received AAA message.

3c.	The NSSAAF provides an acknowledgement to the AAA protocol Re-Auth Request message. If the AMF is not registered in UDM the procedure is stopped here.

4.	If the AMF is registered in UDM, the NSSAAF notifies Re-auth event to the AMF to re-authenticate/re-authorize the S-NSSAI for the UE using Nnssaaf_NSSAA_Notify with the GPSI and S-NSSAI in the received AAA message. The callback URI of the notification for the AMF is derived via NRF as specified in TS 29.501 [62].

5.	If the UE is registered with the S-NSSAI in the Mapping Of Allowed NSSAI, the AMF triggers the Network Slice-Specific Authentication and Authorization procedure defined in clause 4.2.9.1. If the S-NSSAI is included in the Allowed NSSAI for 3GPP access and non-3GPP access, AMF selects an access type to perform NSSAA based on network policies. If the S-NSSAI is only included in the Allowed NSSAI of non-3GPP access and UE is CM-IDLE in non-3GPP access, the AMF marks the S-NSSAI as pending. In this case, when UE becomes CM-CONNECTED in non-3GPP access, the AMF initiates NSSAA if needed. If the UE is registered but the S-NSSAI is not in the Mapping Of Allowed NSSAI, the AMF updates the status of the S-NSSAIs subject to Network Slice-Specific Authentication and Authorization in the UE context such that an NSSAA is executed next time the UE tries to register with the S-NSSAI.

#### 4.2.9.4	AAA Server triggered Slice-Specific Authorization Revocation

Figure 4.2.9.4-1: AAA Server-initiated Network Slice-Specific Authorization Revocation procedure

1.	The AAA-S requests the revocation of authorization for the Network Slice specified by the S-NSSAI in the AAA protocol Revoke Auth Request message, for the UE identified by the GPSI in this message. This message is sent to AAA-P if it is used.

2.	The AAA-P, if present, relays the request to the NSSAAF.

3a-3b.	The NSSAAF gets AMF ID from UDM using Nudm_UECM_Get with the GPSI in the received AAA message.

3c.	The NSSAAF provides an acknowledgement to the AAA protocol Re-Auth Request message. If the AMF is not registered in UDM the procedure is stopped here.

4.	If the AMF is registered in UDM, the NSSAAF notifies Revoke Auth event to the AMF to revoke the S-NSSAI authorization for the UE using Nnssaaf_NSSAA_Notify with the GPSI and S-NSSAI in the received AAA message. The callback URI of the notification for the AMF is derived via NRF as specified in TS 29.501 [62].

5.	If the UE is registered with the S-NSSAI in the Mapping Of Allowed NSSAI, the AMF updates the UE configuration to revoke the S-NSSAI from the current Allowed NSSAI, for any Access Type for which Network Slice Specific Authentication and Authorization had been successfully run on this S-NSSAI. The UE Configuration Update may include a request to Register if the AMF needs to be re-allocated. The AMF provides a new Allowed NSSAI to the UE by removing the S-NSSAI for which authorization has been revoked. The AMF provides new rejected NSSAIs to the UE including the S-NSSAI for which authorization has been revoked. If no S-NSSAI is left in Allowed NSSAI for an access after the revocation, and a Default NSSAI exists that requires no Network Slice Specific Authentication or for which a Network Slice Specific Authentication did not previously fail over this access, then the AMF may provide a new Allowed NSSAI to the UE containing the Default NSSAI. If no S-NSSAI is left in Allowed NSSAI for an access after the revocation, and no Default NSSAI can be provided to the UE in the Allowed NSSAI or a previous Network Slice Specific Authentication failed for the Default NSSAI over this access, then the AMF shall execute the Network-initiated Deregistration procedure for the access as described in clause 4.2.2.3.3, and it shall include in the explicit De-Registration Request message the list of Rejected S-NSSAIs, each of them with the appropriate rejection cause value. If there are PDU session(s) established that are associated with the revoked S-NSSAI, the AMF shall initiate the PDU Session Release procedure as specified in clause 4.3.4 to release the PDU sessions with the appropriate cause value. If the UE is registered but the S-NSSAI is not in the Mapping Of Allowed NSSAI, the AMF updates the status of the S-NSSAIs subject to Network Slice-Specific Authentication and Authorization in the UE context such that an NSSAA is executed next time the UE tries to register with the S-NSSAI.

*** END OF CHANGES ***