3GPP TSG-WG SA2 Meeting #140E e-meeting 	S2-2005618

Elbonia, August 19 – September 1, 2020	(revision of S2-200xxxx)

Source:	Huawei, HiSilicon

Title:	KI #2: interim agreements

Document for:	Approval

Agenda Item:	8.2

Work Item / Release:	FS_eNPN / Rel-17

Abstract: This contribution introduces evaluations and interim agreements for KI#2

1. Introduction/Discussion

Void

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-07 v0.4.0.

# * * * * First change * * * *

7.X	Key Issue #2: NPN support for Video, Imaging and Audio for Professional Applications (VIAPA)

Editor's note:	This clause will provide a general evaluation and comparison of the solutions per Key Issue #<X>.

Sol#1: this solution proposes to use the roaming based architecture and procedures to address this KI for SNPN and PLMN.

Sol#2: this solution proposes to use the roaming based architecture and procedures to address this KI for SNPN and PLMN.

Sol#13: this solution proposes to use the N3IWF based architecture and procedures to address this KI for SNPN and PLMN.

Sol#15: this solution proposes to use the N3IWF based architecture and procedures to address this KI for SNPN and PLMN.

Sol#16: this solution proposes to address this KI for PNI-NPN and PLMN.

Sol#X: this solution proposes to use the roaming based architecture and procedures to address this KI for SNPN and PLMN in case of the UEs only using the credentials/subscription of the PLMN, and proposes to use the N3IWF based architecture and procedures to address this KI for SNPN and PLMN in case of the UEs using both the credentials/subscription of the SNPN and credentials/subscription of the PLMN, and also proposes to address this KI for PNI-NPN and PLMN. This solution further considers how to enable the SNPN operator to control whether a UE can establish an HR PDU session via SNPN, or whether a UE can access to PLMN N3IWF via SNPN, and how to enable the PLMN operator to control whether a UE can perform the secondary 3GPP registration at the PLMN by taking the UE subscription, UE capability (SR or DR), local policy, agreements between SNPN and PLMN, and network topology (whether 5GC is shared between SNPN and PLMN) into account.

# * * * * Second change * * * *

8.X	Key Issue #2: NPN support for Video, Imaging and Audio for Professional Applications (VIAPA)

Editor's note:	This clause will capture conclusions for Key Issue #<X>.

Take Solution X as the baseline.

# * * * * End of changes * * * *