SA WG2 Meeting #140E	S2-2005631

Electronic meeting, Aug 19 – Sep 01, 2020	(Revision of S2-200nnnn)

Source:	Nokia, Nokia Shanghai Bell

Title:	Update of Solution #3 to detail mission critical service activation

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	FS_5MBS / Rel-17

Abstract of the contribution:

This paper proposes an update to solution 3 to detail mission critical service activation.

# 1	Discussion

The call flow showing mission critical service activation for solution 3 is very high-level and contains an editor´s note the role of the MSF-U and MSF-C requires clarifications. The mentioned SIP messages are not fully aligned with MCPTT TS 24.379. Further, existing public safety deployments use the MB2 interface as defined in 3GPP TS 23.468 and it is therefore desirable to show how interworking with that interface can be provided by MSF-U and MSF-C. Using unchanged MB2 interface procedures and signalling between UE and AF enables a smooth migration of mission critical services from LTE to 5MBS and if MSF-C/MSF-U and BMSC are collocated even allows the same mission critical sessions to use both LTE MBMS and 5MBS.

# 2	Conclusions

Proposal: Agree the proposed changes to Solution #3 to detail mission critical service activation.

Proposed changes to TR 23.757

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[3]	3GPP TS 22.101: "Service aspects; Service principles".

[4]	3GPP TS 23.246: "Multimedia Broadcast/Multicast Service (MBMS); Architecture and functional description".

[5]	3GPP TS 23.468: "Group Communication System Enablers for LTE (GCSE_LTE)".

[6]	3GPP TS 26.348: "Northbound Application Programming Interface (API) for Multimedia Broadcast/Multicast Service (MBMS) at the xMB reference point".

[7]	3GPP TS 23.316: "Wireless and wireline convergence access support for the 5G System (5GS)".

[8]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[9]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[10]	3GPP TS 38.300: "NR; Overall description; Stage-2".

[11]	3GPP TR 23.748: "Study on enhancement of support for Edge Computing in 5G Core network (5GC)".

[12]	RP-193248: "New Work Item on NR support of Multicast and Broadcast Services", RAN#86.

[13]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[x]	3GPP TS 23.280: "Common functional architecture to support mission critical services; Stage 2".

[y]	3GPP TS 24.379: "Mission Critical Push To Talk (MCPTT) call control; Protocol specification".

[z]	3GPP TS 23.003: "Numbering, addressing and identification".

##### 6.3.2.2.4	Example of mission critical service activation via MSF and MSF interaction with MBS transport.

Figure 6.3.2.2.4-1 shows an example of MSF (service layer) and MBS transport interaction to activate mission critical services as defined in TS 23.280 [x] and TS 24.379 [y], and AF/MSF interaction when an AF establishes an MBS service session via MSF, and configures UE e.g. via SIP signaling as defined in TS 24.379 [y].

NOTE:	AF and UE may interact using different application layer protocols.

MB2 interface procedures as defined in TS 23.468 [5] are used in this example; using unchanged MB2 interface procedures and signaling between UE and AF enables a smooth migration of mission critical services from LTE to 5MBS and if MSF-C/MSF-U and BMSC are collocated even allows the same mission critical sessions to use both LTE MBMS and 5MBS. TMGIs are used in the MB2 interface and the mission critical signaling between AF and UE as identifier of the multicast session. However, the inband IGMP/MLR Join used in Alternative 1 of Figure 6.3.2.1-1 uses IP multicast addresses as identifier of the multicast session; to allow those different identifier formats to coexist, the MBMS service identifier within the TMGI (see TS 23.003 [z]) is mapped in a standardized way to a source specific IP multicast address, and MCC and MNC are mapped to the source IP address.

Figure 6.3.2.2.4-1: Use of MBS service activation via MSF and MSF interaction with MBS transport

1.	A UE is establishing or invited to a mission critical session (e.g. a MCPTT session) using procedures in TS 23.280 [x] and TS 24.379 [y] (e.g. via a SIP Invite to join a conference)  This may trigger the AF to perform a Multicast group configuration via NEF. An AF may also add other UEs to a mission critical session after it has configured the multicast group.

If the AF decides to configure a related multicast group, steps 2 to 10 apply.

2-4.	The AF may request the allocation of TMGIs via dedicated procedures or include the request for TMGI allocation in step 5. The MSF-C is allocating TMGIs and related expiry times and also terminates MBS services upon time expiry.

5.	the AF sends Activate MBMS Bearer Request (TMGI, service area, optional: media information (FEC/ROHC), , start time) to MSF-C.

6.	The MSF-C maps the TMGI to a source specific IP multicast address and related source IP address and uses them in step 7. The MSF-C also allocates a flow ID for the service area received in step 5.

7.	The MSF-C initiates MBS Multicast group configuration as described in clause 6.3.2.2.2. It handles the service area as described for the location area in solution 21.

8.	If the MSF-C decides to insert an MSF-U for media processing, the MSF-C configures the MSF-U to handle the MBS session and provides the address information received in step 7 as destination address and the media information received in step 5.

9.	The MSF-U allocates a transport address to receive the multicast data and reserves resources for media processing (e.g. FEC, ROHC) if instructed by the MSF-C.

10.	The MSF-C responds to AF with Activate MBMS Bearer Response (TMGI, if MSF-U was inserted MSF-U transport address information and otherwise transport address information received in step 7)

11.	The AF sends a bearer announcement to the UE (e.g. a SIP Message request according to TS 24.379 [y]) and includes the TMGI.

12. The UE maps the TMGI to a source specific IP multicast address.

13.	The UE joins the Multicast group by initiating PDU session establishment/modification procedure including the multicast IP address information as described in clause 6.3.2.1.

14.-17.	Multicast data are transmitted from the AF to the UE. If configured accordingly, the MSF-U performs FEC and/or ROHC related processing of the data.