SA WG2 Meeting #140E	S2-2005657

Electronic meeting, Aug 19 – Sep 01, 2020	(Revision of S2-200nnnn)

Source:	Nokia, Nokia Shanghai Bell

Title:	KI#6 Update of Solution #21

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	FS_5MBS / Rel-17

Abstract of the contribution:

This paper updates solution #21 to resolve the open editor´s notes and align with updates proposed for solution 3.

# 1	Discussion

The following editors´s notes need to be addressed:

Editor's note:	The multicast group join procedure need align with the general multicast group join procedure defined in clause 6.3.2.1, e.g. step 6-17.

# 2	Conclusions

This contribution discussed solution #27 in the context of network deployment in which the mobility to a NG RAN

Proposal: Agree the proposed changes to Solution #27.

Proposed changes to TR 23.757

### .21.1	Functional Description

#### 6.21.1.1	System Architecture

This solution addresses Key Issue 6. The architecture functional entities are described in clause A.1 "5G MBS system architecture based on unicast 5GC". Solution 3 is extended to address specific requirements of local MBMS services.

#### 6.21.1.2	Specific additions to solution 3 for local MBMS services

It is desirable to avoid changing multicast session IDs depending on UE location for services with different content depending on UE location, e.g. for V2X. Otherwise constant updates of the UEs and complex detection procedures or configuration with location dependent multiplex session IDs are required.

A single multicast Session ID is assigned for a multicast group, but the contents distributed as part of the multicast group differ depending on the location area where the UE is residing.

Only the multicast Session ID is announced to the UE and the network selects the related contents depending on the UEs' location.

Content provisioning in proximity to the location area where the content is to be distributed seems desirable to reduce transport delay, that is critical in particular for V2X applications.

It is possible to assign different ingress nodes for different location area dependent content for a multicast group.

To achieve those aims, the multicast group context in the UDR is extended with location area dependent information:

-	A description of the location are, e.g. list of cells, TAIs.

-	A network-internal local area ID assigned to the location area that is unique for the multicast session.

-	An ingress node assigned to the location area.

-	Possibly a multicast transport address assigned to the location area.

An AF may register at the NEF that it provides contents for a multicast group for a specific location area. The NEF then selects an ingress node for the multicast group in the specific location area and a local area identifier is assigned. The information is stored in the UDR.

NOTE: The functionality of the local area identifier is comparable with the MBMS Flow Id.

Alternatively, the UDR can contain configured information based on commercial agreements with the content provider.

An SMF can provide a UE location to the UDR to query for location dependent information within a multicast session context.

The SMF serving a UE provides information about the access to content for the multicast session in the location area towards the RAN. The RAN contacts the related ingress node to join the location area dependent distribution tree for the multicast content

The AMF notifies the SMF when the UE leaves the location area. The SMF obtains multicast context information for the new UE location from the UDR and then provides information about the access to content for the multicast session in the new location area towards the RAN.

### 6.21.2	Procedures

#### 6.21.2.1	Multicast context and Multicast flow setup/modification via PDU Session Modification procedure

The flow is the same as described in clause 6.3.2.1, with the differences highlighted in the description of the steps.

Figure 6.21.2.1-1: PDU Session modification for multicast

1.	The content provider may send a request to register and reserve resources for a multicast group in a specific location area to the NEF as detailed in clause 6.21.2.2. The NEF select MB-SMF as ingress node for the location area and stores related information in the UDR

NOTE:	The request to reserve resources for the corresponding multicast session is optional and can be replaced by configured data based on commercial agreements.

2.	Same as in clause 6.3.2.1

3.	Same as in clause 6.3.2.1. No information about the local content is announced

4.	Same as in clause 6.3.2.1

5.	Same as in clause 6.3.2.1

6.	The SMF queries the UDR for information about the multicast context. If the UDR provides information about location areas stored within the multicast context (including MB-SMF ID, location are and area session ID for each location area), the SMF selects location dependent multicast context information based on the location area where the UE is residing. The SMF determines the applicable MB-SMF ID, area session ID, location area in that manner.

7-8.:	If SMF has no information about the multicast context for the indicated multicast group and area session ID, SMF interacts with MB SMF to retrieve QoS information of the multicast QoS flow(s) for the multicast group and area session ID.

9.	The SMF requests the AMF to transfer a message to the RAN node using the Namf_N1N2MessageTransfer service (N2 SM information (PDU Session ID, Multicast group ID, MB-SMF ID, location area, area session ID),

10.	The RAN is using the received multicast Multicast group ID and area session ID to determine the localized multicast distribution context.

11.	Same as in clause 6.3.2.1.

12.	RAN node checks whether the user plane for the multicast group/context and location area distribution is already established towards the RAN node.

If RAN supports MBS, and if no user plane for multicast group distribution for the area session ID is established towards the RAN node, steps 13 to 17 are executed

13.	RAN signals a request for the user plane establishment towards that AMF [MB-SMF ID, Multicast group ID, area session ID].

14.	Same as in clause 6.3.2.1. MB-SMF identifies media distribution session based on Multicast group ID and area session ID

15-24:	Same as in clause 6.3.2.1.

25.	SMF signals a request for the multicast session distribution towards MB-SMF [Multicast context/group ID, area session ID, downlink tunnel info].

27-37:	Same as in clause 6.3.2.1.

#### 6.21.2.2	Optional Multicast group registration

Figure 6.21.2.2-1: Optional Multicast group registration

1:	AF of content provider may register at the NEF that it provides contents for a multicast session (identified by multicast group ID which may be IP multicast address) for a certain location area (e.g. list of cells, TAIs, geographical area). (Multiple AFs may register for the same multicast session but different location areas.) NEF selects MB-SMF as ingress control node, possibly based on location area.

2:	Same as in clause 6.3.2.2.

3-4:	NEF requests storage of multicast session at UDR and provides multicast session ID, selected MB-SMF ID and location area. The UDR allocates an area session ID for the location area and stores all these data.

5:	NEF request MB-SMF to reserve ingress resources for a multicast distribution session and provides Multicast session ID and area session ID

6.	The MB-SMF sends SM MBS Policy Association Request to PCF with the Multicast group ID, area session ID, and the QoS requirements.

7. The PCF registers at the BSF that it handles the multicast session for the location area. It provides an identifier that the policy association is for multicast and the multicast group ID, area session ID, its own PCF ID and optionally its PCF set ID.

8.	The PCF may query the UDR for policy input related to the multicast session and area session ID

9-13:	Same as in clause 6.3.2.2.

### 6.21.3	Impacts on services, entities and interfaces

Same as in clause 6.3.3. Location area Identifier needs to be stored and signalled in addition.