3GPP TSG-WG SA2 Meeting #139E e-meeting 	S2-2005676

Elbonia, August 19 – September 1, 2020	(revision of S2-200xxxx)

Title:	[DRAFT] LS on Bulk operation of LCS-service

Response to:	LS (C4-203629) on LS on Bulk operation of LCS-service from CT4

Release:	Release 16

Work Item:	5G_eLCS

Source:	[Huawei to be] SA2

To:	CT4

Cc:	CT3

Contact Person:

Name:	Runze Zhou

Tel. Number:

E-mail Address:	zhourunze AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	DocNumber(s) [Description i.e. Draft TS 29.414 v0.1.0]. 
!! WARNING !! Do not insert the file directly as an object in this word document.

1. Overall Description:

SA2 thanks CT4 for the LS on Bulk operation of LCS-service (C4-203629). SA2 has discussed the questions listed in the LS and would like to provide the following answers:

A general information provided to CT4 is SA2 has corrected the TS 23.273 by removing the case MT-LR with bulk operation that triggered by LCS client. It means only AF can trigger MT-LR with bulk operation. Details see attached CR 0130.

- When using Service Based Interface (SBI), which node is expected to be the aggregation point of Bulk Operation of LCS Service, i.e. the node who resolves the group identifier to member UEs and aggregate the location requests and responses for each UE in the group, NEF or GMLC or both?

[SA2 answer] when AF triggers location request with bulk operation, if the LCS QoS provided by AF indicate finer granularity than cell ID, NEF will forward the AF request to GMLC for further handling. Otherwise NEF forwards the AF request to AMF for further handling (no relevance with GMLC).

Once GMLC receives the location request from NEF, it is the GMLC interacts with the UDM to resolve the SUPI of every group member UE by providing the group ID.

In case of LIR:

After retrieving location information of each UE, it is the GMLC aggregates all the UE location and provides to NEF together with group ID.

In case of LDR:

- Periodic UE positioning: GMLC will aggregates all the UE location and provides to NEF together with group ID every periodical interval;

- Event reporting from UE: there is no aggregation by GMLC. Once a UE location event is retrieved, GMLC will provides it to NEF separately;

- If the GMLC is expected to be the aggregation point of Bulk Operation of LCS Service, what should the GMLC respond for a request of bulk operation, especially for 5GC_MT_LR procedure?

NOTE:	CT4 observed that if NEF sends a 5GC-MT-LR request towards GMLC for group of UEs, there is no way for GMLC to relay the multiple responses (one per UE) to the NEF, using current service operations specified in TS 23.273 (Only one service response is allowed per service request in SBI and location for one UE is allowed in Ngmlc_Location_ProvideLocation response).

[SA2 answer] It is possible there are more than one AMF serving the group member UEs, and GMLC will interacts with all the relevant AMFs. When GMLC receives the UE location from AMF(s) for all the group member UEs, GMLC will provide all the UE location result to the NEF, together with the group ID it received before. NEF forwards what GMLC provides to it further to the AF.

- If used as aggregation point, when does the GMLC / NEF acknowledge the request for Bulk Operation, i.e. does it respond the LCS client / AF after it received all positioning responses from network side for all UEs in the group or respond without AMF checking?

[SA2 answer] NEF responds the AF after it received all positioning responses from network side for all UEs in the group from the GMLC.

- If GMLC / NEF responds the LCS client / AF immediately, based on what criterion does the GMLC / NEF decide to respond success or failure or partial success? And How should GMLC / NEF to notify the LCS Client / AF if the requests for positioning for some UEs fail because of some different reasons (e.g. The UE is not online) after GMLC / NEF responded success to LCS client / AF?

[SA2 answer] as described in answers to question 3, GMLC/NEF shall not responds to AF immediately.

Regarding the case that UE location is available, SA2 has defined a “partial failure” mechanism in clause 6.8 TS 23.273.

2. Actions:

To CT3 and CT4 group.

ACTION: 	SA2 asks CT3 and CT4 group to take the above answers into account.

Guidance – Provide clear information on what the other groups must do, and by when

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #141E		October 12 – 23, 2020			Elbonia

TSG-SA2 Meeting #142E		November 16 – 20, 2020			Elbonia