SA WG2 Meeting #140E	S2-2006019

August 19 - September 2, 2020, Elbonia	(revision of S2-2005799, S2-2003651, S2-2000733)

Source:	Apple, Samsung, Interdigital, NEC

Title:	KI #1, KI#3, New Sol: Using Non-3GPP access to deliver NAS Notification message

Document for:	Approval

Agenda Item:	8.10

Work Item / Release:	FS_MUSIM / Rel-17

Abstract of the contribution: This solution proposes to use the procedure in TS 23.502 clause 4.2.3.3 step 4c when the Multi-USIM UE is in CM-CONNECTED state for non-3GPP access, the network needs to signal with the Multi-USIM UE, and the Multi-USIM UE has left the current 3GPP system for 3GPP access.

# 1	Background

This contribution proposes a solution to Key Issue #1: Handling of Mobile Terminated service with Multi-USIM device.

* * * * Start of Changes * * * *

## 6.0	Mapping Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 |
| 1 | X |  |  |  |
| 2 | X |  |  |  |
| 3 | X |  |  |  |
| 4 |  |  | X |  |
| 5 |  |  | X |  |
| 6 |  |  | X |  |
| 7 | X | X |  |  |
| 8 | X |  |  |  |
| 9 | X |  |  |  |
| 10 | X |  |  |  |
| 11 | X |  |  |  |
| 12 | X | X |  |  |
| 13 | X |  |  |  |
| 14 |  | X |  |  |
| 15 |  | X |  |  |
| 16 |  | X |  |  |
| 17 |  | X |  |  |
| 18 |  | X |  |  |
| 19 |  | X |  |  |
| 20 |  | X |  |  |
| 21 |  | X |  |  |
| 22 |  |  | X |  |
| X | X |  | X |  |

* * * * Next Change (all new text) * * * *

## 6.X	Solution #X: Using Non-3GPP access to deliver NAS Notification message

### 6.X.1	Introduction

This solution addresses the Key Issue #1 "Handling of Mobile Terminated service with Multi-USIM device".

This solution is applicable only when UE is in coverage of N3GPP access and therefore cannot be considered as a complete solution for KI#1.

The current specification TS 23.502 [6] clause 4.2.3.3 step 4c covers the case when the UE is in CM-IDLE state in 3GPP access and in CM-CONNECTED state in non-3GPP access, the network may initiate a Network Triggered Service Request procedure for 3GPP access via non-3GPP access.

### 6.X.2	Functional Description

When the network needs to signal (e.g. N1 signalling to UE, mobile-terminated SMS, user plane connection activation for PDU session to deliver mobile terminating user data) with the Multi-USIM UE, and if the Multi-USIM UE:

1.	is simultaneously registered over 3GPP and non-3GPP accesses in the same PLMN;

2.	is in CM-CONNECTED state for non-3GPP access;

3.	has left the current 3GPP system for 3GPP access (e.g. to service a voice call activity on the other USIM).

The UE shall indicate this step to AMF either via NAS Leaving procedure as specified in solution#5, clause 6.5.3.1.1 or the RRC leaving procedure as specified in solution#5, clause 6.5.3.3.1. The UE is either in a state after step 3 of Figure 6.5.3.1.1-1 or after step 2 of Figure 6.5.3.3.1-1; and

4.	the PDU Session ID is associated with 3GPP access (applicable for user plane connection activation for PDU session):

the AMF shall notify the UE through non-3GPP access, i.e. by sending a NAS Notification message containing the 3GPP Access Type to the UE over non-3GPP access and set a Notification timer. If the UE has provided the MUSIM-RAI as specified in clauses 6.5.3.1.1 and 6.5.3.3.1, the UE can include those PDU sessions in the MUSIM-RAI that the UE wants network to trigger (or not trigger) MT service delivery indication using the non-3GPP access. The UE may also indicate to the network that all MT data related to 3GPP access shall be delivered over non-3GPP access.

If the UE is not in CM-CONNECTED state for non-3GPP access and if the UE is leaving the current 3GPP system for 3GPP access, UE may then initiate service request procedure via non-3GPP access as described in clause 4.12.4.1 of TS 23.502 [6] to request re-establishment of the NAS signalling connection.

Editor’s Note: Whether and how to handle the scenario where the UE is registered over 3GPP and non-3GPP access in different PLMNs is FFS.

### 6.X.3	Procedures

Network Triggered Service Request procedure in TS 23.502 [6] clause 4.2.3.3 step 4c applies.

### 6.X.4	Impacts on existing entities and interfaces

UE: upon coordinated leaving requests from the network to be paged via NAS Notification.

AMF: when requested by the UE the AMF shall page the UE via NAS Notification.

* * * * End of Changes * * * *