3GPP TSG-WG SA2 Meeting #140E e-meeting 	S2-2006046

Elbonia, August 19 – September 1, 2020	(revision of S2-2005413)

Source:	Huawei (Rapporteur)

Title:	KI#1: Principles for categorization of Solutions for KI1

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	FS_5MBS / Rel-17

Abstract: This document is trying to find the common aspects of the solutions and achieve the interim conclusions.

1. Introduction/Discussion

This document captures specific principles for 5G MBS.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.757.

# * * * * First change * * * *

7	Conclusions

7.X	Conclusions for Key Issue #1: MBS session management

7.X1	Interim requirements for conclusions

Conclusions will take into account the following agreed system requirements:

- For multicast solutions, signalling from the UE to the network to join a multicast session shall be supported by UE and network. Join/leave operation via CP (NAS) signalling shall be supported.

Editor’s Note: It is FFS if the network and UE shall support multicast session join/leave operation via UP e.g. IGMP Join/Leave.

- For N3 transport of the shared delivery method, GTP-U tunnelling using a transport layer IP multicast method and shared N3 (GTP-U) Point-to-Point tunnel shall be supported with support for QoS.

- Both 5GC Shared MBS traffic delivery method and 5GC Individual MBS traffic delivery method shall be standardized for multicast data delivery.

- The network shall be able to prepare and start the multicast traffic transmission for a MBS session after MBS service is started.

- The network shall support selection of MB-SMF or SMF (depending on solution) at session join.

- For N3 transport of the 5GC shared MBS delivery method, for unicast transport there shall be 1-1 mapping between MBS Session and GTP-U tunnel towards a RAN node, and for multicast transport there shall be 1-1 mapping between MBS Session and the GTP-U tunnel.

Editor's Note: It is FFS whether or not the evaluation criteria should include the requirement that the application should be not aware of 5GS specific or internal information.

# * * * * End of changes * * * *