3GPP TSG-WG SA WG2 Meeting #140E e-meeting	S2-2006259

Elbonia, August 19 - September 1, 2020                                           (revision of S2-2005344)

Source:	Huawei, HiSilicon

Title:	 KI #10, KI#16, Sol# 31: Update solution 31 for NWDAF Assisted UP Optimization for edge computing

Document for:	Discussion / Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_ph2 / Rel-17

Abstract of the contribution: This contribution proposes to update the solution 31 for NWDAF Assisted UP Optimization for edge computing, by adding impacts part and deleting the Editor Note.

- Discussion

This paper is to complete Solution 31 by adding the clause of Impacts on services, entities and interfaces, which is missing at the moment.

On the other hand, an editor note as following needs to be addressed.

//extracted from TR 23.700-91 solution 31//

Editor's note:	It is FFS how the SMF determines the Application Server performance corresponding to the DNAI selected.

The output analytics provided by NWDAF to SMF in solution 31 includes the average and/or variance service MOS, which is to reflect the service quality per whole UP path (including UPF, DNAI) selected.

In order to better reflect the AS performance corresponding to the DNAI, besides the input data from 5GC NFs/OAM, NWDAF may need to takes into account some performance data from AF, e.g. Average Packet Delay, Average Loss Rate, and Throughput, which is defined as in Table 6.49.2-1: Application Server Performance Data from EDN network, clause 6.49, TR 23700-91.

So this paper proposes to add performance data from AF as additional input data, and then the output service experience analytics can better reflect the AS performance corresponding to the DNAI, this will help the SMF determines the Application Server performance corresponding to the DNAI selected based on output analytics provided from NWDAF.

Proposal 1: Add performance data from AF as additional input data for NWDAF to generate service experience analytics and delete the Editor Note: It is FFS how the SMF determines the Application Server performance corresponding to the DNAI selected.

Proposal 2: Complete solution31 by adding Impacts on services, entities and interfaces.

2.	Proposal

It is propose a new solution for KI#11 into the TR 23.700-91.

* * * * First Change * * * *

## 6.31	Solution #31: NWDAF Assisted UP Optimization for edge computing

### 6.31.1	Description

This solution is proposed to address Key Issue #10: NWDAF assisted UP optimization and KI #16: UP optimization for edge computing.

This solution specifies for how NWDAF can provide service experience analytics per UP path including UPF and DNAI to the SMF to assist UP optimization for edge computing.

It is proposed to extend the existing Analytics ID as specified in clause 6.4 of TS 23.288 [5] by adding the additional input/output parameters required for this solution

### 6.31.2	Input Data

The service data collected from the AF/NEF, the network data from other 5GC NFs and the network data from OAM for service experience analytics are defined in Table 6.31.2-1, Table 6.31.2-2 and Table 6.31.2-3, respectively.

Table 6.31.2-1: Service Data from AF related to the Service Experience analytics

| Information | Source | Description |
| --- | --- | --- |
| Application ID | AF | To identify the service and support analytics per type of service (the desired level of service) |
| IP filter information | AF | Identify a service flow of the UE for the application |
| Locations of Application | AF/NEF | Locations of application represented by a list of DNAI(s). The NEF may map the AF-Service-Identifier information to a list of DNAI(s) when the DNAI(s) being used by the application are statically defined. |
| Service Experience | AF | Refers to the QoE as established in the SLA and during on boarding. It can be either e.g. MOS or video MOS as specified in ITU-T P.1203.3 [16] or a customized MOS |
| Performance Data | AF | The performance associated with the communication session of the UE with an Application Server that includes: Average Packet Delay, Average Loss Rate, and Throughput as defined in Table 6.49.2-1: Application Server Performance Data from EDN network, clause 6.49. |
| Timestamp | AF | A time stamp associated to the observed level of Service Experience provided by the AF, mandatory if the observed Service Experience is provided by the ASP. |

NOTE:	Performance Data is a new input from AF, comparing to Service Data from AF defined in Table 6.4.2-1, clause 6.4.2, TS 23.288 [5].

Table 6.31.2-2: QoS flow level Network Data from 5GC NF

| Information | Source | Description |
| --- | --- | --- |
| Timestamp | 5GC NF | A time stamp associated with the collected information. |
| Location Info | AMF | The UE location information when the service is delivered. |
| DNN | SMF | DNN for the PDU Session which contains the QoS flow |
| S-NSSAI | SMF | S-NSSAI for the PDU Session which contains the QoS flow |
| Application ID | PCF/SMF | Used by NWDAF to identify the application service provider and application for the QoS flow |
| UPF info | SMF | UPF ID/address/FQDN information for the UPF serving the UE |
| IP filter information | SMF | Provided by the SMF, which is used by NWDAF to identify the service data flow for policy control and/or differentiated charging for the QoS flow |
| QFI | SMF | QoS Flow Identifier |
| QoS flow Bit Rate | UPF | The observed bit rate for UL direction; and The observed bit rate for DL direction |
| QoS flow Packet Delay | UPF | The observed Packet delay for UL direction; and The observed Packet delay for the DL direction |
| Packet transmission | UPF | The observed number of packet transmission |
| Packet retransmission | UPF | The observed number of packet retransmission |

NOTE:	UPF info is a new input from SMF, comparing to the ones for the QoS flow level Network Data from 5GC NF defined in Table 6.4.2-2, clause 6.4.2, TS 23.288 [5].

Table 6.31.2-3: UE level Network Data from OAM as defined in Table 6.4.2-3, clause 6.4.2, TS 23.288 [5]

| Information | Source | Description |
| --- | --- | --- |
| Reference Signal Received Power | OAM | The per UE measurement of the received power level in a network cell, including SS-RSRP, CSI-RSRP as specified in clause 5.5 of TS 38.331 [7] and E-UTRA RSRP as specified in clause 5.5.5 of TS 36.331 [8] |
| Reference Signal Received Quality | OAM | The per UE measurement of the received quality in a network cell, including SS-RSRQ, CSI-RSRQ as specified in clause 5.5 of TS 38.331 [7] and E-UTRA RSRQ as specified in clause 5.5.5 of TS 36.331 [8] |
| Signal-to-noise and interference ratio | OAM | The per UE measurement of the received signal to noise and interference ratio in a network cell, including SS-SINR, CSI-SINR, E-UTRA RS-SINR, as specified in clause 5.1 of TS 38.215 [9] |

### 6.31.3	Output Analytics

The NWDAF services as defined in the clause 7.2 and 7.3, TS 23.288 [5], are used to expose the analytics.

-	Service Experience per UP path statistics information is defined in Table 6.31.3-1.

-	Service Experience per UP path predictions information is defined in Table 6.31.3-2.

Table 6.31.3-1: Service Experience per UP path statistics

| Information | Description |
| --- | --- |
| ServiceExperience per UP path (1..n) | List of observed service experience information for each Application per UP path (including UPF and DNAI). |
| > Application ID | Identification of the application. |
| > Service Experience Type | Type of Service Experience analytics, e.g. on voice, video, other. |
| >>UE location | Indicating the UE location information (e.g. cell ID, gNB ID, etc) when the UE service is delivered. |
| >>UPF Info | Indicating UPF serving the UE |
| >>DNAI | Indicating which DNAI the UE service uses/camps on |
| >>> Service Experience | Service Experience over the Analytics target period (average, variance), e.g. service MOS. |
| > Spatial validity | Area where the estimated Service Experience applies. If Area of Interest information was provided in the request or subscription, spatial validity may be a subset of the requested Area of Interest. |
| > Validity period | Validity period as defined in clause 6.1.3, TS 23.288 [5]. |

NOTE 1:	 The UPF info is a new output information for the Service Experience statistics as defined in clause 6.4.3, TS 23.288 [5].

Table 6.31.3-2: Service Experience per UP path predictions

| Information | Description |
| --- | --- |
| ServiceExperience per UP path (1..n) | List of observed service experience information for each Application per UP path (including UPF and DNAI). |
| > Application ID | Identification of the application. |
| > Service Experience Type | Type of Service Experience analytics, e.g. on voice, video, other. |
| >>UE location | Indicating the UE location information (e.g. cell ID, gNB ID, etc)when the UE service is delivered. |
| >>UPF Info | Indicating which UPF the UE service uses/camps on |
| >>DNAI | Indicating which DNAI the UE service uses/camps on |
| >>> Service Experience | Service Experience over the Analytics target period (average, variance), e.g. service MOS. |
| > Spatial validity | Area where the estimated Service Experience applies. If Area of Interest information was provided in the request or subscription, spatial validity may be a subset of the requested Area of Interest. |
| > Validity period | Validity period as defined in clause 6.1.3, TS 23.288 [5]. |
| > Probability assertion | Confidence of this prediction. |

NOTE 2:	The UPF info is a new out information for the Service Experience predictions as defined in clause 6.4.3, TS 23.288 [5].

### 6.31.4	Procedures

Figure 6.31.4-1: Procedure for subscription to network performance analytics

1.	The SMF sends Nnwdaf_AnalyticsSubscription_Subscribe or Nnwdaf_AnalyticsInfo_Request (Analytics ID=Service Experience per UP path (UPF and DNAI), Target of Analytics Reporting=any UE, Analytics Filter=Application ID, Area of Interest, and Analytics target Period(s)) to the NWDAF.

2.	The NWDAF provides the data analytics, i.e. the observed Service Experience per UP path (UPF and DNAI) to the consumer SMF by means of either Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Notify.

3.	An AF may send a request to influence SMF routing decisions for traffic of PDU Session as defined in clause 5.6.7, TS 23.501 [2] and clause 4.3.6.4, TS 23.502 [3].

4.	The PCF updates the SMF with corresponding new PCC rule(s) with PCF initiated SM Policy Association Modification procedure as described in clause 4.16.5.2, TS 23.502 [3].

5.	During the procedure that AF influence on traffic routing as defined in 4.3.6.4, TS 23.502 [3], besides the PCC rules from the PCF, if the SMF determines that there are more than one DNAI available for the same application the SMF can also take the Service Experience analytics per UP path from the NWDAF into account to:

-	(re)select UP paths, including UPF and DNAI e.g. as described in clause 4.3.5, TS 23.502 [3];

-	(re)configure traffic steering, updating the UPF regarding the target DNAI with new traffic steering rules.

### 6.31.5	Impacts on services, entities and interfaces

AF:

- Provides Performance Data to NWDAF.

NWDAF:

- Provides Service Experience analytics per UP path (UPF and DNAI) to the consumer SMF.

SMF:

- Takes the Service Experience analytics with UP path from the NWDAF into account to do UP path selection and traffic steering.

* * * * End of change * * * *