3GPP TSG-WG SA2 Meeting #140E 	S2-2006277

Aug 19 – Sep 01, 2020, Elbonia	(revision of S2-205376r01)

Source:	China Mobile, KDDI

Title:	KI #6, Sol #51: Update to clarify additional step between NEF and MEC

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_ph2 / Rel-17

Abstract: This contribution proposes to add the additional step between the NEF and the MEC towards Solution #51.

Discussion

It is recommended to add the additional step of Nnef_AnalyticsExposure_Subscribe service operation between the NEF and the MEC, and then the NEF request Nnwdaf_AnalyticsSubscription_Subscribe service operation. Moreover, the similar updates apply to the notification procedure from the NWDAF to the MEC via the NEF.

Proposal

It is proposed to include the following in the TR 23.700-91.

* * * Start of Change * * * *

6.51	Solution #51: optimization for edge computing

6.51.1	Description

6.51.1.1	General

This is a solution for the Key Issue#16: UP optimization for edge computing.

6.51.1.2	Procedure

Figure 6.51.1.2-1: Procedure for UP optimization for edge computing

1.	The consumer MEC sends an Analytics request/subscribe (Analytics ID = Service Experience, Target of Analytics Reporting = group of UEs in area serviced by the MEC, Analytics Filter information = (Application ID, Analytics target period S-NSSAI, DNN, Area of Interest)) to NWDAF via NEF by invoking Nnef_AnalyticsExposure_Subscribe service operation as specified in subclause 6.1.1.2, TS 23.288 [5]. If the analytics information subscription is authorized by the NEF, the NEF subscribes analytics information to the NWDAF by using Nnwdaf_AnalyticsSubscription_Subscribe service operation as specified in subclause 6.1.1.2, TS 23.288 [5].

2a.	NWDAF subscribes the service data from AF by invoking Nnef_EventExposure_Subscribe or Naf_EventExposure_Subscribe service (Event ID = Service Data, Event Filter information = (Application ID, Area of Interest), Target of Event Reporting = group of UEs in the area serviced by the MEC).

2b.	NWDAF subscribes the network data from 5GC NF(s) by invoking Nnf_EventExposure_Subscribe service operation.

2c.	With these data, the NWDAF estimates the Service experience for the application for the MEC.

3.	The NWDAF provides the analytics information to the MEC via NEF by means of Nnwdaf_AnalyticsSubscription_Notify service operation and the NEF notifies the MEC with the corresponding analytics information by invoking Nnef_AnalyticsExposure_Notify service operation as specified in subclause 6.1.1.2, TS 23.288 [5].

NOTE:	The above steps can reuse the procedure defined in clause 6.4.4, TS 23.288 [5], with the MEC acted as a consumer.

4.	Based on statistics or prediction of observed service experience provided by NWDAF, MEC determines to adjust service parameters. E.g., service parameters of video for adjustment may be bit rate, frame rate, codec format, compression parameter, screen size, etc. In this way, the service parameters adjusted can better match the network conditions and achieve the better user experience.

* * * End of Change * * * *