3GPP TSG-WG SA2 Meeting #140E e-meeting 	S2-2006279

Elbonia, August 19 – September 1, 2020	(revision of S2-2005348)

Source:	Huawei, HiSilicon, China Unicom

Title:	KI#18, Sol#54: Update to define the Real-time communication capability information

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_Ph2 / Rel-17

Abstract: This contribution proposes to update the Solution#54 to define the Real-time communication capability information.

1. Discussion

During the SA2#139E meeting, a new solution for KI#18: Enhancement for real-time communication with NWDAF is approved. The outline of the solution is as follows:

-	During the NWDAF registration from NWDAF to NRF, the NWDAF includes the real-time communication capability information and corresponding Analytics ID(s) as part of its NF Profile.

-	During the NWDAF discovery from NWDAF Service Consumer to NRF, the NWDAF Service Consumer should add an indication to indicate it expects real-time communication with NWDAF for specific Analytics ID(s).

However, the following aspects are unclear:

-	What information should be included in the real-time communication capability information?

-	For an Analytics ID, different NWDAFs support different real-time capabilities, during the NWDAF discovery, only an indication is not enough.

For an Analytics ID, this solution proposes that the real-time communication capability information should include its Supported Analytics Delay. Now it goes to how to evaluate the Supported Analytics Delay. In Release 16, the Network Data Analytics could be statistics or predictions, either in the past or in the future.

-	For the statistics, the data could be prepared in NWDAF in advance, so the Supported Analytics Delay could be equals to the Inference Time. Furthermore, if the Statistics is also prepared in advance in the NWDAF, the Supported Analytics Delay is equal to zero.

-	For the predictions, the data is not stored in the NWDAF in advance, so the Supported Analytics Delay could be equals to the sum of Data Preparation Time, (End Time-Start Time) for Data Transfer and Inference Time.

4. Nnwdaf_AnalyticsSubscription_Notify

(Network Data Analytics)

Consumer NF

NWDAF

(End Time for Data Transfer, Inference Time)

Data Provider

(Data Preparation Time)

1. Nnwdaf_AnalyticsSubscription_Subscribe

(Analytics ID)

2. Nnf_EventExposure_Subscribe

(Event IDs)

3. Nnf_EventExposure_Subscribe

(Data Preparation Time, Start Time for Data Transfer)

Figure 1: Different time consumed during the Analytics derivation

NOTE: The Supported Analytics Delay evaluation could be done during the initial analytics derivation phase for the Analytics ID in the NWDAF.

During the NWDAF registration, as part of the NWDAF Profile, a Support for Real-time Communication Indicator should be included in the Real-time communication capability information.

During the NWDAF discovery, the Support for Real-time Communication Indicator should be also indicated to the NRF, then one or multiple candidate NWDAFs will be provided to the NWDAF service consumer. Then the NWDAF service consumer should negotiate with the candidate NWDAFs to find a suitable NWDAF by comparing the Expected Analytics Delay in the NWDAF service consumer with the Supported Analytics Delay in the candidate NWDAF.

Introducing "Supported Analytics Delay" to Sol#54 can also resolve the following EN:

Editor's note:	The real-time communication capable NWDAF may not possess the requested analytics information when receiving an analytics request (e.g. the real-time communication capable NWDAF receives an analytics request immediately after it completes its registration to NRF).What action will be taken under this situation is FFS.

For another EN:

Editor's note:	Real-time communication may only apply to some of the specific Analytic IDs. Which specific Analytic IDs are applicable to real-time communication is FFS.

This paper proposes to assume that all the Analytics IDs which have been defined in TS 23.288 and will be newly introduced by the SI/WI of eNA_Ph2 are applicable to real-time communication. Further clarifications can be done during the normative work, if needed.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-91 v0.4.0.

# * * * * First change * * * *

6.54	Solution #54: Pre-analytics based solution for Real-Time communication with NWDAF

6.54.1	Description

Editor's note:	Describe the solutions. (sub) clause(s) may be added to capture details, input data, output analytics, procedural flow etc. if necessary.

6.54.1.1	General

This solution addresses key issue #18: Enhancement for real-time communication with NWDAF.

Based on the procedures defined in TS 23.288 [5], an NWDAF analytic process may include the following steps:

1.	NWDAF service consumer subscribe at NWDAF to be notified on analytics information by using Nnwdaf_AnalyticsSubscription service, or requests to get from NWDAF analytics information by using Nnwdaf_AnalyticsInfo service.

2.	NWDAF collects data from the data sources (e.g. NF such as AMF, SMF, PCF, and AF; OAM) as a basis of the computation of network analytics.

3.	NWDAF derives requested analytics information based on the data collected from the data source(s).

4.	NWDAF provides requested analytics information to the NWDAF service consumer by using Nnwdaf_AnalyticsInfo_Request Response or Nnwdaf_AnalyticsSubscription_Notify service operation.

The existing procedures for analytics derivation (step 3 as mentioned above) are triggered by the request from NWDAF service consumer. The observation of the existing mechanism so far is that:

-	The data analytics derivation cannot be done before receiving an analytics request from the NWDAF service consumer.

For preventing the extra time caused by the existing mechanism, this solution proposes to enhance NWDAF to support real-time communication capability, which enables the NWDAF to process data pre-analytics for some specific Analytics ID(s) before receiving the analytics request from the NWDAF service consumer. A real-time communication capable NWDAF should start data collection and corresponding analytics for specific Analytics ID(s) when it completes the NF service Registration procedure as specified in clause 4.17.1, TS 23.502 [3], and continuously derive the analytics results in order to respond to the analytics requester NF as soon as possible.

A real-time communication capable NWDAF should send the real-time communication capability and the Analytics ID(s) which are applicable to real-time communication to the NRF as part of its NF Profile during the NF service Registration procedure as specified in clause 4.17.1, TS 23.502 [3]. The NRF should take the real-time communication capability information per Analytics ID into account during the NWDAF discovery procedure performed by NWDAF service consumer.

A real-time communication capable NWDAF may support several Analytics IDs, and it is possible that some of the Analytics IDs supported by the real-time communication capable NWDAF are applicable to real-time communication while others are not.

During the NWDAF discovery procedure (as specified in clause 4.17.4 or 4.17.9, TS 23.502 [3]), the NWDAF service consumer may indicate whether it expects real-time communication with NWDAF for specific Analytics ID(s) in the service discovery request. Based on the information corresponding to the real-time communication expectation received from the NWDAF service consumer, the NRF selects a suitable real-time communication capable NWDAF for the NWDAF service consumer. In case of delegate service discovery, the indication of whether real-time communication with NWDAF is needed is included in the request sent from the NWDAF service consumer to the SCP. The NWDAF service consumer then establishes the Analytics Request procedure (as specified in clause 6.1.2, TS 23.288 [5]) to the selected real-time communication capable NWDAF.

Upon receiving the Analytics Request from the NWDAF service consumer, the real-time communication capable NWDAF provides the requested analytics information to the NWDAF service consumer based on the pre-analytics result immediately.

6.54.1.2	Procedure for Real-time communication capable NWDAF registration

NWDAF

NRF

NF

(Data Provider)

3. Nnrf_NFManagement_NFRegister_request

(Analytics ID, Real-time communication indicator, Supported Analytics Delay)

2. Calculate Supported Analytics Delay

1a. Nnf_EventExposure_Subscribe (Event ID)

1b. Nnf_EventExposure_Notify

(Collected data, Data preparation delay, Start Time for data transfer)

1. Data collection

1c. Record End Time for data transfer

4. Nnrf_NFManagement_NFRegister_request Response

Figure 6.54.1.2-1: Procedure for Real-time communication capable NWDAF registration

Step 1. During the data collection, for an Analytics ID, each NWDAF evaluates the delay for data collection which including the data preparation time within the NF, data transfer time from the NF to the NWDAF. The data collection time delay is also an average, maximum or minimum value for all the data collection for specific area of interest, which is not a UE or UE group specific.

Step 2. NWDAF calculates the Supported Analytics Delay which including the delay for data collection in Step 1 and the delay for inference within the NWDAF. Similar to data collection time delay, the Supported Analytics Delay for an Analytics ID is also an average, maximum or minimum value for all received Analytic requests for the NWDAF serving area, which is not a UE or UE group specific.

Step 3. The real-time communication capable NWDAF sends Nnrf_NFManagement_NFRegister Request message to NRF, additionally includes the real-time communication capability information (Support for Real-time Communication Indicator, Optionally the Supported Analytics Delay (average, maximum or minimum)) and corresponding Analytics ID(s) which are applicable to real-time communication as part of its NF Profile.

Step 4. The NRF acknowledge NF Registration is accepted via Nnrf_NFManagement_NFRegister response.

6.54.1.3	Procedure for Real-time communication capable NWDAF discovery

NWDAF

NRF

NWDAF Service Consumer

1. Nnrf_NFDiscovery_Request

(Analytics ID, Real-time communication indicator, Optionally an Expected Analytics Delay)

3. Nnwdaf_AnalyticsDelayNegotiation_Create request

(Expected Analytics Delay)

2. Nnrf_NFDiscovery_Request Response

(A set of candidate NWDAF IDs)

4. Nnwdaf_AnalyticsDelayNegotiation_Create response

Figure 6.54.1.3-1: Procedure for Real-time communication capable NWDAF discovery

Step 1: The NWDAF service consumer sends Nnrf_NFManagement_NFDiscover Request message to NRF, includes an indication which indicates it expects real-time communication with NWDAF for specific Analytics ID(s) and Optionally an Expected Analytics Delay.

NOTE:	The "time when analytics is needed" is used for request/response mode and refer to the timestamp when the consumer NF plan to consume the analytics from NWDAF as defined in clause 6.1.3, TS 23.288, while the "Expected Analytics Delay" is used for subscribe/notification mode and refer to an maximum duration in which the consumer can suffer.

Step 2. The NRF determines a set of candidate NWDAF(s) matching the Nnrf_NFDiscovery_Request and sends the candidate NWDAF profiles to the NWDAF service consumer.

Step 3-4 are executed only when the Expected Analytics Delay is not include in Step 1:

Step 3-4. The NWDAF service consumer negotiates with each candidate NWDAF to find a suitable NWDAF by comparing the Expected Analytics Delay in the NWDAF service consumer with the Supported Analytics Delay in the candidate NWDAF.

The following impacts are applicable to clause 4.17.9, TS 23.502 [3] when a NWDAF service consumer requests for the discovery of a real-time communication capable NWDAF with delegated service discovery:

-	Step 1: The NWDAF service consumer sends the service request message to SCP, includes an indication which indicates it expects real-time communication with NWDAF for specific Analytics ID(s).

6.54.1.4	Real-time communication capable NWDAF based data pre-collection and pre-analytics

Figure 6.54.1.4-1: Data pre-analytics procedure

0.	The real-time communication capable NWDAF successfully registered in NRF.

1.	The real-time communication capable NWDAF subscribes to the data source for necessary data/event reporting by sending Data collection Subscribe Request. In cases of different scenarios, the Data collection Subscribe procedure may be:

-	Nnf_EventExposure_Subscribe in case of the data source is an NF.

-	Nnef_EventExposure_Subscribe in case of the data source is an AF. In this case, the service data subscribe request should be forwarded from NEF to AF by invoking Naf_EventExposure_Subscribe service operation.

-	Nnrf_NFManagement_NFStatusSubscribe in case of the data source is an NRF.

-	Subscribe (Input) in case of the data source is an OAM.

The real-time communication capable NWDAF should additionally include a continuous data reporting indication in the Data collection Subscribe Request to indicate the data source to report the subscribed data/event continuously to the real-time communication capable NWDAF until the real-time communication capable NWDAF unsubscribes the data/event reporting. The immediate reporting flag may be used by the real-time communication capable NWDAF, if available, to obtain the current status of the subscribed event.

NOTE:	In case of the data source is an NRF, the real-time communication capable NWDAF may invoke Nnrf_NFDiscovery service before invoking Nnrf_NFManagement_NFStatusSubscribe to obtain the current NF information from the NRF.

The data source responds the request by sending Data collection Subscribe Response to the real-time communication capable NWDAF.

2.	Data source notifies to the real-time communication capable NWDAF the subscribed data/event continuously based on the continuous data reporting indication received from the real-time communication capable NWDAF in Data collection Subscribe Request. The data/event notification can be done by:

-	Nnf_EventExposure_Notify in case of the data source is an NF.

-	Naf_EventExposure_Notify in case of the data source is an AF.

In this case, the service data notification should be forwarded from NEF to real-time communication capable NWDAF by invoking Nnef_EventExposure_Notify service operation.

-	Nnrf_NFManagement_NFStatusNotify in case of the data source is an NRF.

-	Notification (notifyFileReady) in case of the data source is an OAM.

3.	The real-time communication capable NWDAF processes pre-analytics for the Analytics ID(s) which are applicable to real-time communication based on the data/event reported by the data source. The real-time communication capable NWDAF should keep updating the analytics result of specific Analytics ID(s) when receiving new notification from the data source.

NOTE:	The trained data model is derived by the real-time communication capable NWDAF itself during the data pre-analytics procedure or shared from other NWDAFs relies on the conclusion of KI#19.

4.	The real-time communication capable NWDAF sends Data collection Unsubscribe Request to data source when it decides not to collect data from the data source. The Data collection Unsubscribe procedure may be:

-	Nnf_EventExposure_Unsubscribe in case of the data source is an NF.

-	Nnef_EventExposure_Unsubscribe in case of the data source is an AF.

In this case, the service data subscribe request should be forwarded from NEF to AF by invoking Naf_EventExposure_Unsubscribe service operation.

-	Nnrf_NFManagement_NFStatusUnsubscribe in case of the data source is an NRF.

-	Unsubscribe operation in case of the data source is an OAM.

The data source stops reporting data/event to the real-time communication capable NWDAF and responds the request by sending Data collection Unsubscribe Response to the real-time communication capable NWDAF.

6.54.1.5	Analytics Request

Figure 6.54.1.5-1: Network data analytics Request to a real-time communication capable NWDAF

1.	The NWDAF service consumer requests analytics information by invoking Nnwdaf_AnalyticsInfo_Request service operation, and includes real-time communication flag per Analytics ID(s) in the request message. The Nnwdaf_AnalyticsInfo_Request message may be forwarded by an NEF to the real-time communication capable NWDAF in case of the NWDAF service consumer is an AF.

2.	The real-time communication capable NWDAF responds with analytics information to the NWDAF service consumer by Nnwdaf_AnalyticsInfo_Request Response.

6.54.2	Impacts on services, entities and interfaces

Editor's note:	This clause lists impacts to services and interfaces.

NWDAF:

-	Supports to additionally include the real-time communication capability and corresponding Analytics ID(s) which are applicable to real-time communication as part of its NF Profile during the NF service Registration Procedure.

-	Supports to process continuous data collection after the NF service Registration procedure without receiving analytics request from NWDAF service consumer.

-	Supports to process pre-analytics for specific Analytics ID(s) continuously after obtaining the necessary data.

NRF:

-	Supports to maintain the NF Profile of real-time communication capable NWDAF including the real-time communication capability.

-	Supports to discover a real-time communication capable NWDAF based on corresponding indication received from the NWDAF service consumer for real-time communication with NWDAF for specific Analytics ID(s).

NWDAF service consumers:

-	Supports to include an indication in the NF service discovery request which indicates that real-time communication with NWDAF for specific Analytics ID(s) is needed.

-	Supports to include real-time communication flag per Analytics ID(s) in the Nnwdaf_AnalyticsInfo_Request message.

# * * * * End of changes * * * *