3GPP TSG-SA WG2 Meeting #140E (e-meeting)	S2-2006524

Aug 19 – Sep 01, 2020, Elbonia	(was S2-2005091r05)

Source:	NEC

Title:	KI#7 Sol#31: Updates to Solution #31 Steering the UE to a network slice in a different frequency band

Document for:	Approval

Agenda Item:	8.4

Work Item / Release:	FS_eNS_Ph2 / Rel-17

Abstract: This contribution proposes updates to Solution #31 Steering the UE to a network slice in a different frequency band.

# 1. Background

This contribution proposes updates to resolve Editor’s Notes. The following updates in TR23.700-40 v0.4.0

******* Start of the change ********

### 6.31	Solution #31: Steering the UE to a network slice in a different frequency band

### 6.31.1	Introduction

This solution addresses the below requirements from Key Issue #7: Support of 5GC assisted cell selection to access network slice.

-	How does 5GS steer UEs to a 5G-AN (e.g. a specific frequency band) that can support the network slices that the UE can use.

-	What information does 5GS need to take a decision to steer UE to a proper 5G-AN.

### 6.31.2	High Level Description

The solution is based on the following high level assumptions and principles:

-	The UE is allocated Allowed NSSAI which can contain S-NSSAIs supported in different frequency bands, however all S-NSSAIs are supported in all Tracking Areas or the Registration Area.

### 6.31.3	Procedures

Figure 6.31.3.1-1 - Steering the UE to a network slice in different frequency band

1)	The UE is in idle mode, registered via RAN-1 for S-NSSAI-1, which is preferred in frequency band 1 (FB-1) and S-NSSAI-2, which is preferred in frequency band 2 (FB-2).

2)	An application in the UE needs to establish service on S-NSSAI-2 in FB-2.

3)	The UE establishes RRC connection with RAN-1.

4)	The UE triggers PDU Session Establishment Request on S-NSSAI-2 via RAN-1.

5)	The AMF is aware that S-NSSAI-2 is supported with preference in FB-2.

6) AMF requests RAN-1 to steer the UE to a RAN Node supporting S-NSSAI-2 in FB-2.

7)	If the UE has no other active PDU Session(s),  RAN-1 triggers inter-frequency cell change to RAN-2 in FB-2 which supports S-NSSAI-2 and is in UE's location.

If the UE has other active PDU Session(s), the UE stays on RAN-1 and RAN-1 triggers the Dual Connectivity configuration with RAN-2 in order to establish the new PDU Session associated with S-NSSAI-2 on RAN-2 while the PDU Session(s) associated with S-NSSAI-1 stay on RAN-1.

NOTE:	It is for RAN Working Groups to define the inter-frequency cell change in connected mode e.g. Handover, Cell Change Order, RRC release with re-direction or RRC Connection reconfiguration.

8)	AMF continues with the PDU Session establishment procedure on S-NSSAI-2 via RAN-2.

9)	After PDU session on S-NSSAI-2 is released, the UE falls-back (e.g. re-selects back) to a cell in FB-1. This step is optional. If not implemented, the UE will stay on FB-2 until it is steered back to FB-1 due to a new service request on FB-2.

### 6.31.4	Impacts on services, entities and interfaces

-	UE: cell fall-back to a cell on the initial frequency band (optional).

-	AMF: decision and request for handover to a cell from another frequency band.

-	RAN: can decide to stay on the current cell and deploy Dual Connectivity with a cell from another frequency band for the new PDU Session when there are existing PDU Sessions.

******* End of the change *******