SA WG2 Meeting #140E e-meeting 	S2-2006559

Aug 19 – Sep 01, 2020, Elbonia	 (revision of S2-2005199)

Source:	CATT

Title:	KI#3, New Sol: PC5 connection establishment for Layer-3 UE-to-Network Relay without Relay discovery

Document for:	Approval

Agenda Item:	8.8

Work Item / Release:	FS_5G_ProSe / Rel-17

Abstract of the contribution: This paper proposes an alternative solution on PC5 connection establishment procedure without Relay discovery based on the baseline Layer-3 UE-to-Network Relay solution (i.e. solution#6).

Discussion

Solution#6 in TR 23.752 shows the baseline Layer-3 UE-to-Network Relay architecture and procedure, in which standalone Discovery procedure (step 2 of clause 6.6.2) and one-to-one establishment procedure (step 3 of clause 6.6.2) are used to establish the PC5 connection between Remote UE and UE-to-Network Relay.

This contribution proposes an alternative option to establish the PC5 connection by reusing the Layer-2 link establishment procedure defined in TS 23.287 clause 6.3.3.1, in which standalone discovery procedure is not needed and implicit UE-to-Network Relay discovery is embedded into the Direct Communication procedure.

Proposal

It is proposed to add a new solution for Key Issue #3 into TR 23.752.

First change (all text new)

6.X	Solution #X: PC5 connection establishment for Layer-3 UE-to-Network Relay without Relay discovery

6.X.1	Description

This solution addresses Key Issue#3 and provides an alternative option on PC5 connection establishment without Relay discovery on top of solution#6.

Solution#6 in TR 23.752 shows the baseline Layer-3 UE-to-Network Relay architecture and procedure, in which standalone Discovery procedure (step 2 of clause 6.6.2) and one-to-one establishment procedure (step 3 of clause 6.6.2) are used to establish the PC5 connection between Remote UE and UE-to-Network Relay.

This solution proposes an alternative option to establish the PC5 connection by reusing the Layer-2 link establishment procedure defined in TS 23.287 [5] clause 6.3.3.1, in which standalone discovery procedure is not needed and implicit UE-to-Network Relay discovery is embedded into the Direct Communication procedure.

6.X.2	Procedures

The procedure of solution#6 in clause 6.6.2 is reused with the following differences:

Figure 6.X.2-1: ProSe 5G UE-to-Network Relay

0.	This step is same as step 0 of clause 6.6.2.

1.	This step is same as step 1 of clause 6.6.2.

2.	Remote UE sends a Direct Communication Request message to initiate the unicast layer-2 link establishment procedure with ProSe UE-to-Network Relay. This step is same as step 3 of TS 23.287 [5] clause 6.3.3.1 with the following differences:

-	V2X Service Info is replaced by Relay Service Code.

-	Source User Info is Remote UE’s Application Layer ID, and Target User Info is Relay UE’s Application Layer ID.

3.	Security between Remote UE and UE-to-Network Relay is established. This step is same as step 4 of TS 23.287 [5] clause 6.3.3.1 with the following differences:

-	The UE-to-Network(s) that support the Relay Service Code responds by establishing the security with Remote UE (step 3b).

NOTE 1:	The signalling for the Security Procedure is to be defined by SA3.

NOTE 2:	This solution does not preclude authorization procedure for Relay operation with the core network.

4.	A Direct Communication Accept message is sent to Remote UE by the UE-to-Network Relay UE(s) that has successfully established security with Remote UE. This step is same as step 5 of TS 23.287 [5] clause 6.3.3.1 with the following differences:

-	The UE-to-Network(s) that support the Relay Service Code responds the request by sending a Direct Communication Accept message (step 4b).

5.	If Remote UE receives the Accept message from multiple UE-to-Network Relay UEs in step 4, the Remote UE may decide to select one UE-to-Network Relay for further communication and release the link with other UE-to-Network Relays, which may trigger UE-to-Network to perform PDU session release or modification procedure.

6.	The UE-to-Network Relay may establish new PDU session or modify existing PDU session for relaying, this step is same as step 3 of clause 6.6.2.

7.	IPv6 prefix or IPv4 address is allocated for the Remote UE, this step is same as step 4 of clause 6.6.2.

8.	This step is same as step 5 of clause 6.6.2.

Editor's note:	Whether Layer-2 link establishment procedure can be initiated by UE-to-Network Relay is FFS.

6.X.3	Impacts on Existing Nodes and Functionality

The solution has impacts in the following entities:

SMF:

-	Needs to support procedures for Remote UE report.

UE:

-	Needs to support procedures for Remote UE and UE-to-Network Relay.

End of change