SA WG2 Meeting #140E e-meeting	S2-2006588

Aug 19 – Sep 01, 2020, Elbonia

Title:	LS on new PQI support for PC5 communication

Response to:	-

Release:	Rel-17

Work Item:	FS_5G_ProSe

Source:	SA2

To:	RAN1

Cc:	RAN2

Contact person:	Jianhua Liu

liujianhua@oppo.com

Send any reply LS to: 3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

SA2 is studying the PC5 direct communication on the SID of 5G ProSe, which is covering Public Safety and commercial cases. SA2 has assumption the existing V2X mechanism is reused, on top of that, the following new Standardized PQIs for Public Safety and commercial in Table 1 and Table 2 are proposed to be introduced for PC5 communication.

Table 1: New Standardized PQIs for Public Safety

| PQI Value | Resource Type | Default Priority Level | Packet Delay Budget | Packet Error Rate | Default Maximum Data Burst Volume | Default Averaging Window | Example Services |
| --- | --- | --- | --- | --- | --- | --- | --- |
| New value#1 | GBR | 1 | 150 ms | 10-2 | N/A | 2000 ms | Mission Critical user plane Push To Talk voice (e.g. MCPTT) |
| New value#2 |  | 2 | 200 ms | 10-2 | N/A | 2000 ms | Non-Mission-Critical user plane Push To Talk voice |
| New value#3 |  | 2 | 200 ms | 10-3 | N/A | 2000 ms | Mission Critical Video user plane |
| New value#4 | Non-GBR | 1 | 120 ms | 10-6 | N/A | N/A | Mission Critical delay sensitive signalling (e.g. MC-PTT signalling) |
| New value#5 |  | 6 | 400 ms | 10-6 | N/A | N/A | Mission Critical Data (e.g. example services are the same as 5QI 6/8/9) |

Table 2: New Standardized PQIs for commercial service

| PQI Value | Resource Type | Default Priority Level | Packet Delay Budget | Packet Error Rate | Default Maximum Data Burst Volume | Default Averaging Window | Example Services |
| --- | --- | --- | --- | --- | --- | --- | --- |
| New value#1 | Delay Critical GBR | 5 | 5ms | 10-4 | 20000 bytes | 2000 ms | Interactive service - consume VR content with high compression rate via tethered VR headset |
| New value#2 |  | 6 | 10ms | 10-4 | 20000 bytes | 2000 ms | interactive service - consume VR content with low compression rate via tethered VR headset; Gaming or Interactive Data Exchanging; |

Q1): SA2 kindly asks RAN1 whether these new Standardised PQIs can be supported in AS layers.

In addition, MDBV values can be changed according to application requirements to override the default MDBV. For Example, for commercial service, to support required data rate of 10Gbps, the maximum MBDV value set by the application can be 12.5M byte.

Q2): SA2 kindly asks RAN1 what is the maximum MBDV value AS layer can support with the indicated PDB and PER in Table 2.

# 2	Actions

To RAN1

ACTION: SA2 kindly asks RAN WG1 to provide feedback on the above questions.

# 3	Dates of next TSG SA WG 2 meetings

3GPP TSG SA2#141E	12 – 23 October 2020	eMeeting

3GPP TSG SA2#142E	16 – 20 November 2020	eMeeting