SA WG2 Meeting #S2-140E	S2-2004911

19 August - 2 September, 2020, Elbonia	(revision of S2-20xxxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2397 | rev | - | Current version: | 16.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Ammendment of VLAN configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | VERTICAL_LAN |  |  |  |  |  | Date: |  |  | 2020-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | VLAN configuration in currentl specification claus 5.28.3.3 is described as: “The CNC obtains the 5GS bridge VLAN configuration from TSN AF as per IEEE 802.1Q[98] clause 12.10.1.1. The TSN AF and UPF/NW-TT are pre-configured with same 5GS bridge VLAN configuration.” The description only mentions TSN AF, UPF/NW-TT, it is not clear about why UE/DS-TT doesn’t need to be preconfiguration, and how UE/DS-TT ports get VLAN information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | There is a single Bridge VLAN Configuration managed object per Bridge according to IEEE 802.1Q. In current release, because both AF and UPF/NW-TT are preconfigured with VLAN configuration, there is no need to use BMIC to exchange VLAN configuration information. However the VLAN configuration is part of of bridge management information, therefore the preconfiguration in the UPF/NW-TT as per bridge information is good eough. The pre-configure VLAN in UPF/NW-TT can be applied for both NW-TT ports and DS-TT ports. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | VLAN configuration is incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.28.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of Change * * * *

#### 5.28.3.3	VLAN Configuration Information

The CNC obtains the 5GS bridge VLAN configuration from TSN AF as per IEEE 802.1Q[98] clause 12.10.1.1. The TSN AF and UPF/NW-TT are pre-configured with same 5GS bridge VLAN configuration. VLAN pre-configuration in the UPF can be applied for both network and device side ports.

NOTE:	In this release, the VLAN Configuration Information are pre-configured at the TSN AF and the NW-TT and is not exchanged between the TSN AF and the UPF/NW-TT.

* * * * End of Change * * * *